COMPLETE TECHNICAL DOCUMENTATION

IoT Gateway Stack

OpenThread Border Router with MQTT and Node-RED Dashboard

QPG6200L IoT Mesh Network Project

| Document Version | 2.0 (Updated Edition) |
| --- | --- |
| Original Date | March 9, 2026 |
| Last Updated | March 18, 2026 |
| Classification | Internal Technical Documentation |
| Target Audience | Engineers with basic Linux knowledge |
| Estimated Setup Time | 2-3 hours |

This document provides step-by-step instructions to replicate the complete IoT Gateway Stack setup. Every command, configuration file, and troubleshooting step is documented with detailed explanations of what each component does and why it is necessary.

# Table of Contents

1.   Executive Summary and System Overview

2.   Understanding the Technology Stack

3.   Hardware Requirements and Specifications

4.   Prerequisites and Development Environment Setup

5.   Phase 1: Building and Flashing the nRF52840 RCP Firmware

6.   Phase 2: Raspberry Pi Operating System Installation

7.   Phase 3: OpenThread Border Router Installation and Configuration

8.   Phase 4: Mosquitto MQTT Broker Installation and Configuration

9.   Phase 5: Node-RED and Dashboard 2.0 Installation

10.  System Integration and End-to-End Testing

11.  Issues Encountered and Detailed Resolutions

12.  Final System Configuration Summary

13.  Quick Start Guide: Daily Operations

14.  Troubleshooting Guide

15.  Appendix A: Complete Command Reference

16.  Appendix B: Configuration File Contents

17.  Appendix C: Glossary of Terms

# 1.  Executive Summary and System Overview

## 1.1  Purpose of This Document

This document serves as a complete, reproducible guide for setting up an IoT Gateway Stack. It was created during an actual deployment on March 9, 2026, and updated on March 18, 2026 to reflect the current portable demo network configuration. It includes every command executed, every error encountered, and every solution applied. A person with basic Linux command-line knowledge should be able to follow this guide and achieve an identical working system.

v2.0 Update Note: The network configuration has changed from a home ethernet setup (Pi IP: 192.168.1.69) to a portable demo network using the TRENDnet TEW-827DRU router. The Raspberry Pi is now permanently accessible at 192.168.10.102 via a DHCP reservation over ethernet. WiFi on the Pi is not required or used — all demo traffic routes through the wired ethernet connection.

## 1.2  What This System Does

The IoT Gateway Stack enables wireless IoT devices (built with Qorvo QPG6200L chips) to communicate over a Thread mesh network and send their data to a visual dashboard. Here is what each part does:

- Thread Mesh Network: A low-power wireless network protocol (similar to Wi-Fi but designed for IoT devices). Devices can talk to each other and relay messages, creating a self-healing mesh. If one device fails, messages route around it.

- Border Router: The bridge between the Thread network (wireless) and your regular Ethernet/Wi-Fi network. Without this, your Thread devices cannot reach the internet or your local network.

- MQTT Broker: A message routing service. Devices publish data to topics (like home/motion/sensor1), and other applications subscribe to those topics to receive the data. Think of it like a post office for IoT messages.

- Node-RED Dashboard: A visual programming tool that receives MQTT messages and displays them on a web-based dashboard with gauges, charts, and buttons. This is what you see on your tablet or phone.

## 1.3  System Architecture Diagram

Data flows through the system in this order:

QPG6200L IoT Devices (Motion Sensor, Doorbell, Microphone, Speaker) - Run on coin cell batteries for months / years - Communicate using Thread protocol (IEEE 802.15.4)

↓  Wireless 2.4 GHz  ↓

nRF52840 USB Dongle (Radio Co-Processor / RCP) - Receives Thread radio signals - Converts to serial data via USB - Uses Spinel protocol to communicate with Pi

↓  USB Serial Cable  ↓

Raspberry Pi 4 (Gateway Computer) IP: 192.168.10.102 +--------------------------------------------------+ ¦ OTBR (OpenThread Border Router) ¦ ¦ - Manages Thread network ¦ ¦ - Routes IPv6 between Thread and Ethernet ¦ +--------------------------------------------------+ ¦ Mosquitto MQTT Broker (Port 1883) ¦ ¦ - Routes publish/subscribe messages ¦ +--------------------------------------------------+ ¦ Node-RED + Dashboard 2.0 (Port 1880) ¦ ¦ - Visual flow programming ¦ ¦ - Web-based dashboard with gauges and charts ¦ +--------------------------------------------------+

↓  Ethernet Cable  ↓

TRENDnet TEW-827DRU Router (192.168.10.1) - Pi DHCP reservation: 192.168.10.102 - WiFi SSID: TRENDnet827_2.4GHz_QMGR (open, no password)

↓  WiFi  (TRENDnet827_2.4GHz_QMGR)  ↓

Laptop + Lenovo Tab M11 - SSH: ssh pi@192.168.10.102 - Dashboard: http://192.168.10.102:1880/dashboard - OTBR GUI: http://192.168.10.102

## 1.4  Final System Status

After completing all steps in this guide, you will have:

| Component | Status | How to Access |
| --- | --- | --- |
| nRF52840 Dongle | Flashed & Working | Plugged into Pi USB, detected as /dev/ttyACM0 |
| Thread Network | Active (Leader) | Channel 18, PAN ID 0x1905 |
| OTBR Web GUI | Running | http://192.168.10.102 (browser) |
| Mosquitto MQTT | Running | Port 1883 (mqtt://192.168.10.102:1883) |
| Node-RED Editor | Running | http://192.168.10.102:1880 (browser) |
| Dashboard | Ready | http://192.168.10.102:1880/dashboard (browser) |
| SSH | Available | ssh pi@192.168.10.102 |

# 2.  Understanding the Technology Stack

Before diving into the setup, it is important to understand what each technology does and why it was chosen. This section explains each component in detail.

## 2.1  Thread Protocol

### What is Thread?

Thread is a wireless networking protocol designed specifically for IoT devices. It was created by the Thread Group (which includes Google, Apple, Amazon, and others) to address the limitations of existing protocols like Wi-Fi, Zigbee, and Bluetooth.

### Why Thread Instead of Wi-Fi?

- Power Consumption: Wi-Fi devices need to maintain a constant connection and typically require wall power. Thread devices can run on coin cell batteries for years because they only transmit when needed.

- Mesh Networking: Wi-Fi uses a hub-and-spoke model where every device talks directly to the router. Thread creates a mesh where devices relay messages for each other, extending range and improving reliability.

- Self-Healing: If a Thread device fails or moves, the network automatically reconfigures itself to route around the problem. Wi-Fi networks do not have this capability.

- IPv6 Native: Thread uses standard IPv6 addressing, making it easy to integrate with existing IT infrastructure. Each device gets its own IPv6 address.

### Thread Network Concepts

- Leader: One device in the Thread network is elected as the Leader. It manages network-wide configuration and assigns addresses. In our setup, the Border Router acts as the Leader.

- Router: Devices that can relay messages for other devices. They are mains-powered because relaying requires more energy.

- End Device: Battery-powered devices that send and receive their own messages but do not relay for others. Our sensors will be End Devices.

- Border Router: A special device that bridges the Thread network to other IP networks (like your home Ethernet/Wi-Fi). Without a Border Router, Thread devices cannot communicate with the internet or your local network.

- Network Key: A 128-bit encryption key that all devices must have to join the network. This ensures security — unauthorized devices cannot join or eavesdrop.

## 2.2  Radio Co-Processor (RCP) Architecture

The Raspberry Pi does not have a built-in IEEE 802.15.4 radio (the radio type used by Thread). We need to add this capability using an external USB dongle. There are two ways to do this:

- Network Co-Processor (NCP): The dongle runs the full Thread stack. The Pi just sends high-level commands. This is simpler but less flexible.

- Radio Co-Processor (RCP): The dongle only handles the low-level radio operations (transmit/receive packets). The Pi runs the full Thread stack. This gives us more control and allows for easier updates. THIS IS WHAT WE USE.

The nRF52840 dongle communicates with the Raspberry Pi using the Spinel protocol over a USB serial connection. Spinel is a simple binary protocol designed for this purpose.

## 2.3  OpenThread Border Router (OTBR)

OpenThread is Google's open-source implementation of the Thread protocol. OTBR (OpenThread Border Router) is a complete software package that turns a Raspberry Pi (with an RCP dongle) into a Thread Border Router.

### OTBR Components

- otbr-agent: The main service that runs the Thread stack, manages the RCP dongle, handles IPv6 routing, and creates the wpan0 network interface. This is the core of OTBR.

- otbr-web: A web server that provides a graphical interface for managing the Thread network. You can form networks, view connected devices, and configure settings through a browser.

## 2.4  MQTT (Message Queuing Telemetry Transport)

MQTT is a lightweight messaging protocol designed for IoT applications. It uses a publish/subscribe model rather than the traditional client/server model.

### How MQTT Works

- Topics: Messages are organized into topics, which are like channels. Example: home/living-room/motion or home/front-door/doorbell

- Publish: A device sends (publishes) a message to a topic. Example: The motion sensor publishes {"motion": true, "distance": 42} to home/living-room/motion

- Subscribe: Applications subscribe to topics they care about. The broker sends them all messages published to those topics.

- Broker: The MQTT broker (Mosquitto in our case) sits in the middle, receiving all published messages and forwarding them to all subscribers of each topic.

### Why Mosquitto?

Mosquitto is an open-source MQTT broker from the Eclipse Foundation. It is lightweight, reliable, widely used, and runs well on Raspberry Pi. It is the industry standard for small to medium IoT deployments.

## 2.5  Node-RED

Node-RED is a visual programming tool originally developed by IBM. It allows you to wire together IoT devices, APIs, and services using a browser-based flow editor. Instead of writing code, you drag and drop nodes and connect them with wires.

### Why Node-RED?

- Visual Programming: Non-programmers can create IoT applications by connecting nodes visually.

- MQTT Integration: Built-in MQTT nodes make it trivial to subscribe to topics and process messages.

- Dashboard 2.0: The FlowFuse Dashboard extension adds drag-and-drop UI widgets (gauges, charts, buttons) that automatically update in real-time.

- Extensible: Thousands of community-contributed nodes for connecting to databases, cloud services, and more.

# 3.  Hardware Requirements and Specifications

## 3.1  Gateway Hardware (Required)

These are the physical components you need for the gateway:

| Component | Specification | Purpose and Notes |
| --- | --- | --- |
| Raspberry Pi 4 | Model B, 2GB+ RAM recommended | The main gateway computer. Runs Linux and all our software. 2GB RAM is sufficient; 4GB or 8GB provides more headroom. |
| MicroSD Card | 16GB minimum, 32GB recommended, Class 10 or faster | Stores the operating system and all software. Faster cards improve boot time and overall performance. |
| nRF52840 USB Dongle | Nordic Semiconductor PCA10059 | Provides the IEEE 802.15.4 radio for Thread. Must be flashed with RCP firmware before use. Available from Mouser, DigiKey, or direct from Nordic. |
| USB-C Power Supply | 5V 3A (15W) minimum, official Raspberry Pi PSU recommended | Powers the Raspberry Pi. Underpowered supplies cause instability. The official PSU is reliable. |
| Ethernet Cable | Cat5e or Cat6, length as needed | Connects Pi to the TRENDnet router yellow LAN port. Wired connection is more reliable than Wi-Fi for a gateway. |
| TRENDnet TEW-827DRU | AC2600 Dual-Band Router, H/W V2.0R | Portable demo router. Creates private local network. Requires 12V 1.5A power adapter. Pi reserved at 192.168.10.102. |
| Display Device | Lenovo Tab M11 or any device with a web browser | For viewing the dashboard. Can be a tablet, phone, or computer. Any modern web browser works. |

## 3.2  Development/Build Computer (Required for Initial Setup)

You need a separate computer to compile the RCP firmware for the nRF52840 dongle. This computer is only needed during initial setup.

| Component | Specification | Purpose and Notes |
| --- | --- | --- |
| Linux Computer | Ubuntu 20.04 or newer recommended | Used to compile the OpenThread RCP firmware. Can be a dedicated machine, a VM, or Windows WSL2. Our setup used a Dell Latitude 7280 running Ubuntu. |
| SD Card Reader | USB or built-in, microSD compatible | For writing the Raspberry Pi OS image to the microSD card. |

NOTE: The build computer only needs to be used once to flash the nRF52840 dongle. After that, all operations are performed on or via the Raspberry Pi.

# 4.  Prerequisites and Development Environment Setup

Before starting the installation phases, you need to prepare your build computer with the necessary software tools. This section covers everything you need to install.

## 4.1  Required Software on Build Computer

Open a terminal on your Ubuntu build computer and install the following packages:

Step 1: Update the package list (ensures you get the latest versions):

sudo apt update

Explanation: This command downloads the latest list of available packages from Ubuntu's servers. Always run this before installing new software.

Step 2: Install build tools:

sudo apt install -y build-essential cmake ninja-build

Explanation of each package:

- build-essential: Installs GCC compiler, make, and other basic build tools needed to compile C/C++ code.

- cmake: A cross-platform build system generator. The OpenThread project uses CMake to manage its build process.

- ninja-build: A fast build system. CMake generates Ninja build files, which compile code faster than traditional Makefiles.

Step 3: Install Python 3 and pip (package manager for Python):

sudo apt install -y python3 python3-pip

Explanation: Python is used by several build scripts, and pip is needed to install the nrfutil tool later.

Step 4: Install Git (version control system):

sudo apt install -y git

Explanation: Git is used to download (clone) the OpenThread source code from GitHub.

Step 5: Install the ARM cross-compiler toolchain:

sudo apt install -y gcc-arm-none-eabi binutils-arm-none-eabi

Explanation: The nRF52840 uses an ARM Cortex-M4 processor. Since your build computer uses an x86/x64 processor, you need a cross-compiler that runs on x86 but generates ARM code.

- gcc-arm-none-eabi: The GNU C compiler for ARM microcontrollers (bare-metal, no operating system).

- binutils-arm-none-eabi: Binary utilities (linker, objcopy, etc.) for ARM. We use objcopy to convert the compiled binary to Intel HEX format.

Step 6: Install nrfutil (Nordic Semiconductor's flashing tool):

pip3 install --user nrfutil==6.1.7

Explanation: nrfutil is Nordic Semiconductor's command-line tool for creating DFU (Device Firmware Update) packages and flashing them to nRF devices via USB bootloader.

- --user: Installs to ~/.local/bin instead of system-wide, avoiding permission issues.

- ==6.1.7: Specifies version 6.1.7, which is compatible with the nRF52840 dongle's USB bootloader.

Step 7: Add ~/.local/bin to your PATH (if not already):

echo 'export PATH="$HOME/.local/bin:$PATH"' >> ~/.bashrc

source ~/.bashrc

Explanation: This ensures your shell can find the nrfutil command. The first command adds the export to your .bashrc file (runs every time you open a terminal). The second command reloads .bashrc immediately.

Step 8: Verify nrfutil is installed correctly:

nrfutil version

Expected output: nrfutil version 6.1.7

# 5.  Phase 1: Building and Flashing the nRF52840 RCP Firmware

This phase compiles the OpenThread RCP firmware from source code and flashes it onto the nRF52840 USB dongle. After this phase, the dongle will be ready to use as a Thread radio.

## 5.1  Understanding What We Are Doing

The nRF52840 dongle comes from the factory with either no firmware or demo firmware. We need to flash it with OpenThread RCP firmware, which turns it into a Radio Co-Processor that the Raspberry Pi can use for Thread communication.

The process involves: (1) downloading the OpenThread source code, (2) compiling it for the nRF52840 chip, (3) converting the compiled binary to a format the USB bootloader understands, and (4) flashing it via USB.

## 5.2  Clone the OpenThread nRF528xx Repository

Navigate to your home directory and clone the repository:

cd ~

git clone --recursive https://github.com/openthread/ot-nrf528xx.git

Explanation:

- cd ~: Changes to your home directory (e.g., /home/yourname).

- git clone: Downloads a copy of the repository from GitHub.

- --recursive: Also downloads submodules (dependencies). The OpenThread project includes the main OpenThread stack as a submodule.

This may take a few minutes depending on your internet speed. The repository is approximately 200-300 MB.

## 5.3  Navigate to the Repository

cd ot-nrf528xx

## 5.4  Build the RCP Firmware

Run the build script with the correct options:

./script/build nrf52840 USB_trans -DOT_BOOTLOADER=USB

Explanation of each argument:

- ./script/build: The build script provided by the OpenThread nRF528xx repository.

- nrf52840: Specifies the target chip. The nRF52840 is Nordic's most capable Bluetooth/Thread chip.

- USB_trans: Specifies USB transport. The dongle communicates with the Raspberry Pi over USB serial.

- -DOT_BOOTLOADER=USB: Tells the build system to configure the firmware for USB DFU bootloader. The dongle has a built-in USB bootloader that allows firmware updates without special hardware.

Build Progress: The build will show progress like "[123/924] Building CXX object...". This takes 5-15 minutes depending on your computer's speed.

Expected Result: The build completes with "[924/924] Linking CXX executable bin/ot-rcp". The compiled firmware is located at build/bin/ot-rcp.

WARNING: You may see linker warnings about unimplemented POSIX functions (_close, _fstat, _read, _write). These are expected and harmless for bare-metal firmware.

## 5.5  Convert to Intel HEX Format

The build produces an ELF binary, but the USB bootloader expects Intel HEX format:

arm-none-eabi-objcopy -O ihex build/bin/ot-rcp build/bin/ot-rcp.hex

Explanation:

- arm-none-eabi-objcopy: A tool that converts between binary formats.

- -O ihex: Output format is Intel HEX (a text-based format commonly used for programming microcontrollers).

- build/bin/ot-rcp: Input file (ELF format).

- build/bin/ot-rcp.hex: Output file (Intel HEX format).

## 5.6  Create DFU Package

Package the HEX file for USB DFU flashing:

nrfutil pkg generate --hw-version 52 --sd-req=0x00 --application build/bin/ot-rcp.hex --application-version 1 build/bin/ot-rcp.zip

Explanation of each argument:

- pkg generate: Creates a DFU package (a ZIP file containing the firmware and metadata).

- --hw-version 52: Hardware version identifier (52 for nRF52 series).

- --sd-req=0x00: SoftDevice requirement. 0x00 means no SoftDevice is required (we are not using Bluetooth in the RCP).

- --application: Path to the HEX file to include.

- --application-version 1: Version number embedded in the package (can be any number).

- build/bin/ot-rcp.zip: Output ZIP file.

## 5.7  Put the Dongle in Bootloader Mode

Physical steps:

- Plug the nRF52840 dongle into a USB port on your build computer.

- Press the small RESET button on the dongle (it is a tiny button near the USB connector).

- The LED on the dongle should start blinking RED. This indicates bootloader mode.

TIP: If the LED does not blink, try holding the RESET button for 1-2 seconds. Some dongles require a longer press.

## 5.8  Verify the Dongle is Detected

ls /dev/ttyACM*

Expected output: /dev/ttyACM0 (or similar). This is the USB serial port created by the dongle's bootloader.

## 5.9  Flash the Firmware

Use nrfutil to flash the DFU package:

sudo $(which nrfutil) dfu usb-serial -pkg build/bin/ot-rcp.zip -p /dev/ttyACM0

Explanation:

- sudo: Runs with administrator privileges (needed to access USB serial ports).

- $(which nrfutil): Finds the full path to nrfutil. This is needed because sudo may not have ~/.local/bin in its PATH.

- dfu usb-serial: Perform DFU update over USB serial connection.

- -pkg: Path to the DFU package (ZIP file).

- -p: Serial port to use (/dev/ttyACM0).

Expected output: Progress bar showing "Upgrading target..." followed by "Device programmed."

TIP: After successful flashing, the LED on the dongle will STOP blinking. This is correct behavior — the RCP firmware does not include any LED code.

## 5.10  Verify the Flash Was Successful

Unplug and replug the dongle, then verify it appears as a serial device:

ls /dev/ttyACM*

You should see /dev/ttyACM0. The dongle is now ready to be moved to the Raspberry Pi.

# 6.  Phase 2: Raspberry Pi Operating System Installation

This phase prepares the Raspberry Pi with its operating system and enables remote access via SSH.

## 6.1  Download Raspberry Pi Imager

Download Raspberry Pi Imager from: https://www.raspberrypi.com/software/

Install it on your build computer following the instructions for your operating system.

## 6.2  Write the Operating System

- Insert your microSD card into the card reader on your build computer.

- Open Raspberry Pi Imager.

- Click "CHOOSE OS" → "Raspberry Pi OS (other)" → "Raspberry Pi OS (64-bit)".

- Click "CHOOSE STORAGE" and select your microSD card.

- Click the gear icon (⚙) to open advanced options.

- Enable SSH (Use password authentication).

- Set username: pi

- Set password: raspberry (or a password of your choice)

- Optionally configure Wi-Fi (we use Ethernet, so this is not necessary).

- Click "SAVE" to close advanced options.

- Click "WRITE" and wait for the process to complete (5-15 minutes).

WARNING: The advanced options may not always apply correctly. See Section 11 for troubleshooting if SSH does not work.

## 6.3  (If Needed) Manually Enable SSH

If SSH is not working after boot, you can enable it manually by creating an empty file called 'ssh' on the boot partition:

sudo touch /media/$(whoami)/bootfs/ssh

Explanation: The Raspberry Pi checks for this file on boot. If it exists, SSH is enabled. The file is automatically deleted after enabling SSH.

## 6.4  (If Needed) Manually Create User Account

If you get "Permission denied" when logging in, create the user manually by creating a file called 'userconf.txt' on the boot partition:

echo 'pi:'$(echo 'raspberry' | openssl passwd -6 -stdin) | sudo tee /media/$(whoami)/bootfs/userconf.txt

Explanation:

- pi: The username.

- openssl passwd -6: Generates a SHA-512 encrypted password hash.

- tee: Writes the output to the file (with sudo for permissions).

## 6.5  Boot the Raspberry Pi

- Safely eject the SD card from your build computer.

- Insert the SD card into the Raspberry Pi.

- Connect the Ethernet cable from the Pi to a yellow LAN port on the TRENDnet router.

- Plug in the nRF52840 dongle (that you flashed earlier) to any USB port on the Pi.

- Connect the USB-C power supply to the Pi.

- Wait 60-90 seconds for the first boot to complete.

## 6.6  Find the Pi's IP Address

The Pi's IP address is permanently reserved in the TRENDnet router via DHCP reservation. It will always be assigned 192.168.10.102. To confirm it is online, scan the network:

nmap -sn 192.168.10.0/24

Look for raspberrypi.lan in the output. You can also log into the router admin page at http://tew-827dru (admin / Y7TQ4A6!) and check the DHCP client list.

NOTE: The Pi's DHCP reservation is bound to MAC address 88:a2:9e:84:96:c1. This guarantees the Pi always receives IP 192.168.10.102 regardless of what other devices are on the network.

## 6.7  Connect via SSH

From your build computer, open a terminal and connect:

ssh pi@192.168.10.102

Type 'yes' when asked about the fingerprint, then enter the password (raspberry). You should now have a command prompt on the Raspberry Pi.

## 6.8  Update the System

sudo apt update

sudo apt upgrade -y

Explanation: This updates the package list and installs all available updates. This may take 5-10 minutes.

## 6.9  Verify the Dongle is Detected

ls /dev/ttyACM*

Expected output: /dev/ttyACM0. This confirms the nRF52840 dongle is recognized.

# 7.  Phase 3: OpenThread Border Router Installation and Configuration

This phase installs and configures the OpenThread Border Router, which manages the Thread network and bridges it to your Ethernet network.

## 7.1  Clone the OTBR Repository

cd ~

git clone --depth=1 https://github.com/openthread/ot-br-posix

cd ot-br-posix

Explanation: --depth=1 performs a shallow clone (only the latest commit), which is faster and uses less disk space.

## 7.2  Run Bootstrap

./script/bootstrap

What this does: Installs all required dependencies including compilers, libraries, and tools. This takes 10-20 minutes.

## 7.3  Install Node.js and npm (If Not Already Installed)

The web GUI requires npm. If the setup script fails with 'npm not found', install it:

sudo apt install -y nodejs npm

## 7.4  Run Setup

INFRA_IF_NAME=eth0 WEB_GUI=1 ./script/setup

Explanation:

- INFRA_IF_NAME=eth0: Specifies the infrastructure interface (your Ethernet connection). This is where Thread devices will be bridged to.

- WEB_GUI=1: Enables the web-based management interface.

- ./script/setup: Compiles and installs OTBR. This takes 30-60 minutes.

## 7.5  Reboot

sudo reboot

Wait 60 seconds, then reconnect via SSH.

## 7.6  Form the Thread Network

Run these commands to create a new Thread network:

sudo ot-ctl dataset init new

What it does: Generates random network credentials (channel, PAN ID, network key, etc.)

sudo ot-ctl dataset commit active

What it does: Saves the credentials as the active dataset.

sudo ot-ctl ifconfig up

What it does: Enables the Thread network interface (wpan0).

sudo ot-ctl thread start

What it does: Starts Thread networking. The device will become the Leader since it is the first (and only) device.

## 7.7  Verify Thread Network Status

sudo ot-ctl state

Expected output: leader

## 7.8  Record Network Credentials

Run these commands and save the output — you will need these values for your QPG6200L devices:

sudo ot-ctl channel

sudo ot-ctl panid

sudo ot-ctl networkkey

Our values (yours will be different): Channel: 18, PAN ID: 0x1905, Network Key: 67b33c6dca3b4ccf96a56c318b887c39

## 7.9  Fix Web GUI Network Binding (Important!)

By default, the web GUI only listens on localhost (127.0.0.1), so you cannot access it from another computer. Fix this:

echo 'OTBR_WEB_OPTS="-I wpan0 -a 0.0.0.0"' | sudo tee /etc/default/otbr-web

sudo systemctl restart otbr-web

Explanation:

- -I wpan0: Specifies the Thread interface name.

- -a 0.0.0.0: Listen on all network interfaces (not just localhost).

You can now access the OTBR web interface at http://192.168.10.102 (replace with your Pi's IP if different).

# 8.  Phase 4: Mosquitto MQTT Broker Installation and Configuration

## 8.1  Install Mosquitto

sudo apt install -y mosquitto mosquitto-clients

What gets installed: mosquitto (the broker daemon) and mosquitto-clients (command-line tools for testing).

## 8.2  Configure Mosquitto for Network Access

Mosquitto 2.x defaults to localhost-only for security. We need to enable network access:

sudo nano /etc/mosquitto/mosquitto.conf

Add these lines at the end of the file:

listener 1883 0.0.0.0

allow_anonymous true

Explanation:

- listener 1883 0.0.0.0: Listen on port 1883 on all network interfaces.

- allow_anonymous true: Allow connections without username/password. (For production, configure authentication.)

Save and exit (Ctrl+O, Enter, Ctrl+X), then restart Mosquitto:

sudo systemctl restart mosquitto

## 8.3  Test Mosquitto

In one terminal, subscribe to a test topic:

mosquitto_sub -t test/hello -v

In another terminal (SSH session), publish a message:

mosquitto_pub -t test/hello -m "Hello from Mosquitto!"

The subscriber should display: test/hello Hello from Mosquitto!

# 9.  Phase 5: Node-RED and Dashboard 2.0 Installation

## 9.1  Install Node-RED

bash <(curl -sL https://raw.githubusercontent.com/node-red/linux-installers/master/deb/update-nodejs-and-nodered)

When prompted:

- "Are you really sure you want to do this?" → Type y and press Enter

- "Would you like to install the Pi-specific nodes?" → Type y and press Enter

Installation takes 10-20 minutes.

## 9.2  Enable and Start Node-RED

sudo systemctl enable nodered.service

sudo systemctl start nodered.service

What this does: enable makes Node-RED start automatically on boot. start starts it immediately.

## 9.3  Access Node-RED

Open a web browser and navigate to: http://192.168.10.102:1880

## 9.4  Install Dashboard 2.0

- In Node-RED, click the hamburger menu (☰) in the top-right corner.

- Select "Manage palette".

- Click the "Install" tab.

- Search for: @flowfuse/node-red-dashboard

- Click "install" next to the first result, then confirm.

# 10.  System Integration and End-to-End Testing

This section will be expanded as QPG6200L device firmware integration is completed. The gateway stack is fully operational. The next step is integrating Thread + MQTT client code into each QPG6200L device firmware so sensor data flows end-to-end to the dashboard.

## 10.1  Current Integration Status

| Integration Step | Status | Notes |
| --- | --- | --- |
| Gateway stack installed | Complete | OTBR, Mosquitto, Node-RED all running |
| Thread network formed | Complete | Leader active, Channel 18, PAN 0x1905 |
| Demo network configured | Complete | TRENDnet router, Pi at 192.168.10.102 |
| QPG6200L firmware (basic) | Complete | LED blink and sensor reading working |
| Thread stack in QPG6200L FW | In Progress | Adding Thread join + MQTT publish |
| End-to-end data flow | Pending | Requires Thread FW integration |
| Node-RED dashboard flows | Pending | Requires MQTT data to be flowing |

# 11.  Issues Encountered and Detailed Resolutions

## 11.1  SSH Not Working After First Boot

Symptom: Connection refused when trying to SSH to the Pi.

Cause: Raspberry Pi Imager advanced options did not apply SSH configuration correctly.

Resolution: Manually create the SSH enable file on the boot partition of the SD card from your Ubuntu laptop:

sudo touch /media/$(whoami)/bootfs/ssh

The Pi checks for this file on boot and enables SSH if it exists.

## 11.2  Permission Denied When Logging In via SSH

Symptom: SSH connects but rejects the password.

Cause: The pi user account was not created correctly during imaging.

Resolution: Manually create a userconf.txt file on the boot partition:

echo 'pi:'$(echo 'raspberry' | openssl passwd -6 -stdin) | sudo tee /media/$(whoami)/bootfs/userconf.txt

## 11.3  OTBR Web GUI Not Accessible from Network

Symptom: OTBR web interface only loads on the Pi itself (localhost), not from another computer.

Cause: Default OTBR web configuration binds to 127.0.0.1 only.

Resolution: Override the binding address:

echo 'OTBR_WEB_OPTS="-I wpan0 -a 0.0.0.0"' | sudo tee /etc/default/otbr-web

sudo systemctl restart otbr-web

## 11.4  Mosquitto Refusing Remote Connections

Symptom: MQTT clients on other machines cannot connect to port 1883.

Cause: Mosquitto 2.x changed the default to localhost-only connections for security.

Resolution: Add the following to /etc/mosquitto/mosquitto.conf:

listener 1883 0.0.0.0

allow_anonymous true

## 11.5  NetworkManager vs wpa_supplicant Confusion

Symptom: WiFi configuration commands not working; unclear which network manager is active.

Cause: Newer Raspberry Pi OS uses NetworkManager; older versions use wpa_supplicant. The two systems are incompatible.

Resolution: Check which is active:

systemctl status NetworkManager

systemctl status wpa_supplicant

If NetworkManager is active, use nmcli commands. If wpa_supplicant is active, edit /etc/wpa_supplicant/wpa_supplicant.conf directly.

## 11.6  Raspberry Pi WiFi Soft-Blocked by systemd-rfkill

Symptom: wlan0 shows as 'unavailable' in NetworkManager even after rfkill unblock wifi.

Cause: systemd-rfkill service saves and restores the rfkill state across reboots. It had saved the WiFi as blocked from a previous state and kept restoring it.

Resolution: Clear the saved rfkill state:

sudo systemctl stop systemd-rfkill

sudo rm -rf /var/lib/systemd/rfkill/*

sudo rfkill unblock all

NOTE: After extensive troubleshooting, the decision was made to use ethernet instead of WiFi for the Pi gateway connection. This is more reliable for demo purposes and eliminates all WiFi-related issues entirely.

## 11.7  Pi IP Address Changed After Network Switch

Symptom: Pi was previously at 192.168.1.69 (home network). After switching to TRENDnet demo router, old IP no longer works.

Cause: DHCP assigns a new IP on the new network. The TRENDnet router uses the 192.168.10.x subnet.

Resolution: Configured a DHCP reservation in the TRENDnet router admin page (http://tew-827dru) binding Pi MAC address 88:a2:9e:84:96:c1 to IP 192.168.10.102. The Pi will always receive this IP on this router.

# 12.  Final System Configuration Summary

## 12.1  Thread Network Credentials (SAVE THESE!)

These credentials must be programmed into your QPG6200L device firmware for them to join the Thread network:

| Channel | 18 |
| --- | --- |
| PAN ID | 0x1905 |
| Network Key | 67b33c6dca3b4ccf96a56c318b887c39 |

WARNING: Your network key is like a password. Keep it secure. Anyone with this key can join your Thread network.

## 12.2  System Credentials

| System | Username | Password / Notes |
| --- | --- | --- |
| Raspberry Pi SSH | pi | Set during OS install. Default: raspberry |
| Router Admin | admin | Y7TQ4A6! |
| MQTT Broker | N/A | Anonymous access enabled (no credentials required) |
| Node-RED | N/A | No authentication (local network only) |

NOTE: Change the default Raspberry Pi password for production use: SSH in and run 'passwd'

## 12.3  Network Configuration Summary

| Router Model | TRENDnet TEW-827DRU (H/W V2.0R) |
| --- | --- |
| Router Admin URL | http://tew-827dru or http://192.168.10.1 |
| Router Admin Password | Y7TQ4A6! |
| WiFi SSID | TRENDnet827_2.4GHz_QMGR |
| WiFi Password | None — open network |
| Pi IP Address | 192.168.10.102 (DHCP reservation) |
| Pi MAC Address | 88:a2:9e:84:96:c1 |

## 12.4  Port Reference

| Service | Port | Protocol / Notes |
| --- | --- | --- |
| Mosquitto MQTT | 1883 | TCP — all interfaces |
| Node-RED Editor | 1880 | HTTP — http://192.168.10.102:1880 |
| Node-RED Dashboard | 1880/dashboard | HTTP — http://192.168.10.102:1880/dashboard |
| OTBR Web GUI | 80 | HTTP — http://192.168.10.102 |
| SSH | 22 | TCP — ssh pi@192.168.10.102 |

# 13.  Quick Start Guide: Daily Operations

## 13.1  Starting the Gateway (From Power Off)

- Plug the TRENDnet router into a wall outlet (12V 1.5A adapter). Wait 60 seconds for router boot.

- Ensure the nRF52840 dongle is plugged into a USB port on the Pi.

- Connect the Ethernet cable from the Pi to a yellow LAN port on the router.

- Connect the USB-C power supply to the Pi.

- Wait 60-90 seconds for boot to complete.

- All services start automatically — no manual intervention needed.

## 13.2  Accessing the Interfaces

Connect your laptop and tablet to WiFi network TRENDnet827_2.4GHz_QMGR (no password), then use these URLs:

| OTBR Web GUI | http://192.168.10.102 |
| --- | --- |
| Node-RED Editor | http://192.168.10.102:1880 |
| Dashboard | http://192.168.10.102:1880/dashboard |
| SSH Access | ssh pi@192.168.10.102 (password: raspberry) |

## 13.3  Checking System Status

SSH into the Pi and run:

sudo systemctl status otbr-agent    # Thread Border Router

sudo systemctl status otbr-web      # OTBR Web GUI

sudo systemctl status mosquitto     # MQTT Broker

sudo systemctl status nodered       # Node-RED

sudo ot-ctl state                   # Thread network state (should be 'leader')

## 13.4  Shutting Down Safely

WARNING: Never just unplug the power! This can corrupt the SD card.

SSH into the Pi and run:

sudo shutdown -h now

Wait until the green ACT LED stops blinking (about 10 seconds), then disconnect power.

## 13.5  Restarting Services

If a service is not responding, restart it:

sudo systemctl restart otbr-agent

sudo systemctl restart otbr-web

sudo systemctl restart mosquitto

sudo systemctl restart nodered

# 14.  Troubleshooting Guide

## 14.1  Cannot SSH into the Pi

- Check: Pi is powered on and ethernet cable is connected to the TRENDnet router (yellow LAN port).

- Check: Your laptop is connected to TRENDnet827_2.4GHz_QMGR.

- Verify Pi is online:

nmap -sn 192.168.10.0/24

- If Pi is not shown: Check router DHCP client list at http://tew-827dru — Pi should be listed at 192.168.10.102.

## 14.2  Thread Network Not Starting

Check otbr-agent status:

sudo systemctl status otbr-agent

Check if the dongle is detected:

ls /dev/ttyACM*

If /dev/ttyACM0 is missing, unplug and replug the dongle. Restart the agent:

sudo systemctl restart otbr-agent

Check Thread state:

sudo ot-ctl state

If state is 'disabled', run the network formation commands from Section 7.6 again.

## 14.3  MQTT Messages Not Arriving in Node-RED

Verify Mosquitto is running:

sudo systemctl status mosquitto

Test with a manual publish:

mosquitto_pub -h 192.168.10.102 -t test/hello -m "test"

Check Node-RED MQTT input node is connected (green dot under the node in the editor).

## 14.4  Dashboard Not Loading

Verify Node-RED is running:

sudo systemctl status nodered

Try accessing the editor first: http://192.168.10.102:1880

If the editor loads but dashboard does not, check that @flowfuse/node-red-dashboard is installed via Manage Palette.

## 14.5  Services Not Starting After Reboot

Check that all services are enabled to start on boot:

sudo systemctl enable otbr-agent otbr-web mosquitto nodered

Then reboot and verify:

sudo reboot

# 15.  Appendix A: Complete Command Reference

## A.1  Build Computer Commands

sudo apt update && sudo apt upgrade -y

sudo apt install -y build-essential cmake ninja-build python3 python3-pip git gcc-arm-none-eabi binutils-arm-none-eabi

pip3 install --user nrfutil==6.1.7

echo 'export PATH="$HOME/.local/bin:$PATH"' >> ~/.bashrc && source ~/.bashrc

cd ~ && git clone --recursive https://github.com/openthread/ot-nrf528xx.git && cd ot-nrf528xx

./script/build nrf52840 USB_trans -DOT_BOOTLOADER=USB

arm-none-eabi-objcopy -O ihex build/bin/ot-rcp build/bin/ot-rcp.hex

nrfutil pkg generate --hw-version 52 --sd-req=0x00 --application build/bin/ot-rcp.hex --application-version 1 build/bin/ot-rcp.zip

sudo $(which nrfutil) dfu usb-serial -pkg build/bin/ot-rcp.zip -p /dev/ttyACM0

## A.2  Raspberry Pi Setup Commands

sudo apt update && sudo apt upgrade -y

ls /dev/ttyACM*

cd ~ && git clone --depth=1 https://github.com/openthread/ot-br-posix && cd ot-br-posix

./script/bootstrap

INFRA_IF_NAME=eth0 WEB_GUI=1 ./script/setup

sudo reboot

sudo ot-ctl dataset init new && sudo ot-ctl dataset commit active

sudo ot-ctl ifconfig up && sudo ot-ctl thread start

echo 'OTBR_WEB_OPTS="-I wpan0 -a 0.0.0.0"' | sudo tee /etc/default/otbr-web

sudo systemctl restart otbr-web

sudo apt install -y mosquitto mosquitto-clients

bash <(curl -sL https://raw.githubusercontent.com/node-red/linux-installers/master/deb/update-nodejs-and-nodered)

sudo systemctl enable nodered.service && sudo systemctl start nodered.service

## A.3  Daily Operations Commands

ssh pi@192.168.10.102

sudo systemctl status otbr-agent otbr-web mosquitto nodered

sudo ot-ctl state

sudo shutdown -h now

nmap -sn 192.168.10.0/24

# 16.  Appendix B: Configuration File Contents

## B.1  /etc/mosquitto/mosquitto.conf  (additions)

listener 1883 0.0.0.0

allow_anonymous true

## B.2  /etc/default/otbr-web

OTBR_WEB_OPTS="-I wpan0 -a 0.0.0.0"

## B.3  NetworkManager WiFi Profile (TRENDnet)

Location: /etc/NetworkManager/system-connections/TRENDnet.nmconnection

Permissions: 600 (root only)

[connection]

id=TRENDnet

type=wifi

autoconnect=true

[wifi]

mode=infrastructure

ssid=TRENDnet827_2.4GHz_QMGR

[wifi-security]

key-mgmt=none

[ipv4]

method=auto

[ipv6]

method=auto

NOTE: WiFi on the Pi is not used for the demo (ethernet is used instead). This file is present but the WiFi interface experienced persistent soft-blocking issues. See Section 11.6 for details.

# 17.  Appendix C: Glossary of Terms

| Term | Definition | Context |
| --- | --- | --- |
| Thread | A low-power mesh networking protocol for IoT devices based on IEEE 802.15.4 | Wireless communication between QPG6200L devices |
| OTBR | OpenThread Border Router — software that runs on the Pi to manage the Thread network | Core gateway software |
| RCP | Radio Co-Processor — the nRF52840 dongle running low-level radio firmware | Thread radio hardware |
| MQTT | Message Queuing Telemetry Transport — lightweight publish/subscribe messaging protocol | Data routing from devices to dashboard |
| Mosquitto | Open-source MQTT broker from Eclipse Foundation | Installed on Raspberry Pi |
| Node-RED | Visual flow-programming tool for wiring together IoT devices and services | Dashboard and data processing |
| wpan0 | The Thread network interface created by OTBR on the Raspberry Pi | Linux network interface |
| Spinel | Binary protocol used between the RCP dongle and the Pi over USB serial | Internal communication |
| DFU | Device Firmware Update — the process of flashing firmware to the nRF52840 via USB | Firmware flashing |
| IEEE 802.15.4 | The radio standard used by Thread (and Zigbee). Operates at 2.4 GHz with low power | Radio layer specification |
| IPv6 | Internet Protocol version 6 — Thread uses IPv6 natively for device addressing | Network addressing |
| DHCP Reservation | A router configuration that permanently assigns a specific IP to a device based on its MAC address | Network configuration |
| QPG6200L | Qorvo's multi-protocol SoC supporting Thread, Zigbee, and BLE concurrently | IoT device chip |
| SoC | System on Chip — a single integrated circuit containing processor, radio, and peripherals | Hardware term |

--- End of Document ---

Document v2.0 — Updated March 18, 2026  |  Original v1.0 — March 9, 2026