# EW840E5N / LEW840X Gateway Documentation

## 1. Overview

The LEW840X / EW840E5N Gateway is an open-source multiple network gateway supporting WiFi, Ethernet, and LTE-M (optional). It comes pre-loaded with demonstration firmware that sends temperature and humidity sensor data to a Fanstel MQTT server. The gateway can be monitored using the Fanstel MQTT PC tool or mobile app.

Data Flow:

MQTT Tool / App <-> MQTT Broker <-> LEW840X Gateway <-> BLE Sensor

Note: For easier testing and firmware loading, remove the gateway PCB from its enclosure.

1. Overview	1

2. Network Setup	1

Ethernet	1

WiFi	2

3. MQTT Setup	2

4. Firmware Development and Programming	2

ESP32 Firmware	2

nRF52840 Module	2

5. Next Steps	3

nRF9160 Firmware Programming	3

6. Useful Links	3

7. Why This Gateway Was Abandoned	4

## 2. Network Setup

### Ethernet

- Connect the UART_Bridge Board “TO WIFI(ESP)” into JS1 on the LEW840X PCB.

- Ensure the switch on the Bridge board is OFF.

- Connect the micro USB to a PC and open a terminal tool.

- Plug in Ethernet and reset the board.

- The gateway should obtain a DHCP IP address.

### WiFi

- Hold SW1 while resetting the board.

- The gateway enters AP mode.

- On a smartphone, connect to FanstelGW_XXXXX.

- Open a browser at 192.168.4.1 to set up your router SSID and password.

- Verify WiFi connection in the log.

## 3. MQTT Setup

- Connect the UART_Bridge Board “TO LOG” into JS1.

- Open the Fanstel MQTT PC tool and enter the gateway ID (e.g., esp32_XXXXXX).

- Confirm the temperature and humidity data is published from the gateway to the MQTT broker.

## 4. Firmware Development and Programming

### ESP32 Firmware

- Open mos.yml in the folder mqtt_sensorWiFi_Ethernet_esp32.

- Update your WiFi credentials:

- ["wifi.sta.ssid", "YourSSID"]

- ["wifi.sta.pass", "YourPassword"]

- Build firmware:

mos build --platform esp32

- Connect UART_Bridge Board “TO WIFI(ESP)” into JS1 with switch ON.

- Flash firmware:

mos flash --port COM9

### nRF52840 Module

- Connect nRF52840 DK or nRF9160 DK debug out to JS2.

- If using nRF9160 DK, set PROG/DEBUG switch to nRF52.

- Install nRF5x Command-Line Tools:
https://www.nordicsemi.com/Software-and-Tools/Development-Tools/nRF5-Command-Line-Tools

- Install nRF Connect desktop:
https://www.nordicsemi.com/Software-and-Tools/Development-Tools/nRF-Connect-for-desktop

## 5. Next Steps

### nRF9160 Firmware Programming

- Connect nRF9160 DK debug out to JS3.

- Open nRF Connect Programmer and load the SoftDevice and application.

- Erase and write firmware.

MQTT IDs:

- ESP32: e.g., 6CA674

- nRF9160: always starts with 91 (e.g., 91A674)

## 6. Useful Links

- LEW840X Product Specifications: PDF

- Arduino ESP32 driver: https://github.com/espressif/arduino-esp32/blob/master/docs/arduino-ide/windows.md

- Mongoose OS Quickstart:

  - Setup

  - Develop in C

- Mongoose OS Forum: https://community.mongoose-os.com/

- Nordic Documentation: https://infocenter.nordicsemi.com/index.jsp?topic=/com.nordic.infocenter.nrf52/dita/nrf52/development/nrf52_dev_kit.html&cp=1_1

- Nordic SDK: https://developer.nordicsemi.com/nRF5_SDK/nRF5_SDK_v15.x.x/

- Nordic Development Zone: https://devzone.nordicsemi.com/

- Preloaded Test Tools: Dropbox

## 7. Why This Gateway Was Abandoned

- Limited documentation and community support.

- Requires additional components for Thread and Zigbee connectivity (BLE only by default).

- Higher complexity for integration compared to other gateways.