# Technical Documentation: IoT Gateway Stack

# Project Context:

QPG6200L based IoT devices communicating over a Thread mesh network, managed by an OpenThread Border Router (OTBR) running on a Raspberry Pi 4 with an nRF52840 USB dongle as the Radio Co-Processor (RCP), with Mosquitto as the MQTT broker and Node-RED with Dashboard 2.0 as the user interface.

# Table of Contents

Project Context:	1

Table of Contents	1

1. System Architecture Overview	4

1.1 What we are Building	4

1.2 The Hardware Stack	4

1.3 Why Each Component is Needed	4

2. Understanding Every Protocol and Technology	5

2.1 Thread: The Wireless Mesh Protocol	5

2.2 IEEE 802.15.4: The Radio Standard	6

2.3 RCP vs NCP: Two Ways to Add Thread to a Linux Host	6

2.4 Spinel Protocol	6

2.5 MQTT: The Messaging Protocol	6

2.6 CoAp: Constrained Application Protocol	7

2.7 Mosquitto MQTT Broker	7

2.8 Node-RED	7

2.8.1 Dashboard 2.0	8

3. Phase 1: Flashing the nRF52840 Dongle as an RCP	8

3.1 What is Needed	8

3.2 Installing Build Dependencies	8

3.3 Installing nrfutil	9

3.4 Cloning the OpenThread nRF28xx Repository	9

3.5 Building the RCP Firmware	9

3.6 Converting to Intel HEX Format	9

3.7 Creating the DFU Package	10

3.8 Putting the Dongle into DFU Mode	10

3.9 Flashing the DFU Package	10

4. Phase 2: Installing OTBR on Raspberry Pi (Native Method)	11

4.1 Preparing the Raspberry Pi	11

4.2 Plug in the nRF52840 Dongle	11

4.3 Cloning the OTBR Repository	11

4.4 Running the Bootstrap Script	12

4.5 Running the Setup Script	12

4.6 Enabling the Web GUI (Optional but Recommended)	12

4.7 Starting OTBR Without Rebooting	13

4.8 Forming a Thread Network	13

5. Phase 3: Installing Moquitto MQTT Broker	14

5.1 Installing Mosquitto	14

5.2 Understanding Moquitto 2.x Security Change	14

5.3 Configuring Mosquitto for Network Access	14

5.4 Configuring Persistence (Recommended)	15

5.5 Testing Mosquitto	15

6. Phase 4: Installing Node-RED and Dashboard 2.0	16

6.1 Installing Node-RED Using the Official Script	16

6.2 Enabling Node-RED as a Service	16

6.3 Accessing the Node-RED Editor	16

6.4 Installing Dashboard 2.0	17

6.5 Creating Basic MQTT to Dashboard Flow	17

Step 1: Add an MQTT Input Node	17

Step 2: Add a JSON Node (if needed)	17

Step 3: Add a Function Node for Logic	18

Step 4: Add Dashboard Nodes	18

Step 5: Deploy	19

7. Phase 5: Configuring CoAp in Node-RED	19

7.1 Understanding When You Need This	19

7.2 Installing the CoAP Node	19

7.3 Setting Up a CoAP Server in Node-RED	19

Step 1: Drag a coap in node onto the workspace. Configure it:	19

Step 2: Add a coap response node connected to the coap in to send an acknowledgement back to the device.	20

Step 3: Connect the coap in output to your existing processing chain (json → function → dashboard nodes).	20

Step 4: The CoAP server in Node-RED listens on UDP port 5683 by default (the standard CoAP port). Your QPG6200L firmware needs to POST to coap://<raspberry-pi-mesh-local-ipv6>/sensors/motion.	20

7.4 CoAP Request Node (Polling Mode)	20

Step 1: Add an inject node configured to trigger every 5 seconds.	20

Step 2: Add a coap request node.	20

Step 3: Connect inject → coap request → json → function → dashboard.	20

7.5 CoAP Observe Mode (Push like from Devices)	20

8. Phase 6: Wiring Everything Together (The Data Flow)	21

8.1 The Motion Sensor Data Path	21

Step 1 (Physical):	21

Step 2 (Firmware):	21

Step 3 (Thread transmission):	21

Step 4 (Radio path):	21

Step 5 (Dongle → OTBR):	21

Step 6 (OTBR processing):	21

Step 7 (Linux routing):	22

Step 8 (Node-RED processing):	22

Step 9 (Dashboard display):	22

8.2 Designing Your MQTT Topic Hierarchy	22

8.3 The Commissioning Flow (BLE→Thread)	23

9. Verification and Testing at Every Stage	23

9.1 Verifying the RCP Dongle	23

9.2 Verifying OTBR	23

9.3 Verifying Mosquitto	23

9.4 Verifying Node-RED	24

9.5 Verifying the Full End-to-End Data Path	24

10. Troubleshooting Common Problems	24

10.1 Dongle Not Appearing as /dev/ttyACM0	24

Symptom: After plugging in the nRF52840 dongle, no /dev/ttyACM device appears.	24

Cause 1: The dongle was not flashed correctly.	24

Cause 2: USB cable is power-only (no data lines).	24

Cause 3: USB port permissions.	24

10.2 otbr-agent Fails to Start	25

Symptom: sudo service otbr-agent status shows failed or activating (start) stuck.	25

Cause: The serial port for the dongle is wrong in the service configuration.	25

10.3 Thread Devices Cannot Join	25

Symptom: QPG6200L device attempts to join but never connects.	25

Cause 1: Network Key mismatch.	25

Cause 2: Channel mismatch.	25

Cause 3: Commissioner window closed.	25

10.4 Mosquitto Refuses External Connections	25

Symptom: mosquitto_sub -h <pi-ip> -t test fails with "Connection refused".	25

10.5 Node-RED MQTT Node Shows "Disconnected"	26

Symptom: The MQTT in node shows "disconnected" under it in the flow editor.	26

10.6 Dashboard Not Showing Updates	26

Symptom: MQTT in node shows "connected" but dashboard gauge does not update.	26

Useful Links	26

# 1. System Architecture Overview

## 1.1 What we are Building

We have four custom wireless IoT devices based on the QPG6200L SoC – a motion sensor, a doorbell, a speaker, and a microphone. These devices need to:

- Talk to each other wirelessly over a mesh network (so they can relay messages even if they are far apart and not in direct range of the gateway).

- Send their data to a gateway (Raspberry Pi 4)

- Have that data displayed in a live web dashboard

## 1.2 The Hardware Stack

## 1.3 Why Each Component is Needed

The nRF52840 dongle is the actual radio. The Raspberry Pi has no 802.15.4 radio built in so it cannot speak Thread natively. The dongle plugs into USB and acts as a modem; the Raspberry Pi tells it what to do over a serial connection.

The OTBR (OpenThread Border Router) is the software running on the Raspberry Pi that manages the Thread network. It creates a virtual network interface called wpan0, handles routing IPv6 packets between the Thread mesh and the local Ethernet/Wi-Fi network, and provides a web GUI for network management.

Mosquitto is the MQTT broker. It is a message routing hub. The QPG6200L devices publish sensor data topics (e.g, home/motion/distance), Mosquitto receives those messages and distributes them to any subscriber. Node-Red subscribes to those topics.

Node-RED is the visual programming environment that subscribes to MQTT topics, processes the incoming messages (e.g., converts raw distance values to “motion detected” strings), and displays them on the Dashboard 2.0 web interface.

CoAP (Constrained Application Protocol) is relevant because Thread natively uses CoAp internally for network management (the Leader device uses CoAP to manage the router registry). When the QPG6200L devices communicate over thread, they use UDP-based IPv6 messaging. At the application layer the firmware can send data either as raw UDP, as CoAP, or you can have the gateway application translate incoming CoAP/UDP messages into MQTT. Node-RED can speak CoAP directly via the node-red-contrib-coap package, which mean you can receive CoAP messages from Thread devices and republish them as MQTT messages.

# 2. Understanding Every Protocol and Technology

## 2.1 Thread: The Wireless Mesh Protocol

Thread is a wireless mesh network protocol built on top of IEEE 802.15.4, which is the same radio standard that Zigbee uses. IEEE 802.15.4 is the physical radio layer (how bits are sent) and thread is the networking layer on top (how packets are addressed, routed, and delivered). Thread is IP-native. Every Thread device gets a real IPv6 address meaning the QPG6200L motion sensor has an IP address. You can ping it, send it a UDP packet or make CoAP request to it directly.

Thread is also a mesh network. Devices relay messages for each other. If the doorbell device is too fat from the Raspberry Pi gateway the motion sensor will automatically relay the doorbells messages. This is called multi-hop routing. The network self-heals: if one device drops out, the others re-route around it automatically.

The QPG6200L datasheet confirms it is fully IEEE 802.115.4 compliant and supports the 2.4 GHz band. It implements the real-time MAC (Medium Access Control) layer in hardware, handles CDMA/CA collision avoidance automatic ACKs, and address filtering. The SoC also supports ConcurrentConnect meaning it can run BLE commissioning simultaneously with Thread operation without dropping packets.

Thread network roles:

- Leader: One device per network, manages routing assignments. Uses CoAP protocol internally.

- Router: Always-on devices that forward packets. Can become the leader.

- End Device: Simple device, does not forward for others. Can be sleepy (turns radio off between transmissions to save battery).

- Border Router (OTBR): The gateway between the Thread mesh and the wider IP network (LAN/internet).

The QPG6200L devices will operate as Routers or End Devices based on how we configure our firmware.

## 2.2 IEEE 802.15.4: The Radio Standard

This is the physical and MAC layer that Thread (and Zigbee) operate on. Relevant facts:

- Frequency: 2.4 GHz same banc as Wi-FI and Bluetooth

- Data Rate: 250 kbits/s. Low data rate, but Thread is for sensor data, not streaming.

- Range: Typically 10-30 meters indoors. With the QPG6200L antenna diversity the effective range could increase.

- Power: Very low. The QPG6200L can sleep at 0.9 uA and wake up within 2.1 ms to receive or transmit.

## 2.3 RCP vs NCP: Two Ways to Add Thread to a Linux Host

When configuring a linux computer (raspberry pi) to a Thread Radio chip (nRF52840) there are two architectures:

NCP (Network Co-Processor): The full Thread protocol stack runs on the radio chip. The linux host just sends high-level commands and receives processed data. This is simpler but limits control.

RCP (Radio Co-Processor): Only the bare radio layer runs on the chip. The full Thread stack (OpenThread) runs on the linux host (raspberry pi). The host sends raw radio frames to the chip via the Spinel protocol over USB serial. This is what we will use and is also the recommended approach for OTBR because:

- You get full control over the Thread stack on the PI

- You can update Thread software without reflashing the radio chip.

- The official OTBR is designed for this RCP architecture.

- The nRF52840 dongle naturally fits this role because it is a small USB stick with 802.15.4 radio,

## 2.4 Spinel Protocol

This is the communication protocol between the raspberry pi and the nRF52840 dongle. This is implemented by OTBR. The OTBR agent talks to the dongle over this serial port (/dev/ttyACM0) using Spinel commands.

## 2.5 MQTT: The Messaging Protocol

MQTT stands for Message Queuing Telemetry Transport. It is a publish-subscribe protocol over TCP. Here is how it works:

- A broker (Mosquitto) runs on the Rasberry Pi and listens on port 1883.

- A publisher (A Node-RED function node, or QPG6200L Device application) connects to the broker and sends a message to a topic for example: home/motion/sensor1 with payload {“distance_cm”: 42, “motion”: true}.

- Any subscriber that previously told the broker “I want messages  on home/motion/#” immediately receives a copy of that message.

Why MQTT and not HTTP? HTTP requires full TCP handshake and headers for every message which is too heavy for a sensor sending readings every second. MQTT reuses a persistent TCP connection. The minimum overhead per message is just 2 bytes. It also has a quality of service levels: QoS 0 (fire and forget), QoS 1 (at least once), QoS 2 (exactly once). For sensor data, QoS 0 or 1 is appropriate.

## 2.6 CoAp: Constrained Application Protocol

CoAP is the IoT equivalent of HTTP, but running over UDP instead of TCP. THis matters because:

- Thread devices use IPv6 over UDP natively.

- CoAP messages fit into a single UDP packet.

- CoAP supports GET, POST, PUT, DELETE which are the same concepts as HTTP REST, so it is easy to understand

- CoAP has a confirmable mode where the receiver sends an acknowledgement

- CoAP has an observe mode where a client subscribes to a resource and the server pushes updates

In our architecture CoAP is relevant in two places. First, Thread uses CoAP internally for its management plane (the leader device handles CoAP requests). Second our QPG6200L application firmware can optionally expose sensor data as CoAP resources – the pi Running Node-RED can then make CoAP GET requests or listen for CoAP observe notifications. Alternatively, the firmware sends MQTT directly over Thread’s IPv6 network but both approaches are valid. The CoAp to MQTT bridge in Node-RED handles the translation if we go the CoAP route.

## 2.7 Mosquitto MQTT Broker

Mosquitto is the open source Eclipse Foundation MQTT broker. It is lightweight enough to run on a raspberry pi. Version 2.x has changed the default configuration compared to 1.x: by default it only accepts connections from localhost. We must explicitly configure a listener to allow our Thread devices to connect.

## 2.8 Node-RED

Node-RED is a visual flow based programming tool built on Node.js. We connect nodes together with wires to create a data processing pipeline. For our project:

- An mqtt in node subscribes to a topic on mosquitto and outputs the message payload whenever a new message arrives.

- A json node applies custom logic (e.g., if distance_cm changed by more than 2 cm, set motion = true)

- A ui_guage or ui_tect node from dashboard 2.0 displays the value in the web UI.

- A coap in or coap request node from node-red-contrib-coap receives or requests coAp messages

Node-RED runs on port 1880 and the Dashboard 2.0 UI is served on port 1880/ui. We just opne a browser to http://<raspberry-pi-ip>:1880/ui to see the dashboard.

### 2.8.1 Dashboard 2.0

Dashboard 2.0 uses Vue.js provides modern, responsive widgets. For our project we just drag and dashboard nodes onto our flows.

# 3. Phase 1: Flashing the nRF52840 Dongle as an RCP

We are converting the nRF52840 dongle from a NCP-firmware device into an RCP device that our raspberry pi’s OTBR software can control.

## 3.1 What is Needed

- Computer for compilation (Linux recommended)

- ARM GNU Toolchain (for compiling code targeting the nRF52840’s ARM Cortex-M4 processor).

- Cmake and ninja-build build system tools

- Nrfutil – Nordic Semiconductor’s common line tool for packaging and flashing firmware

- Python 3 with pip

- The nRF52840 dongle physically available.

## 3.2 Installing Build Dependencies

Open terminal and run:

sudo apt-get update

sudo apt-get install -y cmake ninja-build python3 python3-pip git

sudo apt-get install -y gcc-arm-none-eabi binutils-arm-none-eabi

Explanation of each package:

- cmake: the build system configuration tool. The OpenThread project uses CMake to describe how to compile the code.

- ninja-build: a fast build execution tool that CMake will use.

- python3 and python3-pip: needed for the nrfutil packaging tool.

- git: for cloning the source repositories.

- gcc-arm-none-eabi: the cross-compiler. "arm" means it targets ARM processors (the nRF52840 has a Cortex-M4 ARM core). "none" means no operating system. "eabi" is the embedded ABI calling convention. This compiler runs on your PC but produces code that runs on the nRF52840.

- binutils-arm-none-eabi: companion tools like objcopy for converting the compiled binary to different formats.

## 3.3 Installing nrfutil

nrfutil is Nordic Semiconductor's tool for creating DFU (Device Firmware Update) packages and flashing them to devices. The nRF52840 dongle has a built-in bootloader that accepts DFU packages which is how you flash it without a separate programmer.

pip3 install --no-dependencies nrfutil==6.1.7

The specific version 6.1.7 is important. Later versions changed their command interface. It can be verified with:

nrfutil version

## 3.4 Cloning the OpenThread nRF28xx Repository

Run the following commands to build and program the RCP firmware onto an nRF52840 Dongle. Clone the OpenThread nRF528xx platform repository into the current directory:

git clone --recursive https://github.com/openthread/ot-nrf528xx.git

cd ot-nrf528xx

The --recursive flag is critical. OpenThread is composed of many submodules. If you forget --recursive, the build will fail because source files will be missing.

## 3.5 Building the RCP Firmware

Once inside the ot-nrf528xx directory, run:

./script/build nrf52840 USB_trans -DOT_BOOTLOADER=USB

Breaking down each part of this command:

- `./script/build` — the build script provided by the repository. It sets up the CMake configuration and runs the compiler.

- `nrf52840` — specifies you are targeting the nRF52840 chip.

- `USB_trans` — specifies USB as the transport mechanism (how the RCP communicates with the host). This means the dongle will appear as a USB CDC-ACM device (a virtual serial port) when plugged in.

- `-DOT_BOOTLOADER=USB` — this flag configures the correct memory map for the pre-installed USB bootloader on the nRF52840 dongle. This flag is required for both RCP and NCP designs to configure the correct memory map for the pre-installed Bootloader and to enable the USB DFU trigger. If the flag is not present, the compiled firmware cannot be loaded onto the dongle.  This is a critical flag — forgetting it produces a binary that cannot be flashed over USB.

The build takes several minutes. When it completes, you will find the output at:

build/bin/ot-rcp

This is an ELF (Executable and Linkable Format) binary the compiled program. You need to convert it to a hex file, then package it for DFU.

## 3.6 Converting to Intel HEX Format

The DFU tool requires an Intel HEX format file:

arm-none-eabi-objcopy -O ihex build/bin/ot-rcp build/bin/ot-rcp.hex

arm-none-eabi-objcopy is a tool from binutils-arm-none-eabi. It converts between binary formats. -O ihex means "output format: Intel HEX". The Intel HEX format encodes binary data as ASCII text with addressing information.

## 3.7 Creating the DFU Package

Run the following command to generate the DFU package:

nrfutil pkg generate \

--hw-version 52 \

--sd-req=0x00 \

--application build/bin/ot-rcp.hex \

--application-version 1 \

build/bin/ot-rcp.zip

What each parameter means:

- pkg generate — creates a DFU package (a zip file containing the firmware and metadata).

- --hw-version 52 — hardware version of the chip (nRF52 series).

- --sd-req=0x00 — softdevice requirement. 0x00 means no softdevice required (OpenThread does not use Nordic's SoftDevice BLE stack; it uses its own direct radio access).

- --application build/bin/ot-rcp.hex — the firmware file to include.

- --application-version 1 — version number for the firmware.

- build/bin/ot-rcp.zip — the output DFU package file.

## 3.8 Putting the Dongle into DFU Mode

Put the dongle in DFU mode by simply pressing the RESET button. The reset button is on the far side of the board from the USB connector.

When the dongle is in DFU mode, the red LED on the dongle starts blinking repeatedly. You will also see a new device appear on your system you can verify with:

ls /dev/tty*

Or check dmesg | tail -20 to see the USB device detection messages.

## 3.9 Flashing the DFU Package

Flash the DFU package using nrfutil:

nrfutil dfu usb-serial -pkg build/bin/ot-rcp.zip -p /dev/ttyACM0

Where /dev/ttyACM0 is the port where the dongle appeared in DFU mode. If it appears as a different port (like /dev/ttyACM1), adjust accordingly.

After flashing:

- The LED on the dongle will stop blinking (this is normal the RCP firmware has no LED blinking code).

- The dongle is now permanently flashed as an RCP. If you ever need to update it, you put it in DFU mode again and repeat this process.

- Remove the dongle from your build machine

# 4. Phase 2: Installing OTBR on Raspberry Pi (Native Method)

You take the nRF52840 dongle and plug it into the Raspberry Pi 4. This phase installs and configures the OpenThread Border Router software on the Pi.

## 4.1 Preparing the Raspberry Pi

Start with a fresh Raspberry Pi OS installation (64-bit Raspberry Pi OS Lite is recommended for a headless, resource-efficient setup, but the Desktop version also works). Once booted, connect via SSH or directly with a keyboard/monitor.

Update the system first outdated packages can cause dependency conflicts:

sudo apt-get update

sudo apt-get upgrade -y

Install git:

sudo apt-get install -y git

## 4.2 Plug in the nRF52840 Dongle

Physically insert the flashed nRF52840 dongle into a USB port on the Raspberry Pi. Verify it was detected:

ls /dev/tty*

You should see /dev/ttyACM0 appear. This is the virtual serial port through which OTBR will communicate with the dongle. If it does not appear, try a different USB port. You can also check dmesg | tail -20 and look for lines mentioning "cdc_acm" or "nRF52840".

## 4.3 Cloning the OTBR Repository

On Raspberry Pi, clone ot-br-posix from GitHub:

git clone --depth=1 https://github.com/openthread/ot-br-posix

cd ot-br-posix

The --depth=1 flag is a "shallow clone" it downloads only the latest commit, not the full history. This is faster and saves disk space on the Pi's SD card.

## 4.4 Running the Bootstrap Script

OTBR has two scripts that bootstrap and set up the Thread Border Router:

The first script, bootstrap, downloads and installs all the dependencies that OTBR needs to compile and run. Run it:

./script/bootstrap

This script installs many packages including cmake, ninja-build, avahi-daemon (for mDNS discovery), radvd (router advertisement daemon, needed for IPv6 routing), ipset, nftables, and others.

Important: During this process, the script may disable the Pi's Wi-Fi interface if it detects a NetworkManager configuration. This is by design OTBR takes over some network management. If you are relying on Wi-Fi for your internet connection during this process, connect via Ethernet cable instead.

## 4.5 Running the Setup Script

The second script compiles and installs the actual OTBR software. You must tell it which network interface connects to your infrastructure (Ethernet or Wi-Fi):

If your Raspberry Pi uses Ethernet:

INFRA_IF_NAME=eth0 ./script/setup

If your Raspberry Pi uses Wi-Fi:

INFRA_IF_NAME=wlan0 ./script/setup

OTBR works on both a Thread interface and infrastructure network interface (e.g. Wi-Fi/Ethernet) which is specified with INFRA_IF_NAME. The Thread interface is created by OTBR itself and named wpan0 by default.

This script will:

- Compile the OTBR agent from source using CMake and Ninja.

- Install the compiled binary to /usr/sbin/otbr-agent.

- Create and enable a systemd service called otbr-agent that starts automatically on boot.

- Configure radvd for IPv6 routing.

- Configure firewall rules.

## 4.6 Enabling the Web GUI (Optional but Recommended)

If you want the OTBR web interface (accessible via browser to form the Thread network), add WEB_GUI=1 to both scripts:

WEB_GUI=1 ./script/bootstrap

INFRA_IF_NAME=eth0 WEB_GUI=1 ./script/setup

## 4.7 Starting OTBR Without Rebooting

The otbr-agent service is now enabled and will start upon reboot. To instead start the service immediately without rebooting, use the server script:

./script/server

Or simply reboot:
sudo reboot

After rebooting, verify OTBR is running:

sudo service otbr-agent status

You should see output similar to:

otbr-agent.service - Border Router Agent

Loaded: loaded (/lib/systemd/system/otbr-agent.service; enabled)

Active: active (running) since ...

Main PID: 2997 (otbr-agent)

otbr-agent[2997]: Border router agent started.

otbr-agent[2997]: [INFO]-CORE----: Notifier: StateChanged [NetData PanId NetName ExtPanId]

The key line to look for is `Border router agent started.` which confirms the agent is running and has established communication with the RCP dongle.

## 4.8 Forming a Thread Network

Access the OTBR web GUI by opening a browser on any device on your LAN and navigating to:

http://<raspberry-pi-ip-address>

(Port 80 — the web GUI runs on the standard HTTP port.)

In the web interface, click "Form" to create a new Thread network. You will be prompted for:

- Network Name: Give it a recognizable name, e.g., QPG6200L-IoT.

- PAN ID: A 16-bit identifier for your network. Default is fine.

- Channel: Which 802.15.4 channel to use (11–26). Default is fine.

- Extended PAN ID: 64-bit identifier. Default (randomly generated) is fine.

- Network Key: 128-bit encryption key. This is the master key that all devices must share to join the network. Note it down — you will need to program it into your QPG6200L firmware.

- Passphrase: Used by the Commissioner to generate the PSKc (Pre-Shared Key for the Commissioner). This is different from the Network Key.

Click "Form" and the OTBR will initialize the Thread network. The wpan0 interface will come up.

Verify the Thread interface is active:

sudo ot-ctl state

You should see leader — your Raspberry Pi's OTBR has become the Thread Leader (the first and so far only device managing the network).

sudo ot-ctl ipaddr

This will show the IPv6 addresses assigned to the OTBR on the Thread network a link-local address (starting with fe80::), a mesh-local address (starting with fd), and potentially a routable address.

# 5. Phase 3: Installing Moquitto MQTT Broker

Now that the Thread network exists, you need the MQTT broker that will receive messages from your IoT devices and feed them to Node-RED.

## 5.1 Installing Mosquitto

sudo apt-get update

sudo apt-get install -y mosquitto mosquitto-clients

mosquitto is the broker itself. mosquitto-clients installs two command-line tools: mosquitto_pub (for publishing test messages) and mosquitto_sub (for subscribing to topics). These are invaluable for testing.

After installation, Mosquitto starts automatically. Verify it:
sudo systemctl status mosquitto
Enable it to start on boot (it likely already is, but to be safe):
sudo systemctl enable mosquitto

## 5.2 Understanding Moquitto 2.x Security Change

This is a critical point. With version 2.0, by default, Mosquitto only allows connections from localhost for security reasons. This means your IoT devices (which are on a different IP address, even if they are on the same LAN through Thread) cannot connect to Mosquitto unless you change the configuration.

Check the current version:
mosquitto -v 2>&1 | head -3

## 5.3 Configuring Mosquitto for Network Access

You need to edit the Mosquitto configuration file:

sudo nano /etc/mosquitto/mosquitto.conf

Find the existing content and add the following at the end. For a development/testing environment with no authentication:

listener 1883 0.0.0.0

allow_anonymous true

For a production environment with username/password authentication:

listener 1883 0.0.0.0

allow_anonymous false

password_file /etc/mosquitto/passwd

If you chose the password option, create a user:
sudo mosquitto_passwd -c /etc/mosquitto/passwd your_username

You will be prompted to enter and confirm a password. Note: the -c flag creates the password file. If you run it again with -c for a second user, it will overwrite the file. Use -b to append additional users.

Save the configuration file (Ctrl+O, Enter, Ctrl+X in nano) and restart Mosquitto:
sudo systemctl restart mosquitto

## 5.4 Configuring Persistence (Recommended)

Mosquitto can save messages with the "retain" flag to disk, so subscribers that connect later still get the last known value. Add these lines to `/etc/mosquitto/mosquitto.conf`:
persistence true

persistence_location /var/lib/mosquitto/

log_dest file /var/log/mosquitto/mosquitto.log

Restart after changes:
sudo systemctl restart mosquitto

## 5.5 Testing Mosquitto

Open two terminal windows on the Raspberry Pi (or two SSH sessions).

In terminal 1, subscribe to a test topic:

mosquitto_sub -t test/hello -v

In terminal 2, publish a test message:
mosquitto_pub -t test/hello -m "Hello from Mosquitto"

You should immediately see test/hello Hello from Mosquitto appear in terminal 1. If you do, Mosquitto is working correctly. This confirms the pub-sub mechanism functions before involving any IoT devices.

# 6. Phase 4: Installing Node-RED and Dashboard 2.0

## 6.1 Installing Node-RED Using the Official Script

We provide a script to install Node.js, npm and Node-RED onto a Raspberry Pi and other platforms running a Debian based OS. Running the following command will download and run the script: 
sudo apt install -y build-essential git curl

bash <(curl -sL https://github.com/node-red/linux-installers/releases/latest/download/update-nodejs-and-nodered-deb)

During installation, you will be asked two questions:

- "Are you really sure you want to do this?" type y and press Enter.

- "Would you like to install the Pi-specific nodes?" type y and press Enter. These include useful nodes for Raspberry Pi GPIO.

The script completes five different tasks: removes any pre-existing Node-RED and Node.js installations, installs the current Node.JS LTS release using the NodeSource package repository, then uses npm to install the latest version of Node-RED.

## 6.2 Enabling Node-RED as a Service

Enable Node-RED to start automatically on boot:
sudo systemctl enable nodered.service

sudo systemctl start nodered.service

Verify it is running:
sudo systemctl status nodered.service

You should see active (running)

## 6.3 Accessing the Node-RED Editor

Open a browser on any device on the same network and navigate to:

http://<raspberry-pi-ip-address>:1880

You will see the Node-RED flow editor. The left panel is the palette (available nodes). The center is the workspace. The right panel shows node configuration, debug output, and the information panel.

## 6.4 Installing Dashboard 2.0

In the Node-RED editor, click the hamburger menu (≡) in the top right corner, then select "Manage palette". Click the "Install" tab. In the search box, type:

@flowfuse/node-red-dashboard

Click the Install button next to it. Wait for the installation to complete. You will need to confirm any prompts. After installation, you will see a set of ui_* nodes appear in the palette under a "Dashboard 2.0" category.

Alternatively, install via the terminal:

node-red-stop

cd ~/.node-red

npm install @flowfuse/node-red-dashboard

node-red-start

## 6.5 Creating Basic MQTT to Dashboard Flow

This guide will help create your first working flow. This flow will receive MQTT messages from your IoT devices and display the values on the dashboard.

### Step 1: Add an MQTT Input Node

Drag an mqtt in node from the palette (under "network") onto the workspace. Double-click it to configure it:

- Server: Click the pencil icon to add a new server. Enter localhost as the server address and 1883 as the port (since Mosquitto runs on the same Pi). If you set up authentication, fill in the username and password on the Security tab. Click Add.

- Topic: Enter the topic your devices will publish to, e.g., home/motion/sensor1. You can use wildcards: home/motion/# subscribes to all motion sensor topics.

- QoS: 0 for simplicity.

- Output: auto-detect (parsed JSON, string or buffer).

- Name: Give it a descriptive name like "Motion Sensor 1 In".

Click done

### Step 2: Add a JSON Node (if needed)

If your devices publish JSON payloads as strings (common), add a json node after the mqtt in node. Connect the output of mqtt in to the input of json. Double-click the json node, set Action to "Always convert to JSON object". This ensures the payload is a parsed JavaScript object regardless of whether it arrived as a string or already parsed.

### Step 3: Add a Function Node for Logic

Add a function node. This lets you write custom JavaScript. Connect the json output to this node. Inside the function node, write:

// Example: extract distance from payload and determine motion

var distance = msg.payload.distance_cm;

var prev = context.get('prev_distance') || 0;

if (Math.abs(distance - prev) > 2) {

msg.payload = {

distance_cm: distance,

motion: true,

status: "Movement detected!"

};

} else {

msg.payload = {

distance_cm: distance,

motion: false,

status: "No movement"

}

}

context.set('prev_distance', distance);

return msg;

context.get and context.set are Node-RED's way of storing values between messages within a single node (node context). This mirrors the logic in your main.c firmware file where prev_distance is used to detect changes.

### Step 4: Add Dashboard Nodes

From the palette (Dashboard 2.0 section), drag a ui_text node onto the workspace. Connect the function output to it. Configure:

- Group: Create a new Group (and a new Page if needed). This is the dashboard layout container.

- Label: "Motion Status"

- Value format: {{msg.payload.status}}

Add a ui_gauge for the distance value:

- Label: "Distance (cm)"

- Value:{{msg.payload.distance_cm}}

- Min/Max: 0 / 400

### Step 5: Deploy

Click the red "Deploy" button in the top right. The flows are now active. Access the dashboard at:

http://<raspberry-pi-ip-address>:1880/dashboard

(Dashboard 2.0 serves at /dashboard rather than /ui in the older version.)

# 7. Phase 5: Configuring CoAp in Node-RED

CoAP integration allows Node-RED to directly communicate with Thread devices using the CoAP protocol over UDP the same protocol Thread uses natively for its management plane.

## 7.1 Understanding When You Need This

You need the CoAP node when your QPG6200L firmware sends data as CoAP messages over the Thread IPv6 network. The OTBR routes these IPv6 packets to the Raspberry Pi's network stack, and Node-RED can receive them using CoAP.

If instead your firmware publishes MQTT directly (by connecting over Thread to the Raspberry Pi's Mosquitto broker at its mesh-local IPv6 address), you do not need CoAP in Node-RED. Both approaches are valid; CoAP is the more lightweight option for constrained devices.

## 7.2 Installing the CoAP Node

Install the node-red-contrib-coap package. This module requires at least Node version 12.

Via the Node-RED Palette Manager: search for node-red-contrib-coap and click Install.

Via terminal:

node-red-stop

cd ~/.node-red

npm install node-red-contrib-coap

node-red-start

After restart, you will see three new nodes in the palette:

- coap in acts as a CoAP server endpoint (listens for incoming requests)

- coap request acts as a CoAP client (sends requests to a CoAP server)

- coap response sends a response back in a coap in flow

## 7.3 Setting Up a CoAP Server in Node-RED

To receive data pushed from your QPG6200L devices via CoAP POST:

### Step 1: Drag a coap in node onto the workspace. Configure it:

- Method: POST (your devices will POST sensor data to a resource URI)

- URL: /sensors/motion (or whatever path your firmware uses)

### Step 2: Add a coap response node connected to the coap in to send an acknowledgement back to the device.

### Step 3: Connect the coap in output to your existing processing chain (json → function → dashboard nodes).

### Step 4: The CoAP server in Node-RED listens on UDP port 5683 by default (the standard CoAP port). Your QPG6200L firmware needs to POST to coap://<raspberry-pi-mesh-local-ipv6>/sensors/motion.

To find the Raspberry Pi's mesh-local IPv6 address on the Thread network:

sudo ot-ctl ipaddr

Look for the address starting with `fd` (the mesh-local prefix you configured when forming the network). This is the IPv6 address your devices will target.

## 7.4 CoAP Request Node (Polling Mode)

Instead of devices pushing data, Node-RED can poll devices periodically. This is useful for devices that act as CoAP servers exposing resources:

### Step 1: Add an inject node configured to trigger every 5 seconds.

### Step 2: Add a coap request node.

Configure:

- Method: GET

- URL: coap://[<device-thread-ipv6-address>]/sensor/distance (use square brackets for IPv6 in URLs)

- Confirmable: checked (ensures delivery acknowledgement)

- Content-Format: text/plain

### Step 3: Connect inject → coap request → json → function → dashboard.

The coap request node supports methods GET, PUT, POST, DELETE, FETCH, PATCH, and iPATCH. It can be configured for confirmable messages, observe mode (for subscribe-like behavior), and multicast.

## 7.5 CoAP Observe Mode (Push like from Devices)

CoAP's observe extension lets Node-RED subscribe to a resource on a device and receive automatic updates when the value changes — very similar to MQTT subscribe:

In the `coap request` node, check the "Observe" checkbox. Node-RED will send a GET request with the Observe option set to 0. The device firmware will then send new values whenever they change, without Node-RED needing to poll.

# 8. Phase 6: Wiring Everything Together (The Data Flow)

This section describes the complete end-to-end data path from physical measurement to dashboard display.

## 8.1 The Motion Sensor Data Path

Here is exactly what happens from the moment your ultrasonic sensor detects movement to when the dashboard updates:

### Step 1 (Physical):

The MaxBotix LV-MaxSonar-EZ MB1000-000 ultrasonic sensor emits a pulse. The echo time is measured by your QPG6200L firmware running in the `sensor_Task` FreeRTOS task in `main.c`.

### Step 2 (Firmware):

The firmware calculates the distance in cm using (time_us * 343) / 20000. If the distance change exceeds 2 cm (your prev_distance threshold), the device determines movement has occurred.

### Step 3 (Thread transmission):

The QPG6200L firmware transmits a message over the Thread network. This could be a CoAP POST to the Pi's IPv6 address with a JSON body like {"distance_cm": 42, "motion": true}, or it could be an MQTT PUBLISH over a TCP connection to Mosquitto's IPv6 address.

### Step 4 (Radio path):

The 802.15.4 radio in the QPG6200L chip (ConcurrentConnect mode) transmits the IEEE 802.15.4 frame. If other Thread routers are in between, they relay the packet (mesh routing). Eventually the packet reaches the nRF52840 dongle.

### Step 5 (Dongle → OTBR):

The nRF52840 dongle's RCP firmware receives the 802.15.4 frame and passes it to the Raspberry Pi over USB using the Spinel protocol. The `otbr-agent` process on the Pi receives this raw frame.

### Step 6 (OTBR processing):

The otbr-agent reconstructs the IPv6 packet from the Thread frame and injects it into the Linux network stack via the wpan0 interface.

### Step 7 (Linux routing):

The Linux kernel on the Pi receives the IPv6 packet on `wpan0` and routes it to the appropriate process. If it is a UDP/CoAP packet destined for port 5683, Node-RED's CoAP server receives it. If it is a TCP packet destined for port 1883, Mosquitto receives it.

### Step 8 (Node-RED processing):

The mqtt in or coap in node receives the message. The flow processes it through json and function nodes, applies the same motion detection logic, and sends the result to the dashboard nodes.

### Step 9 (Dashboard display):

The browser, which has the dashboard page open, receives an update via WebSocket (Node-RED Dashboard uses WebSocket to push updates to the browser in real time without needing a page refresh). The distance value and motion status update on screen.

## 8.2 Designing Your MQTT Topic Hierarchy

A well-designed topic structure makes your system maintainable. Here is a recommended hierarchy for our four-device system:

home/

devices/

motion_sensor_1/

distance_cm

motion_detected

status

doorbell_1/

button_pressed

state

speaker_1/

status

volume

microphone_1/

status

audio_level

alerts/

motion

doorbell

Using this structure, Node-RED can subscribe to home/# to receive everything, or home/devices/motion_sensor_1/# to receive only the motion sensor topics. Mosquitto handles the routing based on topic matching.

## 8.3 The Commissioning Flow (BLE→Thread)

Before a QPG6200L device can join the Thread network, it needs to be commissioned it needs to receive the Network Key and other network credentials. The QPG6200L supports BLE for commissioning: it advertises via BLE, a commissioner (which can be the OTBR's commissioning agent, or a mobile app) connects via BLE and transfers the Thread credentials, then the device joins the Thread network.

The OTBR provides a Commissioner. On the OTBR web GUI, you can open a commissioning window that allows new devices to join. The QPG6200L firmware's BLE peripheral mode handles this joining process.

After successful commissioning, the device's radio transitions from BLE advertising to 802.15.4 Thread mode, joins the mesh, gets an IPv6 address, and starts communicating.

# 9. Verification and Testing at Every Stage

Never proceed to the next phase without verifying the current phase works. Here are the verification commands for each stage.

## 9.1 Verifying the RCP Dongle

After plugging the flashed dongle into the Pi:

ls -la /dev/ttyACM0

Should show the device exists. Then:

sudo ot-ctl state

After OTBR is running, this should return leader (if the network is formed) or disabled (if not yet formed — that is fine before you form the network).

## 9.2 Verifying OTBR

sudo service otbr-agent status

Look for active (running) and Border router agent started. in the output.

Check the Thread interface exists:

ip link show wpan0

Should show the wpan0 interface with <BROADCAST,MULTICAST,UP> flags.

Ping within the Thread network:

sudo ot-ctl ping <thread-device-ipv6-address>

## 9.3 Verifying Mosquitto

# Test pub/sub locally

mosquitto_sub -t test/# -v &

mosquitto_pub -t test/sensor -m '{"value": 42}'

# You should see: test/sensor {"value": 42}

kill %1

Test remote access from another machine on your LAN:

mosquitto_sub -h <raspberry-pi-ip> -t test/# -v

If this fails, check your mosquitto.conf has the listener configured for 0.0.0.0.

## 9.4 Verifying Node-RED

sudo systemctl status nodered.service

Open browser to http://<pi-ip>:1880 — you should see the flow editor.

## 9.5 Verifying the Full End-to-End Data Path

Simulate a device message using the command-line MQTT client from the Pi itself:

mosquitto_pub -h localhost -t home/devices/motion_sensor_1/distance_cm -m '{"distance_cm": 42, "motion": true}'

If your Node-RED flow is set up correctly (subscribed to home/devices/motion_sensor_1/#), the dashboard should update within milliseconds.

For CoAP testing, install the coap-cli tool:

npm install -g coap-cli

Then send a test CoAP request to your Node-RED CoAP server:

coap post coap://localhost/sensors/motion -p '{"distance_cm": 35}'

You should receive a CoAP 2.01 Created response, and the data should flow through Node-RED to the dashboard.

# 10. Troubleshooting Common Problems

## 10.1 Dongle Not Appearing as /dev/ttyACM0

### Symptom: After plugging in the nRF52840 dongle, no /dev/ttyACM device appears.

#### Cause 1: The dongle was not flashed correctly.

Fix: Re-enter DFU mode (hold reset button) and re-flash.

#### Cause 2: USB cable is power-only (no data lines).

Fix: Use a different USB cable known to support data transfer.

#### Cause 3: USB port permissions.

Fix: Add your user to the dialout group: sudo usermod -a -G dialout $USER, then log out and back in.

## 10.2 otbr-agent Fails to Start

### Symptom: sudo service otbr-agent status shows failed or activating (start) stuck.

#### Cause: The serial port for the dongle is wrong in the service configuration.

Fix: Check the actual port:

ls /dev/ttyACM*

Edit the service configuration:

sudo nano /etc/default/otbr-agent

Look for the line containing spinel+hdlc+uart:///dev/ttyACM0 and change ttyACM0 to the correct port if needed.

## 10.3 Thread Devices Cannot Join

### Symptom: QPG6200L device attempts to join but never connects.

#### Cause 1: Network Key mismatch.

Fix: Ensure the Network Key in your firmware matches the OTBR network's key. Get the key from OTBR: sudo ot-ctl networkkey.

#### Cause 2: Channel mismatch.

Fix: Check sudo ot-ctl channel and ensure your device is scanning the correct channel.

#### Cause 3: Commissioner window closed.

Fix: On OTBR web GUI, open the commissioner window again. The default window is only open for a limited time.

## 10.4 Mosquitto Refuses External Connections

### Symptom: mosquitto_sub -h <pi-ip> -t test fails with "Connection refused".

Fix: Verify your /etc/mosquitto/mosquitto.conf has listener 1883 0.0.0.0 and restart: sudo systemctl restart mosquitto.

Check if the port is actually listening:

sudo ss -tlnp | grep 1883

## 10.5 Node-RED MQTT Node Shows "Disconnected"

### Symptom: The MQTT in node shows "disconnected" under it in the flow editor.

Fix: Check that the broker address in the node configuration matches exactly. If Node-RED and Mosquitto are on the same Pi, use localhost. Check Mosquitto is running: sudo systemctl status mosquitto. Check authentication: if you configured a password, ensure the node has the correct credentials.

## 10.6 Dashboard Not Showing Updates

### Symptom: MQTT in node shows "connected" but dashboard gauge does not update.

Fix 1: Ensure you have deployed the flow (clicked the red Deploy button after making changes).

Fix 2: Open the Debug panel (bug icon on the right). Add a debug node connected to your MQTT in node's output. Look for incoming messages.Pr

Fix 3: Check the topic in your mqtt in node matches exactly what the device is publishing. Topics are case-sensitive. home/Motion and home/motion are different topics.

# Useful Links

CoAP configuration on Node-RED

Raspberry Pi Node-Red Use MQTT To Communicate Between Two Raspberry Pi Devices

Thread / Matter BorderRouter with Raspberry Pi and nRF52840-dongle

How to Install OTBR (OpenThread Border Router) on a Raspberry Pi

🚀 Build an MQTT Broker & Node-RED Dashboard on Raspberry Pi 4 | Hub360 Embedded Sphere

Part 3: Installing and Using MQTT Broker (Mosquitto) & Node-Red - HackMD

OpenThread Border Router Setup

Configuring OpenThread Radio Co-processor on nRF52840 Dongle — Matter documentation

Create an RCP USB dongle | Golioth Thread Demo