QORVO_RADIO2 Project - Lessons Learned - Senior Design II

| Date | Revision | Author | Comments |
| --- | --- | --- | --- |
| 2026-02-27 | Initial Draft | Abdul B. | Original Document |
| 2026-03-03 | Rev1 | Luke M. | Added Cases 1-4 |
| 2026-03-10 | Rev2 | Abdul B. | Added Cases 5-6 |

Table of Contents

1 Project Overview/Information 	1

2 Summary	1

3 Lessons Learned	2

4 Conclusion	2

# Overview of this Document

# This document will serve as a record of challenges each team member encountered throughout the duration of this project as well as their corresponding resolutions and insights to these impediments. The structure of this document will allow each group member to document the specific obstacles faced, while also allowing each of them to analyze the root causes, and speak about the lessons that they learned throughout the process. This will allow them to facilitate problem solving skills, while growing their professional development within their respective areas of work within the project.

# Project Overview/Information:

Qorvo has tasked the team with developing an improved radio-based IoT control system as a follow-up to a previous senior design project. The problem was the need for a more capable, multi-device radio platform that could reliably transmit and receive data across different protocols while remaining portable and user-friendly.

The proposed solution was to design and build a set of battery-powered remote modules using the Qorvo QPG6200L SoC. These modules would support multiple peripherals, use bi-directional communication with a web interface, and demonstrate practical IoT control capabilities. The primary project deliverable was a fully functional multi-device IoT system ready for demonstration at the senior design expo.

3 Summary:

4 Lessons Learned:

| Case | Description | Impact | Cause | Action | Lesson Learned | Team Member |
| --- | --- | --- | --- | --- | --- | --- |
| J-Link Circuit revision | Revised J-Link Circuit by adding a 10kOhm pulldown resistor for the GPIO5/PROG_ENn pin on the QPG6200L. This ensures that the GPIO5 pin will be in a low state enabling the programming mode for software integration. | The lack of a pull down resistor on the QPG6200L doesn't let the QPG6200L enter program mode. Program mode allows software integration, further delaying project completion. | Unclear and incomplete understanding of J-Link connection circuit for programing functionality. | Revised PCB drawing with 10kOhm pull down resistor in GPIO5 pin. | Thoroughly and double check pinout diagrams and tables for all connections to ensure component connection functionality. | Luke M. |
| Battery Holder Footprint Revision | Revised battery holder footprint on KiCad to match datasheet. The initial footprints did not have the appropriate positive and negative terminal connections creating unusable connections for power. | Without appropriate pads the batteries cannot transfer power to the circuit. Since the board was designed with external pins for power connection the board can still be powered, just not by the desired coin cell batteries. | Rushed design process of footprint creation, caused oversight of appropriate pad connections. | Revised PCB drawing with appropriate pad connections | Double check all footprints after completing PCB design. | Luke M. |
| Small Footprint sizes | Selected an Inductor that had footprints smaller than can assemble with ease. | Due to the footprint size the time of assembly was greatly increased to place a simple SMD inductor. | Incomplete understanding of footprint and device sizes. | Revise PCB with footprints for a component of identical inductance with larger footprint. | Double check component sizes of components that will be difficult to assemble. | Luke M. |
| Manufacturer selection | Initially select JLC PCB as the desired manufacture for all PCB’s. Due to tariffs UNC Charlotte cannot support orders from China. The team switched to OSH Park without extensive research into their manufacturing spec. This required a circuit redesign for an IC footprint that was smaller than minimum spec. | Switching manufactures delayed assembly by a month, and required circuit redesign. | Unclear understanding of JLC ordering issues. | Order from OSH Park instead. | Check with ISL puchusing for vendor confirmation. | Luke M. |
| Current measurement | Due to the configuration of the PCB, current was a little more challenging to measure until we removed the resistor at R1. | Due to this, there will need to be more resistors put in place on PCB to make testing for current effective. | Improper way for testing for current. | PCB revisions to allow for proper measurements of current to be performed. | An innovative way of measuring current was found through troubleshooting for the best option. | Abdul B. |
| Power Supply | The original power source (coin cell batteries) didn’t work well with the PCB. The components on the PCB were drawing more power than the coin cells could handle. | The power being taken was too much for the coin cell and they were not a good match for what we needed. | Not enough power being given to the PCB | We worked with the idea to use an external power supply. | Thoroughly research the proper specs and how they will go with the project before ordering components. | Abdul B. |

5 Conclusion: