Power Supply Redesign

Objective

The purpose of this document is to document the process of redesigning the power supply circuit for the replicated PCB board. The chosen power supply CR2032 from Murata Electronics supports the voltage and mAh specifications when connected in series but does not support the constant amp draw specification. An alternative power supply will need to be sourced and implemented to ensure remote device functionality.

Minimum Specifications

- V = 3V (2 supplies connected in series)

- V = 6V (Single supply)

- mAh = 500

- I = 50 mA

Solution

UBBL20-FL

The UBBL20-FL would act as a single power supply solution with the following specs:

- Voltage range: 6-8.4V

- Max current discharge: 2.5A continuous

- 2400 mAh

- Rechargerable

- $57.76 each

Although this solution is low in stock (102 left on mouser), this solution indicates that similar products are available.

UBBP01

The UBBP01 would act as a series connected power supply, requiring two packs per board.

- Voltage range: 3-4.2V

- Max current discharge: 1.8A continuous

- 1800 mAh

- Rechargeable

- $35.72 each

- High in stock

The UBBP01 will need a cell balancing module to ensure healthy battery life expectancy, such as the BQ29209DRBR. The B2B-PH-K-S 2 pin connector will be used to connect the batteries.

The 1 Ohm resistor will need to be replaced by a 0.1 Ohm resistor in order to meet the power spec of the Schottky diode.

A fuse on the power voltage trace is optional but recommended to protect from overcurrent due to the large increase in amperage dissipation from the upgraded batteries.

Per sponsor recommendation the BQ29209DRBR IC solution was scratched. The lithium-ion batteries will be charged using a device such as the Adafruit 4410.