Slide 5

| Engineering Standards |  |  |
| --- | --- | --- |
| Standard | What It Is | How It Will Be Applied |
| IPC-2221 | PCB Design Standard | Used to guide the design, layout, and fabrication of custom PCBs for remote modules, ensuring reliability, manufacturability, and electrical integrity. |
| IEEE 802.3 | Ethernet Standard for Wired LAN Communication | Applied to any wired network interfaces or connections between the gateway and web interface for stable data transfer. |
| IEEE 802.11 | Wi-Fi Standard Operating in the 2.4 GHz ISM Band | Implemented for Wi-Fi connectivity in the gateway or modules to enable bi-directional communication with the web interface. |
| IEEE 802.15.1 | Bluetooth/BLE Standard for short-range wireless communication | Utilized in remote modules for low-energy, short-range transmission and receiving, supporting at least one of the required IoT protocols. |
| IEEE 802.15.4 | Zigbee and Thread Protocol foundation for low-power mesh networking | Applied to enable mesh topology in remote devices, supporting Zigbee or Matter over Thread for multi-device control and bi-directional communication. |
| Matter Protocol Specification | Application layer standard for secure, interoperable smart devices | Integrated as an application layer on top of Thread or Zigbee to ensure secure, interoperable control of remote devices via the web interface. |
| FCC Title 47 CFR Part 2 | Covers frequency allocation and equipment authorization | Ensured for all transmitting devices to comply with frequency allocation requirements in the 2.4-2.5 GHz ISM band. |
| FCC Title 47 CFR Part 15 | Regulations for unlicensed RF devices (e.g., Wi-Fi, Bluetooth, Zigbee) | Applied to verify that remote modules and the overall system operate within unlicensed limits without causing interference. |
| FCC Title 47 CFR Part 18 | Regulations for ISM equipment | Used to ensure compliance for industrial, scientific, and medical equipment operating in the ISM band. |

Slide 9

| Demo Idea |  | Smart Watch |  | Entryway |  | Greenhouse |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Weight | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score |
| Power Consumption | 5 | 2 | 10 | 4 | 20 | 4 | 20 |
| Device Interface | 3 | 1 | 3 | 5 | 15 | 3 | 9 |
| Enclosure Size | 5 | 2 | 10 | 4 | 20 | 4 | 20 |
| Peripheral Used | 1 | 5 | 5 | 5 | 5 | 5 | 5 |
| Weight | 2 | 3 | 6 | 4 | 8 | 5 | 10 |
| Creativity | 5 | 5 | 25 | 4 | 20 | 4 | 20 |
| Max Score: | 105 | Total: | 59 | Total: | 88 | Total: | 84 |

Slide 13

Slide 16

| Doorbell |  | RP3502ARED |  | PR144C1900 |  |
| --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight (1-5) | Score | Weighted Score | Score | Weighted Score |
| QPG6200L Peripheral | 5 | 5 | 25 | 5 | 25 |
| Power Supply Required (V) | 1 | 5 | 5 | 5 | 5 |
| Estimated battery life | 5 | 5 | 25 | 5 | 25 |
| Lead Time | 2 | 5 | 10 | 5 | 10 |
| Cost | 3 | 5 | 15 | 4 | 12 |
| Estimated size (3"x3"x3") | 2 | 4 | 8 | 4 | 8 |
|  |  | Total: | 88 | Total: | 85 |

Slide 20

| Microphone |  | 3492 |  | IM69D127 |  |
| --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight (1-5) | Score | Weighted Score | Score | Weighted Score |
| Cost | 1 | 3 | 3 | 5 | 5 |
| Power Supply Required (V) | 3 | 5 | 15 | 5 | 15 |
| Estimate Battery Life | 5 | 3 | 15 | 5 | 25 |
| Lead Time | 2 | 5 | 10 | 5 | 10 |
| Estimated weight (< 0.5 lbs) | 4 | 5 | 20 | 5 | 20 |
| Estimated size (3"x3"x3") | 4 | 2 | 8 | 5 | 20 |
|  |  | Total: | 71 | Total: | 95 |

Slide 22

| Door Lock |  | YRD410-BLE-BSP |  | Tapo D225 |  |
| --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight (1-5) | Score | Weighted Score | Score | Weighted Score |
| QPG6200L Peripheral | 5 | 5 | 25 | 5 | 25 |
| Power Supply Required (V) | 1 | 3 | 3 | 5 | 5 |
| Estimated battery life | 5 | 4 | 20 | 4 | 20 |
| Lead Time | 2 | 5 | 10 | 5 | 10 |
| Cost | 3 | 5 | 15 | 4 | 12 |
| Estimated size (3"x3"x3") | 2 | 4 | 8 | 4 | 8 |
|  |  | Total: | 81 | Total: | 80 |

Slide 24

| Motion detection |  | Ultrasonic Sensor |  | Motion Sensor |  |
| --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight (1-5) | Score | Weighted Score | Score | Weighted Score |
| Cost | 1 | 3 | 3 | 5 | 5 |
| Power Supply Required (V) | 3 | 5 | 15 | 3 | 9 |
| Estimate Battery Life | 5 | 3 | 15 | 5 | 25 |
| Lead Time | 2 | 5 | 10 | 3 | 6 |
| Estimated weight (< 0.5 lbs) | 4 | 5 | 20 | 5 | 20 |
| Estimated size (3"x3"x3") | 4 | 5 | 20 | 4 | 16 |
|  |  | Total: | 83 | Total: | 81 |

Slide 27

| Antenna |  | MEA-5000-LP-CM |  | MEA-5000-LP-CM |  |
| --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight (1-5) | Score | Weighted Score | Score | Weighted Score |
| Cost | 1 | 3 | 3 | 5 | 5 |
| Standards | 5 | 5 | 25 | 3 | 15 |
| Lead Time | 2 | 5 | 10 | 2 | 4 |
| Estimated weight (< 0.5 lbs) | 2 | 5 | 10 | 5 | 10 |
| Estimated size (3"x3"x3") | 5 | 4 | 20 | 3 | 15 |
| Frequency Range | 5 | 5 | 25 | 5 | 25 |
|  |  | Total: | 68 | Total: | 49 |

Slide 30

Slide 31

| Wireless Protocols |  | Zigbee |  | BLE |  | Thread Over Matter |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Section Criteria/Category | Weight | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score |
| Range | 5 | 4 | 20 | 3 | 15 | 4 | 20 |
| Power Consumption | 1 | 4 | 4 | 5 | 5 | 4 | 4 |
| Data Rate | 3 | 2 | 6 | 4 | 12 | 2 | 6 |
| Scalability (Network Size) | 3 | 5 | 15 | 2 | 6 | 4 | 12 |
| Interoperability | 4 | 3 | 12 | 5 | 20 | 5 | 20 |
| Topology Support | 4 | 5 | 20 | 3 | 12 | 5 | 20 |
| Security | 3 | 4 | 12 | 4 | 12 | 5 | 15 |
| Max Score: | 115 | Total | 89 | Total | 82 | Total | 97 |

Slide 32

| User Display |  | PineTab2 |  | Lenovo Tab M10 Plus 3rd Gen |  | Panasonic Toughbook |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Weight | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score |
| Cost | 4 | 5 | 20 | 5 | 20 | 2 | 8 |
| OS Compatibility | 5 | 5 | 25 | 5 | 25 | 4 | 20 |
| Interactivity | 4 | 4 | 16 | 4 | 16 | 4 | 16 |
| Battery Life | 3 | 4 | 12 | 4 | 12 | 5 | 15 |
| Durability | 2 | 3 | 6 | 3 | 6 | 5 | 10 |
| Ports | 3 | 3 | 9 | 3 | 9 | 5 | 15 |
| Size/Weight | 2 | 4 | 8 | 5 | 10 | 4 | 8 |
| Visual Engagement | 3 | 4 | 12 | 5 | 15 | 4 | 12 |
| Compatibility with Project | 5 | 4 | 20 | 5 | 25 | 4 | 20 |
| Max Score: | 155 | Total: | 128 | Total: | 138 | Total: | 124 |

Slide 34

| User Interface |  | Custom Website |  | ThingsBoard |  |
| --- | --- | --- | --- | --- | --- |
| Category | Weight | Score | Weighted Score | Score | Weighted Score |
| Development Effort | 5 | 2 | 10 | 5 | 25 |
| Customization | 5 | 5 | 25 | 3 | 15 |
| Cost | 4 | 3 | 12 | 5 | 20 |
| Learning Curve | 4 | 2 | 8 | 4 | 16 |
| Scalability | 3 | 4 | 12 | 5 | 15 |
| Integration with IoT | 5 | 3 | 15 | 5 | 25 |
| Time Spent | 4 | 2 | 8 | 5 | 20 |
| Visual Engagement | 3 | 4 | 12 | 3 | 9 |
| Compatibility | 5 | 4 | 20 | 4 | 20 |
| Max Score: | 190 | Total: | 122 | Total: | 165 |

Slide 36

| Gateway |  | EW840E5N |  | Raspberry Pi 5 |  |
| --- | --- | --- | --- | --- | --- |
| Category | Weight | Score | Weighted Score | Score | Weighted Score |
| Cost | 4 | 4 | 16 | 3 | 12 |
| Protocol Support | 5 | 5 | 25 | 3 | 15 |
| Ease of Setup | 4 | 4 | 16 | 3 | 12 |
| Range | 3 | 5 | 15 | 3 | 9 |
| Power | 2 | 4 | 8 | 4 | 8 |
| Bi-directional Communication | 5 | 5 | 25 | 5 | 25 |
| Mesh Topology Support | 3 | 5 | 15 | 4 | 12 |
| Independence | 3 | 4 | 12 | 5 | 15 |
| Compatibility | 5 | 5 | 25 | 4 | 20 |
| Size/Weight | 2 | 3 | 6 | 4 | 8 |
| Max Score: | 180 | Total: | 163 | Total: | 136 |

Slide 42

Slide 43: Budget

| Category | Item | Quantity | Unit Price | Subtotal | Notes |
| --- | --- | --- | --- | --- | --- |
| Mechanical | Plastic Case | 1 | 72.00 | 72.00 | Enclosures for remote devices |
| Mechanical | PLA Filament | 1 | 24.00 | 24.00 | For 3D printing custom parts |
| Mechanical | M3 Inserts | 1 | 18.00 | 18.00 | Threaded inserts for assembly |
| Mechanical | M3 Bolts | 1 | 24.00 | 24.00 | Fasteners for enclosures |
| Mechanical Subtotal |  |  |  | 138.00 |  |
| Electrical | Doorbell | 6 | 3.00 | 18.00 | For remote sensing use case |
| Electrical | Ultrasonic Sensor | 6 | 36.00 | 216.00 | Distance/proximity sensors |
| Electrical | LED Strip | 1 | 53.00 | 53.00 | For lighting control demo |
| Electrical | Microphone | 6 | 0.50 | 3.00 | For remote microphone use case |
| Electrical | Speaker | 2 | 28.00 | 56.00 | Audio output peripherals |
| Electrical | DAC | 6 | 4.25 | 25.50 | Digital-to-analog converters |
| Electrical | Audio Amplifier | 6 | 0.60 | 3.60 | For speaker amplification |
| Electrical | J-Link | 1 | 960.00 | 960.00 | Debugging tool for QPG6200L ) |
| Electrical | J-Link Adapter | 1 | 50.00 | 50.00 | Adapter for J-Link |
| Electrical | Batteries | 20 | 0.82 | 16.40 | Coin cell for remote devices |
| Electrical | Battery Holder | 20 | 1.63 | 32.60 | Holders for batteries |
| Electrical | Door Lock | 1 | 203.99 | 203.99 | For smart lock demo |
| Electrical | PCB | 1 | 365.00 | 365.00 | Custom PCB for 6 assemblies |
| Electrical | Antenna | 12 | 13.30 | 159.60 | For RF connectivity |
| Electrical | QPG6200L | 10 | 4.68 | 46.80 | For RF connectivity |
| Electrical Subtotal |  |  |  | 2,209.49 |  |
| Computer | EW840E5N Gateway | 1 | 114.62 | 114.62 | Multi-protocol (BLE/Thread/Zigbee) to WiFi/Ethernet |
| Computer | PK-LEW840X Programmer | 1 | 36.00 | 36.00 | Required for gateway programming |
| Computer | Lenovo Tab M10 Plus 3rd Gen | 1 | 228.00 | 228.00 | Display the user sees |
| Computer Subtotal |  |  |  | 378.62 |  |
| Grand Total (With 20% Safety Factor) |  |  |  | 2,726.11 | Covers all hardware for IoT demos, enclosures, and control |