Air Quality Monitor

- Description: A device measuring CO2, VOCs, and temperature; displays data with pollution alerts. Enclosure is more rugged for outdoor use

- Peripherals: ADC (gas/temperature sensors), PWM (LED pollution indicators), USB (data export).

- Protocols: Bluetooth LE for mobile sync, Zigbee for indoor mesh.

Sensors

CO2: Sensirion SCD41

- I2C interface

- 2.4-5.5V

- Average current of 0.5mA in low-power mode

- Size 10x10x7mm

- Compatible via QPG6200L’s I2C

- Low Power for coin cell (Samples every 5s, sleeps otherwise)

- https://www.digikey.com/en/products/detail/sensirion-ag/SCD41-D-R1/13684000?msockid=00e65354aaa86af21c164779ab1f6bb3

VOCs (Volatile organic compounds): Bosch BME680

- I2C Interface

- 2.4-5.5V

- Average current of 0.1-3mA

- Size 3x3x0.93mm

- Measures VOCs, temperature, humidity

- https://www.digikey.com/en/products/detail/bosch-sensortec/BME680/7401317?msockid=00e65354aaa86af21c164779ab1f6bb3

Temperature: TMP36

- Analog Output

- 2.5-5.5V

- Redundant with BME680 but adds analog peripheral use

Position Tracker

- Description: Attach-on tags reporting GPS and status via web interface. Enclosure is tamper-resistant.

- Peripherals: ADC (GPS/accelerometer/vibration sensor)), UART (vehicle interface), PWM (status LED).

- Protocols: Zigbee for convoy mesh, Thread for secure command links.

Sensors:

GPS: u-bloc NEO-M9N

- UART/I2C

- 1.65-3.6V

- Average current 25mA during acquisition but 8mA continuous

- 16x12x2.4mm

- https://www.digikey.com/en/products/detail/u-blox/NEO-M9N-00B/12149174?msockid=00e65354aaa86af21c164779ab1f6bb3

Accelerometer: STMicroelectronics LIS3DHTR

- I2C/SPI

- 1.71-3.6V

- Ultra low power 2uA in low power mode

- 3x3x1,,

- Connect via I2C for motion detection

- https://www.digikey.com/en/products/detail/stmicroelectronics/LIS3DHTR/2334355

Vibration: Piezo Vibration Sensor SW-18010P

- Analog

- 3.5V

- Quiescent <1uA

- 5mm diameter

- https://www.sunrom.com/p/vibration-sensor-sw-18010p

The Things Network 
https://www.thethingsnetwork.org/

Fix matrices

Buck converter or 5V for battery

CDR Charlotte

1 Main board with add on board

Arduino Hats/ Stackable shields

JTEG Programmer

Separate Physical board with same base design

So layout one board with all the sensors we may use and then just program them differently

https://www.qorvo.com/design-hub/design-tools/interactive/energy-budget-calculator

Briefly Touch on Previous Project or not Even bring it up