### Concerns with Using Only USB for the LED Strip

#### 1. Control and Data Transfer Issues

- No data control: Strip uses USB for power only; QPG6200L's USB Device mode can't send commands for colors/dimming.

- Lacks programmability: Can't integrate IoT features like app control; still needs physical IR remote.

#### 2. Hardware and Integration Challenges

- Connector limitations: AU-Y1005 USB-A works for power but adds no control value; data lines (DP/DM) unused by strip.

- Underutilizes SoC: Ignores peripherals like GPIO/IR Generator, potentially wasting PCB space.

#### 3. Reliability and User Experience Concerns

- Manual control required: Defeats IoT purpose.

- Interference potential: USB traces near 2.4 GHz antennas may cause noise

### IR Generator Solution Overview

#### 1. Hardware Setup

- Add 940nm IR LED (~$0.10) to GPIO0 (high-drive sink, 100mA for 10-20 ft range).

- Point LED at strip's IR receiver; extend with wire if needed.

- Power strip via USB-A and 5V regulator (upgrade to 1A+ if necessary).

#### 2. Software Configuration

- Use IR Generator modes: Pattern (0/1 sequences), Time (ON/OFF durations), Event (trigger on commands).

- Supports NEC protocol (38kHz carrier); program on ARM Cortex-M4 with no CPU overhead.

#### 3. Obtaining IR Codes

- Decode remote: Use TSOP4838 receiver (~$1) on spare GPIO; capture pulses with code (e.g., IRLib-adapted).

- Example codes for similar strips: On (0xFF00FF), Off (0xFF9867), Red (0xFF1AE5)—test/adjust.