- Zigbee

Zigbee is a low power, wireless mesh networking protocol based on the IEEE 802.15.4 standard, designed for short range IoT applications like home automation, smart lighting, and industrial sensing.

- Architecture and How It Works:

  - Operates on a star, tree, or mesh topology, where devices (end nodes, routers, coordinators) form self healing networks.

  - Uses Direct Sequence Spread Spectrum (DSSS) modulation for reliable data transmission.

  - Data rates: Up to 250 kbps

  - Range: 10-100 meters indoors, extendable via mesh hopping

  - Security: AES-128 encryption, with network and application layer keys

  - Power Consumption: Very low, ideal for battery powered devices (e.g., sleeps most of the time, wakes for shirt bursts)

- Key Features:

  - Mesh networking: Routers relay messages, improving coverage and reliability

  - Profiles: Application specific stacks like Zigbee home automation or Zigbee light link

  - Interoperability: Devices from different vendors can join the same network if certified by the Zigbee Alliance (Connectivity Standards Alliance, CSA)

  - Peripherals integration: Supports interfaces like UART, SPI, I2C, which align with QPG6200L’s peripherals (e.g., PWM for lighting, ADC for sensing)

- Advantages:

  - Excellent for large scale networks (up to 65000 nodes)

  - Low latency for control applications

  - Mature ecosystem with low cost chips

- Disadvantages:

  - Lower data rates compared to WiFi or Bluetooth

  - Potential interference in crowded 2.4 GHz band

  - Not natively IP-based, requiring gateways for internet connectivity

- Relevance to Project

  - Supports required mesh topology and bi-directional communication

  - Could be combined with matter for cross protocol control

  - Zigbee could pair with BLE for a hybrid demo

- Thread

Thread is an IP-based mesh networking protocol for low-power IoT, also built on IEEE 802.15.4, emphasizing secure, reliable home and building automation.

- Architecture and How It Works:

  - Mesh topology with border routers connecting to the internet (IPv6 addressing via 6LoWPAN)

  - Devices: Full Thread Devices (routers), Minimal Thread Devices (end nodes), and Border Routers.

  - Data rates: Up to 250 Kbps

  - Range: Similar to Zigbee (10-100m, extendable via mesh)

  - Security: DTLS (Datagram Transport Layer Security) over UDP, with device commissioning and encryption.

  - Power: Ultra-low, with sleepy end devices for battery life

- Key Features:

  - Native IPv6 support: Seamless integration with IP networks, no need for protocol translation

  - Self healing mesh: Automatic rerouting if a node fails

  - Commissioning: Easy device joining via NFC or out of band methods

  - Scalability: Supports hundreds of devices in a network

- Advantages

  - IP compatibility makes it future proof for cloud integration

  - Better security than older protocols

  - Low power ideal for always on meshes

- Disadvantages:

  - Requires a border router for internet access

  - Higher complexity in setup compared to bluetooth

  - Limited to low data rate applications

- Relevance to Project:

  - Thread over matter for mesh demos

  - UART/USB for configuration, PWM/ADC for application use cases like remote sensing

- Matter

Matter is an open-source, IP based application layer protocol for smart home IoT, running over existing transports like WiFi, Ethernet, Thread or Bluetooth.

- Architecture and How It Works

  - Not a standalone protocol; Its a unified standard on top of others (e.g., Matter over Thread uses Thread’s mesh for transport)

  - Uses IPv6 and UDP/TCP for communication

  - Data rates: Dependent on underlying protocols (e.g., 250 kbps over Thread)

  - Range: Inherits from transport layer (e.g., mesh-extended via Thread)

  - Security: Strong cryptography with device attestation, secure commissioning via QR codes or NFC

  - Ecosystem: Backed By CSA, Apple, Google, Amazon for cross-platform compatibility

- Key Features:

  - Multi-admin: Devices controllable by multiple ecosystems (e.g., Alexa, Google Home).

  - Device types: Standardized clusters for lights, sensors, etc

  - Local control: Works offline via local network.

  - Over the air updates: For security and features.

- Advantages:

  - Solves fragmentation; one protocol for all smart devices

  - High interoperability across vendors

  - Future proof with IP focus

- Disadvantages:

  - Still emerging so ecosystem is growing

  - REquires compatible hardware/chips

  - Overhead from IP can slightly increase power use

- Relevance to Project:

  - Enables the required interoperability

  - Could implement matter clusters for bidirectional web control

  - Could bridge matter over thread to control interface for stretch goal

- Bluetooth/BLE (Bluetooth low energy)

Bluetooth is a short range wireless protocol; BLE is its low power variant optimized for IoT

- Architecture and How It Works:

  - Classic Bluetooth: Point to point or star topology for higher data (audio/streaming)

  - BLE: Star or mesh (via Bluetooth mesh) with central/peripheral roles.

  - Data rates: Classic up to 3 Mbps; BLE up to 2 Mbps (Bluetooth 5+)

  - Range: 10 - 100m; extendable with mesh

  - Security: AES-128, pairing modes (e.g., LE Secure connections)

  - Power: BLW is ultra low with advertising beacons for discovery

- Key Features

  - GATT profiles: For services like heart rate or battery level

  - Mesh: Bluetooth Mesh for many to many networking (flooding based)

  - Audio: LE Audio for low power streaming (e.g., microphone use case)

  - Beasons: For proximity sensing

- Advantages:

  - Ubiquitous (built into phones/tablets)

  - Fast connection times

  - Versatile for both data and audio

- Disadvantages:

  - Classic Bluetooth higher power than BLE

  - Mesh can be less efficient than Thread/Zigbee in large networks

  - Interference in 2.4 GHz

- Relevance to project:

  - Supports use cases like microphones (using PDM)

  - Pairs with Zigbee for multi protocol control