### 1. Overview

- The IoT system consists of a User Interface (Tablet/App), a gateway, and multiple IoT devices such as a speaker & microphone, movement detector, door lock, and doorbell.

- Each device communicates with the gateway using a specific IoT protocol (BLE, Zigbee, or Thread), while the gateway connects to the user interface through Wi-Fi.

- This layered communication ensures low power usage, secure connectivity, and reliable data transmission.

### 2. Data Direction and Communication Flow

#### A. Tablet/App ↔ Gateway (Wi-Fi)

- Protocol: Wi-Fi

- Data Flow:

  - From Tablet to Gateway: Sends user commands (e.g., “Unlock door”, “Turn off light”).

  - From Gateway to Tablet: Sends sensor updates and device status (e.g., “Door is locked”, “Motion detected”).

- Reason for Wi-Fi:
 Wi-Fi provides high bandwidth, longer range, and direct internet access, making it ideal for connecting the user’s tablet to the gateway.
 It enables fast communication for user interaction and real-time monitoring.

#### B. Gateway ↔ Speaker & Microphone (BLE)

- Protocol: Bluetooth Low Energy (BLE)

- Data Flow:

  - From Gateway to Device: Sends playback or control commands (e.g., “Activate speaker”, “Start recording”).

  - From Device to Gateway: Sends audio input or sensor data (microphone feedback, voice detection).

- Reason for BLE:
 BLE is low power, supports short-range communication, and is efficient for devices that exchange small data packets intermittently.
 It’s perfect for wearable or audio-based components that don’t need constant high-speed data transfer.

#### C. Gateway ↔ Movement Detector (Zigbee/Thread)

- Protocol: Zigbee or Thread

- Data Flow:

  - From Device to Gateway: Sends motion and lighting status data (e.g., “Movement detected”, “Light level low”).

  - From Gateway to Device: Sends control signals (e.g., “Turn on light”, “Activate sensor”).

- Reason for Zigbee/Thread:
 Both Zigbee and Thread form mesh networks, allowing data to hop between devices, extending range and reliability.
 They are low power and robust, ideal for always-on sensors distributed across a smart home.

#### D. Gateway ↔ Door Lock (Zigbee/Thread)

- Protocol: Zigbee or Thread

- Data Flow:

  - From Tablet → Gateway → Door Lock: Unlock/lock commands.

  - From Door Lock → Gateway → Tablet: Status updates (e.g., “Door locked successfully”).

- Reason for Zigbee/Thread:
 Zigbee/Thread provide secure, low-latency, and energy-efficient communication, ensuring the door lock remains responsive but consumes minimal power.
 The mesh capability ensures connection reliability even if one node fails.

#### E. Gateway ↔ Doorbell (BLE/Zigbee)

- Protocol: BLE or Zigbee

- Data Flow:

  - From Device to Gateway: Button press signals and chime triggers.

  - From Gateway to Device: Sends control responses (e.g., activating chime, LED indicator).

- Reason for BLE/Zigbee:
 The doorbell requires low power and instant response, which BLE provides efficiently.
 Zigbee is also suitable if the doorbell must integrate into a home mesh network for wider coverage.