Types of Mesh Topology

- Full Mesh

- Every node connects directly to all others creating n(n-1)/2 links for n nodes.

- Ideal for small networks needing maximum redundancy and low latency

- Less practical for battery powered remotes due to high resource demands but could be tested in small demos

- Partial Mesh

- Nodes connect to a subset of others with some acting as routers to relay data

- More scalable for larger IoT networks; common in protocols like Zigbee and thread

How mesh networking works

- Node Roles

- End Devices

  - Low power nodes that communicate only with parents; they sleep to save energy,

- Routers

  - Always on relays that forward data extending range

- Coordinator/gateway

  - Manages the network, handling protocol translation and internet connectivity

  - For the project the QPG6200L acts as the end devices/routers

- Data Transmission and Routing

- Data hips between nodes using algorithms like AODV (in zigbee) or RPL (in thread), choosing optimal paths based on signal strength or energy

- Self Healing

  - If a node fails the network reroutes automatically

- Multi-Hop

  - Extends coverage without increasing transmit power, supporting the QPG6200L antenna

- Key Mechanisms

- Discovery and joining

  - Nodes scan for networks (using QPG6200Ls multi radio scanning) and join securely

- Security

  - Encryption and authentication

  - Ensures mesh integrity

- Concurrency

  - QPG6200L’s ConcurrentConnect allows simultaneous operation across protocols

Advantages and disadvantages for IoT applications

- Advantages

- Reliability/Redundancy

  - Multiple paths reduce downtime, supporting goal of controlling multiple remote devices without latency tradeoffs

- Extended Range

  - Multi-hop covers larger areas; QPG6200L’s antenna diversity boosts effective range by ~70% in meshes

- Low power

  - End devices sleep saving battery

  - Multi channel so it avoids congestion

- Scalability and interoperability

  - Easily add devices

- Disadvantages

- Complexity

  - Routing overhead; mitigated by QPG6200L’s optimized SoC and SDK

- Latency in Large Meshes

  - Multihop delays

- Interference

  - In 2.4 GHz band

- Resource Use

  - Higher for routers

Protocol Specific Mesh Implementations

- Zigbee

- Partial Mesh on IEEE 802.15.4; uses cluster tree or star mesh hybrids

- Routing: AODV for dynamic paths; support up to 65,000 nodes

- Low-power: End devices poll routers; ideal for lighting/sensing use cases

- Thread (Basis for matter)

- IPv6 based partial mesh; low power, self healing with RPL routing

- Nodes: Full Thread Devices (routers) and minimal thread devices (end nodes)

- Matter over Thread: Add application layer interoperability for  seamless device control

- Enables IP based web interfaces; QPG6200L SDK includes Matter-Zigbee combos for concurrent meshes

- Bluetooth Mesh

- Flood based partial mesh (no routing tables; messages broadcast and relay)

- Low power via BLE; supports publish-subscribe models for groups

- For short range, interactive elements; concurrent with 802.15.4 for multi-protocol

QPG6200L Integration with Mesh

The QPG6200L SoC is optimized for IoT meshes featuring:

- ConcurrentConnect™: Enables simultaneous Zigbee, Thread/Matter, and BLE operation on different channels, with multi-radio scanning (no blind spots) and multi-channel (up to 3 IEEE 802.15.4 PANs).

- Antenna Diversity: Improves range and robustness in meshes by selecting the best signal path.

- Secure Features: PUF-based storage, secure boot, and attestation prevent mesh vulnerabilities.

- Peripherals for Mesh Apps: PWM (lighting), ADC (sensing), PDM (audio)—project requires at least 3.

- SDK Support: Version 1.0.0 includes combo examples (e.g., Matter-Zigbee light switch), quickstarts, and app notes for multi-protocol meshes.