Figure of the QPG6200L Functional Block Diagram

- Temperature Sensor was suggested to be used

- Wanted to record voice input through microphone

- Looks like there’s support for BTE, Thread, Zigbee

- Security:

  - The QPG6200L is equipped with a low-power Secure Element subsystem providing secure operation functionalities to the application. This subsystem contains a dedicated secure microcontroller, memories and interfaces. It can work independently from the Application Microcontroller, IEEE 802.15.4 MAC and Bluetooth LL.

  - The Secure Element Subsystem supports:

    - Secure Boot (rooted in ROM with Boot Key in OTP), Physical Unclonable Function (PUF)-based Secure (Key) Storage, Secure (Key) Provisioning, Secure Debugging, Secure Device Attestation.

    - Flexible Tampering Detection (Voltage tampering, BBPLL tampering, Boot authentication failure, PK

    - ROM integrity error, SEUC RAM ECC correction or failure, Unauthorized access, Watchdog time-out).

    - Restriction Locks to certain security-sensitive hardware aspects of QPG6200L, e.g., register maps,NVM regions, OTP

- GPIO Pins

- GPIO Pin Assignment Tables for various functions

- Wake up: an event that can be triggered on a rising edge, on a falling edge, or on both edges seen on the pin.

- High Drive Sink: can be enabled for driving a high-power circuit or anything requiring more power

- ANIO / ADC

Components:

- High-resolution 16-bit ADC

- General Purpose 11-bit ADC

- Low-power Comparator.

- Battery / Temperature Monitor: The ADC module can be configured by software to monitor the power supply level internally; no external components are required. The power supply level and temperature are measured separate from the ANIO pins

- Keyboard Scanner

- The QPG6200L has an integrated 8x8 Keyboard Scanner with ultra-low power wake-up on key press, keyboard scan and de-bounce. The keyboard scan operation is triggered by a change on the IO (key press), as well as by a timed event (for de-bouncing). The keys are organized in a matrix. The mapping of the physical keys (row/column) to the application or profile-defined keys is software configurable. The maximum number of keyboard columns and rows depends on the number of other GPIO-devices enabled; see Table 38.

- Column = Input Row = Output

- InfraRed (IR) generator.

- The IR carrier and modulation parameters and codes are defined by the application software. The IR generator supports a wide range of common IR protocols.

- The IR generator supports multiple modulation modes:

  - Pattern based: input is a pattern of 0’s and 1’s in RAM

  - Time based: input is a sequence of ON and OFF times

  - Event based: modulation is controlled by scheduled actions.

- LED Generators (4 that with configurable function and events)

- It supports:

- 8-bit Pulse-Width Modulation (PWM)

- Fade-in/Fade-out

- Duty cycling to adjust brightness and save power

- LED Pixel Protocol Master (RTZ)

- two LED Pixel Protocol Masters which support many commercially available LED pixel driver IC’s using the RTZ (Return To Zero) protocol

- This will enable sending frames for all pixels, without microcontroller intervention

- Supports Sequential (Neopixel) and Random (Christmas tree) addressing

- PDM Microphone Interface

- The QPG6200L contains two Pulse-Density Modulation (PDM) MEMS Microphone Interfaces. It provides conversion from a Pulse Density Modulated signal (PDM), which is typically used for low-cost digital microphones, to a 16-bit Pulse Code Modulated signal (PCM). The PCM signal can then be used for further processing and compression of the audio using the embedded Audio Signal Processors.

- The PDM interfaces supports:

  - A Clock and Data pin for interfacing with a PDM MEMS microphone.

  - Optionally Capturing Data on the Rising and Falling Edge of the Clock for Stereo operations.

  - Configurable frequency of Clock signal: Default = 2 MHz and Digital frequency synthesizer for generating frequencies up to 3072 kHz.

- PWM Engine

- The PWM peripheral enables the generation of 16-bit pulse width modulated signals on an assigned GPIO.

- Generic Timers

- contains a number of generic timers; 6x 32-bits or 12x 16-bits (software selectable), allowing time intervals to be defined by the user

- Each timer has 4 capture/compare channels allowing generating interrupts when the count value matches the compare value, or to capture the count value on a configurable event (e.g., an external pin event).