Batteries

- Lithium coin cell batteries

  - Designed Low power IoT devices

- Voltage required = 1.7V-3.6V

  - Range is compatible with typical output of a new lithium coin cell

    - Starts at 3V, decreases over time

- Power efficiency- designed for best energy efficiency, allowing for longer life span

- Integrated features- has built in battery monitors, to measure supply voltage and remaining capacity

  - Has ultra low power voltage detectors and overvoltage protection

- As CR2032 discharges, internal resistance increases which causes voltage drop under high load

- Pulling current from 200mA to 500mA possible for short duration of time

- Voltage can drop half a volt in 30 min through a sustained load

- Possibility of voltage instability under high current

- CR123A or CR2 commonly used batteries

- Can try wiring coin cells in parallel to increase total current

  - Can reduce strain on each individual battery

    - Not ideal for long term high current draw

- Nominal voltage of 3V lithium cell = 3.6V to 3.7V when fully charged

- Max output current for CR2032 = 6.8mA to 15mA for few seconds, can go higher for shorter periods of time

  - (check datasheets, add capacitor, sue for low power applications)

Battery Housing

- Digikey

  - https://www.digikey.com/en/articles/how-to-select-a-coin-cell-battery-holder-for-maximum-shock-and-vibration

- Battery- CR2032

- https://www.amazon.com/Energizer-2032-Battery-CR2032-Lithium/dp/B01L0ISVLU?th=1

- https://www.amazon.com/CR2032-Lithium-Battery-Volt-Button/dp/B081DR39YW?th=1

- 500mAh coin cell battery

  - https://www.mouser.com/c/power/batteries/coin-cell-battery/?capacity=500%20mAh&srsltid=AfmBOorlCv1H_7uDXSYrEnWcWViRHwEyZTnW8krE_ywlvxdefS5e0ZxT

Buck Converters

- Used to decrease/ step down higher DC voltage to lower DC voltage

  - Makes it ideal for powering sensitive electronic components that need lower, stable voltage

- Energy harvesting, battery charging, efficient power regulation

- Provides efficiency, stability, versatility

- Buck converter not necessarily needed for qpg6200l because of integrated function and low power optimization

  - Integrated Function- SoC includes functionality on chip, which simplifies circuit design

  - Low Power Optimization- ?

  - Without buck converter, BOM cost can be reduced = not as expensive

PCB layers

- Different materials that form foundation and conductive paths of PCB with common types

  - Dielectric, copper solder mask, silkscreen

- Layers include- substrate, copper, solder mask, silkscreen

- Single layer, double layer, multi layer

- 2 layers should be sufficient

  - Less expensive, easier design and layout,

  - QPG6200L specifically designed to support 2 layers

    - Should work fully without additional layers if constructed correctly

- Make sure there is proper grounding, antenna placement, component placement, trace routing

  - Grounding- critical for RF performance and noise suppression

  - Antenna Placement- antenna and surrounding components must be isolated to avoid interference from other circuitry

  - Component Placement- Sensitive components must be as close to chip as possible to minimize trace lengths (Ex: RF front end)

  - Trace Routing- RF traces need to have controlled impedance and be short as possible

    - Helps to avoid abrupt corners, routing away from other digital signals to prevent integrity issues

PCBs

- Most compatible = QPG6200LDK IoT Development kit

  - Includes carrier board, plug on radio board, debugging tools, connectivity options, etc.

- Aspects to consider- concurrentconnect technology, antenna layout, security, simplified design

- Can use QPG6200LDK-01 IoT development kit as reference

- Microcontrollers

- Sensors

  - Presence and motion sensor

  - Biometric readers

  - PIR

  - RFID/NFC

- Actuators

  - Relays/ motor drivers

  - Cameras

  - microphones/ speakers

  - EEPROM/ flash storage

- Communication/ Security

  - Wireless connectivity

  - Secure boot encryption

  - Tamper detection

- Consider multilayer PCB and surface mount technology

- Wireless performance

- Power management

- Environmental protection to protect form the elements

- Modular design to simply upgrades- breaks down design into smaller, testable, functianl reusable units

  - Simplifies debugging, maintenance, scalability

- Entryway- QPG6200L, RF components, antenna, circuits for diversity, power management

  - inputs/outputs- digital communication signals, I2C, SPI, UART, GPIO

  - Power supply module, sensor interface modules, actuator and indicator modules, system integration and expansion

- Standardize connectors, explicitly define interface (Ex: power, data, control), optimize layout (put components like antenna on outer layer, use hierarchical design by creating schematic for each function

Lighting system best suited = Matter over Thread, Zigbee, Bluetooth Low Energy

- https://talkingiot.io/smart-home-connectivity-bridging-todays-needs-with-tomorrows-possibilities-with-qorvo-qpg6200l-soc-qpg6200ldk-01/

Camera

- Can be incorporated by using SoC as communication, control hub for separate camera module

- QPG6200L doesn’t directly control camera, but multi protocol function is ideal for it

Door Lock- Yale Assure Lock 2 Keypad with bluetooth = $169.99 https://shopyalehome.com/products/yale-assure-lock-2-keypad-with-bluetooth?variant=41535299518596

- Ecobee SMart video doorbell- https://www.bestbuy.com/product/ecobee-smart-video-doorbell-wired-with-advanced-person-and-package-detection-black/JXC53VLVQC/sku/6561053?extStoreId=1108&utm_source=feed&ref=212&loc=20802387732&gclsrc=aw.ds&gad_source=1&gad_campaignid=20812590616&gbraid=0AAAAAD-ORIhyC9XqUkGKKGtfdHTIRPGB_&gclid=CjwKCAjwisnGBhAXEiwA0zEOR9vo9iD7bNWCyKarONP0esOTNxjEcF7zs_0KFKHXIOJayltiI-zYkRoCL0UQAvD_BwE

- Front Door Security Plus Bundle- https://us.store.tp-link.com/products/front-door-security-bundle-2k-video-doorbell-camera-smart-door-lock-1?country=US&currency=USD&utm_medium=product_sync&utm_source=google&utm_content=sag_organic&utm_campaign=sag_organic&gad_source=1&gad_campaignid=22341788155&gbraid=0AAAAA-7-kOQKIxb6LIj5myoi2f91JOXxh&gclid=CjwKCAjwisnGBhAXEiwA0zEORxZynFui6D-DWreEbQGb28CB63B8-7Wvw43Xk1HA9PAoFlw2DQBp8hoCaP8QAvD_BwE

Gateway - Diagram of what else is needed for the gateway (power) as well as power consumption estimate.