®

1 Product Overview

QPG6200L

Multi-Protocol SoC

Data Sheet

The QPG6200L is the industry’s most reliable and robust wireless  multi-standard System-on-Chip. Featuring Qorvo’s ConcurrentConnectTM technology, the QPG6200L supports the  latest standards for Matter (over Thread), Zigbee and Bluetooth®  Low Energy in a truly concurrent way.

The low-power QPG6200L enables the adoption of Matter while supporting existing  Zigbee networks without tradeoffs in latency and link performance.

Featuring ConcurrentConnectTM Technologies: • Antenna Diversity for Bluetooth Low Energy and IEEE 802.15.4 enables increased effective range and interference robustness. • Multi-Radio capability allows continuously scanning for incoming packets across Bluetooth Low Energy and IEEE 802.15.4 protocols with no observable blind spots. • Multi-Channel capability allows operating in up to 3 IEEE 802.15.4 PANs on different channels.

• Integrated Secure Element Subsystem featuring:

• Secure Boot (rooted in ROM with Boot Keys  in OTP)

• Secure Storage (PUF-based)

• Secure Provisioning

• Secure Debugging

• Secure Device Attestation.

2 Functional Block Diagram

• Designed and optimized for low-power  IoT end node applications such as:

• Connected Lighting

• Smart Sensors

• Wearables

• Gateways.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 1 of 59 www.qorvo.com

®

3 Key Features

Radio

✔ 2.4-GHz RF Transceiver compliant with: Bluetooth Low Energy v 5.4 and IEEE

802.15.4 for wireless personal area networks  (WPAN).

✔ Preamble-based Antenna Diversity for  Bluetooth Low Energy and IEEE 802.15.4 ✔ IEEE 802.15.4 Packet-in-Packet resync. ✔ Transmitter output power programmable in  1 dB steps, stable over voltage and

temperature

✔ Integrated RF filter and matching

✔ Compliant with worldwide RF regulations:  ETSI EN 300 328 (Europe),

FCC CFR47 Part 15 (US), and

ARIB STD-T-66 (Japan)

Real-Time Bluetooth Low Energy Controller ✔ Bluetooth v 5.4 compliant LE Controller ✔ Enhanced data rate (2 Mbit/s)

✔ Long Range Coded PHY

✔ Link Layer Privacy

✔ Advertising Extensions

✔ Full connection utilization

Real-Time 802.15.4 Medium Access Control ✔ IEEE 802.15.4-compliant MAC

✔ Automatic ACK handling and retransmissions ✔ Address recognition and packet filtering

Multi-Protocol Support

✔ Hardware accelerated Dynamic Multi-Protocol Bluetooth Low Energy and IEEE 802.15.4  Communications

Secure Element Subsystem

✔ Dedicated microcontroller for Secure Element  operation

✔ Hardware accelerated AES and CCM/CCM*  encryption and decryption with 128, 192 and  256-bit keys

✔ Hashing engine: SHA-128, SHA-2 (SHA-256,  SHA-512)

✔ Public Key Crypto: Elliptic Curve; Support for  ECDSA, ECDH, P256, Curve25519, J-Pake,  ECMQV, EdDSA, etc.

✔ Support for PUF (Physical Unclonable  Function)

✔ Secure boot from ROM with boot keys in OTP ✔ Cryptographic Random Number Generator.

PRELIMINARY

QPG6200L Multi-Protocol SoC

Integrated Application Microcontroller ✔ Arm Cortex-M4 processor with DSP functionality ✔ Up to 192 MHz clock speed

✔ ETM Trace

Memory

✔ 2048 kB Non-Volatile Memory (RRAM), of  which 8 kB is One-Time Programmable

✔ 336 kB Low Leakage Retention RAM 16-channel DMA Engine

Peripherals and Interfaces

✔ 14 programmable GPIO lines

(up to 7 more when swapped with other pins,  4 of these are input only)

✔ 2 analog input lines

✔ 16-bit high-resolution and 11-bit GP ADC ✔ Temperature sensor

✔ Timers: 6x 32-bit or 12x 16-bit

✔ PWM Engine for 8 outputs

✔ LED Generator (8-bit PWM) with fading support for 4 signaling LEDs

✔ LED Pixel Protocol Master (RTZ) for dynamic  control of LED light strips

✔ USB 2.0 Device supporting Full Speed (12 Mbit/s) data rate

✔ 2x PDM Microphone Interface

✔ 1x SPI Master and 1x SPI Slave interface ✔ 2x I2C Master and 1x I2C Slave interface ✔ 2x I2S Master/Slave interface for digital audio  devices

✔ 3x UART interface

✔ Keyboard Scanner

✔ IR Generator; High Drive Sink for IR

Power Management

✔ Operating voltage range: 1.71 ... 3.6V ✔ Integrated Regulators

✔ Integrated SIDO DC/DC Buck Converter ✔ Low power Sleep modes:

Using internal LjRC oscillator: 0.9 µA

Using 32 MHz crystal oscillator: 200 µA  ✔ Data and state retention in all Sleep modes

Dimensions and Layout

✔ QFN32: Quad Flat No-lead package,  32 pins, 4x4 mm.

✔ No RF shielding required.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 2 of 59 www.qorvo.com

®

4 Ordering Information Table 1: Ordering and Packing Information

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Chip Package | Tx Power | Stnd. GPIO+ ANIO Lines | Optional extra GPIO Lines (*) | Packing | Unit Qty |
| --- | --- | --- | --- | --- | --- |
| QFN32 | 10 dBm | 14+2 | 7 | 13” Tape & Reel | 4000 |
|  |  |  |  | Sample Reel | 100 |

Part Number Box Dimensions (interior), [mm]

QPG6200LTR13337x337x27 QPG6200LSR 370x160x80 (*) see section 7.1 for details.

All QPG6200L part numbers contain 2 MB NVM (incl. OTP) and 336 KB RAM.

For ordering the QPG6200LDK-01 (IoT Dev Kit for QPG6200L) please refer to Qorvo’s website.  See also section 8.9 and 13.

5 Contact Information

Technical support:

www.qorvo.com/support/technical-support

Sales:

www.qorvo.com/support/how-to-buy/contact-a-sales-rep

A 3-page QPG6200 Product Brief is available on Qorvo’s website.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 3 of 59 www.qorvo.com

®

Table of Contents

PRELIMINARY QPG6200L Multi-Protocol SoC

1 Product Overview .......................................................................................................................................1 2 Functional Block Diagram...........................................................................................................................1 3 Key Features ..............................................................................................................................................2 4 Ordering Information...................................................................................................................................3 5 Contact Information ....................................................................................................................................3 6 Radio and Communication Protocols .........................................................................................................6

6.1 2.4 GHz Radio.....................................................................................................................................6 6.1.1 RF Ports with Integrated Baluns and Filter ....................................................................................6 6.1.2 Radio Configurations......................................................................................................................6 6.1.3 RSSI ...............................................................................................................................................6 6.1.4 Transmit Power Control .................................................................................................................6 6.1.5 ConcurrentConnect Antenna Diversity...........................................................................................7

6.2 IEEE 802.15.4 Communications .........................................................................................................7 6.2.1 2.4 GHz IEEE 802.15.4 Transceiver..............................................................................................7 6.2.2 Real-Time Medium Access Control (MAC)....................................................................................7 6.2.3 Link Quality Indication ....................................................................................................................7 6.2.4 Clear Channel Assessment (CCA) ................................................................................................7 6.2.5 Packet-in-Packet Resynchronization .............................................................................................7

6.3 Bluetooth Communications .................................................................................................................8 6.3.1 2.4 GHz Bluetooth Low Energy PHY Layer ...................................................................................8 6.3.2 Real-Time Bluetooth Low Energy Link Layer.................................................................................8 6.3.3 Full Connection Utilization..............................................................................................................8

6.4 Multi-Protocol Stack Support...............................................................................................................8 6.5 HW-Accelerated Dynamic Multi-Protocol Support ..............................................................................9 6.6 Coexistence Interface..........................................................................................................................9

7 Peripherals................................................................................................................................................11 7.1 IO Pins...............................................................................................................................................11 7.2 GPIO..................................................................................................................................................11 7.3 ANIO / ADC .......................................................................................................................................12 7.4 Battery / Temperature Monitor ..........................................................................................................12 7.5 Keyboard Scanner.............................................................................................................................12 7.6 IR Generator......................................................................................................................................13 7.7 LED Generator ..................................................................................................................................13 7.8 LED Pixel Protocol Master (RTZ)......................................................................................................13 7.9 PDM Microphone Interface................................................................................................................13 7.10 PWM Engine .....................................................................................................................................14 7.11 Generic Timers..................................................................................................................................14

8 System, Interfacing and Programming .....................................................................................................15 8.1 Internal Microcontroller......................................................................................................................15 8.2 Memory Architecture .........................................................................................................................15 8.3 System States ...................................................................................................................................16

8.3.1 System Active...............................................................................................................................16 8.3.2 System Sleep ...............................................................................................................................16 8.4 Oscillators & Clocks ..........................................................................................................................17 8.5 Event Scheduler................................................................................................................................17 8.6 Power Modes ....................................................................................................................................17 8.7 Watchdog ..........................................................................................................................................19 8.8 Interfacing..........................................................................................................................................19 8.8.1 UARTs..........................................................................................................................................19 8.8.2 USB Device..................................................................................................................................19 8.8.3 QSPI Master.................................................................................................................................19 8.8.4 SPI Master....................................................................................................................................19 8.8.5 SPI Slave......................................................................................................................................20 8.8.6 I2C Master ....................................................................................................................................20 8.8.7 I2C Slave ......................................................................................................................................20 8.8.8 I2S Master/Slave...........................................................................................................................20 8.8.9 SPI Programming Interface..........................................................................................................21 8.8.10 Clock Output.................................................................................................................................21

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information © 2023-2024 Qorvo, Inc. Page 4 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

8.9 Application Programming Information ...............................................................................................21 8.10 NVM Programming and Configuration ..............................................................................................22 8.11 Debug Mode......................................................................................................................................22

9 Power Management..................................................................................................................................23 9.1 DC/DC Converter ..............................................................................................................................23 9.2 Low Voltage Behavior .......................................................................................................................24

10 Security .....................................................................................................................................................25 11 Electrical Characteristics ..........................................................................................................................26 11.1 Absolute Maximum Ratings...............................................................................................................28 11.2 Recommended Operating Conditions ...............................................................................................29 11.3 Current Consumption ........................................................................................................................30 11.4 Receiver Characteristics ...................................................................................................................32 11.5 Transmitter Characteristics ...............................................................................................................36 11.6 Digital Timing Characteristics............................................................................................................38 11.7 Reset, Wake up and Sleep Timing Characteristics...........................................................................40 11.8 NVM Characteristics..........................................................................................................................41 11.9 Crystal Oscillator Specifications........................................................................................................42 11.9.1 32 MHz Crystal Oscillator.............................................................................................................42 11.9.2 32 KiHz Crystal Oscillator (optional) ............................................................................................43 11.10 Internal Pull-up / Pull-down Characteristics ......................................................................................43 11.11 ADC Characteristics ..........................................................................................................................43 11.12 Battery and Temperature Monitor Characteristics ............................................................................44 11.13 DC/DC Converter Information ...........................................................................................................45 11.14 GPIO0 High Drive Sink Characteristics.............................................................................................45 11.15 64 kHz LjRC Oscillator Characteristics .............................................................................................46 12 Device Information....................................................................................................................................47 12.1 Package.............................................................................................................................................47 12.1.1 Pin Assignments ..........................................................................................................................47 12.1.2 Package Drawing and Recommended Land Pattern...................................................................52 12.1.3 Package Information ....................................................................................................................53 12.1.4 Thermal Characteristics ...............................................................................................................53 12.1.5 Tape and Reel Information...........................................................................................................54 12.2 Moisture/Reflow Sensitivity ...............................................................................................................56 12.3 Environmental Compliance ...............................................................................................................56 13 Development Kit .......................................................................................................................................56 Abbreviations ....................................................................................................................................................57 Important Notices..............................................................................................................................................58 Product Status ..................................................................................................................................................58 Document History .............................................................................................................................................59

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information © 2023-2024 Qorvo, Inc. Page 5 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

6 Radio and Communication Protocols

6.1 2.4 GHz Radio

The QPG6200L radio transceiver provides all the functionality for the Physical layer (PHY) for both IEEE  802.15.4 and Bluetooth communications. The transceiver contains a highly configurable pulse shaping filter  that can be used for both Bluetooth Low Energy and 802.15.4 transmission.

This section describes the generic features; following sections describe the specific features for IEEE 802.15.4  and Bluetooth.

6.1.1 RF Ports with Integrated Baluns and Filter

The antenna ports output is 50 Ω single ended.

6.1.2 Radio Configurations

The QPG6200L supports different radio configurations. It can be configured to use a different receive and  transmit antenna. Antenna Diversity configuration is also possible for both Bluetooth Low Energy and IEEE  802.15.4 communications (see section 6.1.5). Two sample configurations are depicted below, but others are  also possible.

Sample Configuration 1 (Figure 1):

• Single ended 50 Ω antenna.

• Using antenna 0 (RF0 pin) for both RX and  TX.

• Antenna diversity disabled.

Sample Configuration 2 (Figure 2):

• 2 single-ended 50 Ω antennas.

• Antenna diversity enabled. TX on same  antenna as was selected best by RX.

6.1.3 RSSI

Figure 1: Single Antenna

Figure 2: Two Antennas

The PHY’s RSSI circuitry measures the received signal energy level and this value is converted to dBm values in the Hardware Abstraction Layer (HAL). See Table 15 for the range and accuracy. RSSI data is available for both Bluetooth Low Energy and IEEE 802.15.4 communication.

6.1.4 Transmit Power Control

The transmitter output power is configured by software, in steps of 1 dB.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 6 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

6.1.5 ConcurrentConnect Antenna Diversity

Preamble based antenna diversity enables the PHY to choose the optimal antenna for every individual packet  and increases the performance of the receiver in environments that are dominated by multipath fading effects  and interference situations. In receive mode the PHY selects the antenna based on the best signal quality  (signal-to-noise/interference ratio).

For typical indoor usage in an environment with 50 ns delay-spread and 2 MHz signal bandwidth using the  Rayleigh fading model, antenna diversity with 2 antennas results in a ~8 dB improved link budget (at a 1%  outage probability) compared to no antenna diversity. This translates into 70% more reliable range (using a  log-distance breakpoint model1 with path loss coefficients g1=2 (free space propagation) and g2=3.5 above  the breakpoint at 10 m).

Unless configured otherwise, the QPG6200L will use the same antenna for transmission as the one that was  used for the reception of the last packet.

1 Refer to “T.S. Rappaport, Wireless Communications – Principles & Practice, Prentice Hall, 1996” for this model. 6.2 IEEE 802.15.4 Communications

6.2.1 2.4 GHz IEEE 802.15.4 Transceiver

The QPG6200L radio is compliant with the IEEE 802.15.4 2020 standard.

The QPG6200L supports all the IEEE Standard 802.15.4 defined channels in the 2.4 GHz ISM license-free  frequency band (channels 11 .. 26).

The channel number (k) and center frequency (Fc) relate as follows: Fc = 2405 + 5(k – 11) [MHz]

6.2.2 Real-Time Medium Access Control (MAC)

The QPG6200L implements the 2.4 GHz OQPSK-required MAC features of the IEEE Standard 802.15.4 used  by Thread/Matter and Zigbee (including Zigbee RF4CE). The MAC provides a packet-level service to the protocol stack, and handles packet transmissions and receptions autonomously, including:

• Performing CSMA/CA to avoid collisions when transmitting packets;

• Adding CRC and Sequence number;

• Acknowledgement handling for transmitted packets, including automatic retransmissions; • Address recognition and packet filtering on received packets, including CRC checking; • Acknowledgement handling for received packets, including automatic acknowledge transmission.

6.2.3 Link Quality Indication

In addition to the RSSI, there is also a link quality indication (LQI) determined for each received IEEE 802.15.4  data packet, for use at the network and application layers.

6.2.4 Clear Channel Assessment (CCA)

The PHY can perform a clear channel assessment (CCA) to avoid collisions. The IEEE 802.15.4 standard  defines 3 CCA methods; the QPG6200L supports:

• CCA mode 1, Energy Detect (ED): the medium is considered busy when the measured energy in the  selected channel is above a certain threshold. This CCA threshold is programmable. • CCA mode 3, Energy Detect (ED) and Carrier Sense (CS): the medium is considered busy when a  valid IEEE 802.15.4 carrier is detected in the selected channel and the measured energy in the  selected channel is above the programmable threshold.

6.2.5 Packet-in-Packet Resynchronization

If the QPG6200L is receiving a packet from one node and is interrupted by the reception of another stronger  packet from another node, the receiver will resynchronize to the latter and continue to receive and process this  packet. This allows one packet (the strongest) to be received where otherwise both packets would have been  lost. Packet-in-Packet collisions can occur in situations when neighbor network packets are received at a low  level and in hidden node situations where not all nodes can see each other.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 7 of 59 www.qorvo.com

®

6.3 Bluetooth Communications

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L implements the Bluetooth Low Energy (LE) Controller functionality, including PHY, Link Layer  and HCI according to the Bluetooth Core Specification v 5.4 for Bluetooth Low Energy. When combined with  a Bluetooth Low Energy Host Stack, it supports all GATT-based profiles and services, and it can operate as a  Broadcaster, Observer, Central and Peripheral device.

6.3.1 2.4 GHz Bluetooth Low Energy PHY Layer

The QPG6200L implements the Bluetooth LE PHY layer, supporting all the (40) Bluetooth defined frequency  channels in the 2.4 GHz ISM license-free frequency band.

The Bluetooth LE Controller supports the basic data rate of 1 Mbit/s, the enhanced data rate of 2 Mbit/s, as  well as the Long Range Coded PHY data rates of 125 kbit/s and 500 kbit/s.

6.3.2 Real-Time Bluetooth Low Energy Link Layer

The Real-Time Link Layer implements the real-time functions of the Bluetooth LE Link Layer (LL) protocol for  the Advertising, Scanning, Initiating and Connection States. Multi-state operation is supported: a multi-level  priority mechanism ensures appropriate scheduling of Advertising, Scanning, Initiating and Connection events.

In the Connection State, the Real-Time Link Layer maintains the LE Asynchronous Connection-oriented  Logical (LE ACL) transport on master and/or slave connections, allowing transfer of control (LE-C) and user  (LE-U) data. High-throughput applications are supported via a dedicated queue per (LE ACL) connection, thus  ensuring efficient filling of Connection Events.

The maximal PDU size supported is 261 bytes (resulting in a full PDU payload size of 255 bytes). Together  with 4 bytes of access code and 3 bytes of CRC, this gives a total packet size, excluding preamble, of  268 bytes.

The QPG6200L supports the Link Layer Privacy and Advertising Extensions as defined in the Bluetooth Core  v 5.4 Specification.

6.3.3 Full Connection Utilization

The QPG6200L has been optimized for audio streaming over Bluetooth Low Energy with a full Bluetooth  connection utilization under high CPU load conditions. This implies that the system can fill the complete  connection with Bluetooth packets allowing to achieve the maximal bandwidth of the connection, even when  the CPU is processing audio (decimation, equalizing, compressing), under a large variety of Bluetooth

connection configurations; normal data and high data rate, short and long connection intervals, short and long  Bluetooth packets.

6.4 Multi-Protocol Stack Support

The QPG6200L can support multiple protocol stacks. IEEE 802.15.4 based stacks can interface  simultaneously with the QPG6200L MAC API through a MAC Dispatcher, while the Bluetooth Low Energy  Stack can interface with the QPG6200L at the HCI level at the same time.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 8 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

6.5 HW-Accelerated Dynamic Multi-Protocol Support

The QPG6200L has HW support to seamlessly interleave Bluetooth Low Energy communications  (advertisements and Bluetooth connections) with IEEE 802.15.4 communications for applications that support  dual mode communication (e.g., Bluetooth Low Energy / Thread). The Dynamic Multi-Protocol Support lets the  IEEE 802.15.4 MAC autonomously schedule and interleave IEEE 802.15.4 traffic to ensure that Bluetooth  connection traffic is maintained without any connection drops.

For use cases where more airtime is needed for Bluetooth data transfers, the following additional modes are  supported:

• Best effort mode, for non-real time (bulk) Bluetooth data transfers, where the Bluetooth LE controller  frees up airtime in the connection when an IEEE 802.15.4 data packet is queued.

• Controlled bandwidth mode, for real time (streaming) Bluetooth data transfers, where the Bluetooth  LE controller reserves a configurable portion of the connection for IEEE 802.15.4 communication.

! This feature can be combined with ConcurrentConnect Multi-Channel Support to support three IEEE 802.15.4 stacks (each on its own channel), while simultaneously acting as a Bluetooth Low Energy peripheral and maintaining a Bluetooth connection with a smartphone. This can also be combined with ConcurrentConnect Multi-Radio, allowing to combine Bluetooth LE scanning and 802.15.4 RX ON.

6.6 Coexistence Interface

This Interface can be used to enable coexistence of the QPG6200L radio with other potentially interfering  radios (Bluetooth, Wi-Fi) within the same device. This enables the following features:

• Avoid RX desensitization of the QPG6200L radio when another radio within the same device would  transmit while the QPG6200L radio is in receive mode.

• Avoid RX desensitization of another radio within the same device when the QPG6200L radio would  transmit while the other radio is in receive mode.

• Allow the QPG6200L radio and the other radios within the same device to share the same transmit  frequencies (TX channel overlap).

Radio coexistence is controlled by an arbiter, that may grant a radio access to the medium depending on the  state of the other radios and assigned priorities. The QPG6200L contains an arbiter but can also use an  external arbiter. The Coexistence Interface is user-configurable, and has two modes of operation:

Master : The internal arbiter of the QPG6200L is used; up to 2 other radios can be interfaced to this  arbiter. The arbiter uses a straightforward request-granted approach with 4 priority levels.

Slave : The QPG6200L will request access to the medium from an external arbiter.

Table 2 below shows the signaling lines between the QPG6200L and the arbiter or other radio(s). The mapping  of these signals to pins is given in Table 40.

Table 3 contains the options for coex signal configuration.

Table 2: Coex Signal Names

Input/Output

Input

Output

Input

Input

Output

Input

Master Signal Name Function

COEX_MS_REQA Request signal from other radio A

COEX_MS_GRANTA Grant signal to other radio A

COEX_MS_PRIOA Priority signal from other radio A

COEX_MS_REQB Request signal from other radio B

COEX_MS_GRANTB Grant signal to other radio B

COEX_MS_PRIOB Priority signal from other radio B

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 9 of 59 www.qorvo.com

®

Input/Output

Output

Output

Input

Output

Output

PRELIMINARY

QPG6200L Multi-Protocol SoC

Slave Signal Name Function

COEX_SL_REQ Request signal from QPG6200L radio to the external arbiter COEX_SL_REQ_NOT Request signal from QPG6200L radio to the external arbiter  (inverted)

COEX_SL_GRANT Grant signal from the external arbiter to the QPG6200L radio COEX_INT_PRIO_0 Priority signal from QPG6200L radio to the external arbiter COEX_INT_PRIO_1 Priority signal from QPG6200L radio to the external arbiter

Table 3: Signal Configuration Options

Input Signal Effect

Not Mapped - Treat as ’0’ input value is always treated as a ’0’

Not Mapped - Treat as ’1’ input value is always treated as a ’1’

Pin ’high’ is a logic ’1’; ’low’ is a logic ’0’

Inverse Pin ’high’ is a logic ’0’; ’low’ is a logic ’1’

Output Signal Effect

Not Mapped -

Push Pull Push Pull Driver; logic ’1’ is ‘high’, logic ’0’ is ‘low’

Inverse Push Pull Push Pull Driver; logic ’1’ is ‘low’, logic ’0’ is ‘high’

Open Drain Open Drain Driver; logic ’1’ is ‘high’, logic ’0’ is ‘low’

Inverse Open Drain Open Drain Driver; logic ’1’ is ‘low’, logic ’0’ is ‘high’

Until the software in the QPG6200L has been initialized, the coexistence signaling pins are floating.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 10 of 59 www.qorvo.com

®

7 Peripherals

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L features a set of peripherals and allows configuration of the mapping between the IO signals  needed by the peripherals and the available IO pins.

7.1 IO Pins

The QPG6200L features many IO pins that can be configured for pre-defined functional signals; see the Pin  Assignments in section 12.1.1.

The complete pin configuration with associated settings is retained when going to Sleep mode. Table 4: GPIO and ANIO Configurations

I/O Type Number

GPIO (programmable General

Purpose I/O) 14x

ANIO (programmable Analog I/O) 2x

pins for the optional ANIO’s can  alternatively be used as GPIO pins for the optional DC/DC  converter can alternatively be used  as GPIO

pin for the optional LPBIAS can

2x

(input only) 2x

alternatively be used as GPIO 1x

pins for the optional 32 KiHz  reference crystal can alternatively  be used as GPIO

2x

(input only)

max. number of GPIO’s 17x input/output

+ 4x input only

7.2 GPIO

The QPG6200L features programmable GPIO lines that are mapped individually to functional signals, as  specified in the Pin Assignments in Table 37 and Table 38, with following settings:

• Pull-up/Pull-down: most GPIO pins can be configured to the following pull-up/down modes:  o Floating mode – Pull modes disabled

o Pull down mode

o Pull up mode.

Exceptions are: GPI25 and GPI26 are always in floating mode.

Unless specified otherwise, at power-up/reset all GPIO pins are default set in floating mode. The GPIO pins can be individually weakly pulled up or weakly pulled down during active as well as  Sleep states.

• Wake up: except for GPIO19 and GPIO20, all GPIO pins can be configured as wake-up pin. Each of  these can be configured to trigger a wake-up event on a rising edge, on a falling edge, or on both  edges seen on the pin.

• Drive strength: the drive strength of each GPIO output can be configured individually to 2 mA, 4 mA,  8 mA or 12 mA. This is applicable only to standard GPIO’s that support both input and output, so not  for input only GPIO’s (GPI).

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 11 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

• High Drive Sink: on GPIO0 a high drive sink (MOSFET N-Type) can be enabled, suitable for e.g.,  driving a high-power IR LED circuit. Figure 3 shows an IR LED circuit with the high drive sink enabled.  For comparison, Figure 4 shows an example without high drive sink.

Figure 3: IR with High Drive Sink Figure 4: IR without High Drive Sink

7.3 ANIO / ADC

The QPG6200L has integrated ADC’s that can be used to monitor external analog signals via the ANIO pins.  2 ANIO’s are provided for inputs to:

• High-resolution 16-bit ADC

• General Purpose 11-bit ADC

• Low-power Comparator.

The QPG6200L contains a full switch matrix (MUX)

connecting any ANIO to the ADC or the LPCMP.

ANIO’s can be combined for differential measurements.

Section 11.11 provides the ADC’s accuracy and other

characteristics.

Figure 5: ANIO Connect Options

7.4 Battery / Temperature Monitor

The ADC module can be configured by software to monitor the power supply level internally; no external  components are required. The power supply level and temperature are measured separate from the ANIO  pins.

7.5 Keyboard Scanner

The QPG6200L has an integrated 8x8 Keyboard Scanner with ultra-low power wake-up on key press,  keyboard scan and de-bounce. The keyboard scan operation is triggered by a change on the IO (key press),  as well as by a timed event (for de-bouncing). The keys are organized in a matrix. The mapping of the physical  keys (row/column) to the application or profile-defined keys is software configurable.

The maximum number of keyboard columns and rows depends on the number of other GPIO-devices enabled;  see Table 37 and Table 38.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 12 of 59 www.qorvo.com

®

7.6 IR Generator

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L has an InfraRed (IR) generator. The IR carrier and modulation parameters and codes are  defined by the application software. The IR generator supports a wide range of common IR protocols. The IR generator supports multiple modulation modes:

• Pattern based: input is a pattern of 0’s and 1’s in RAM

• Time based: input is a sequence of ON and OFF times

• Event based: modulation is controlled by scheduled actions.

7.7 LED Generator

The QPG6200L supports up to 4 signaling LEDs, with configurable function and events. The LED generator  supports:

• 8-bit Pulse-Width Modulation (PWM)

• Fade-in/Fade-out

• Duty cycling to adjust brightness and save power.

7.8 LED Pixel Protocol Master (RTZ)

The QPG6200L contains two LED Pixel Protocol Masters which support many commercially  available LED pixel driver IC’s using the RTZ (Return To Zero) protocol. This will enable  sending frames for all pixels, without microcontroller intervention.

• Supports Sequential (Neopixel) and Random (Christmas tree) addressing

• Highly configurable to support many protocols

• Efficient RAM usage for the pixel frames.

7.9 PDM Microphone Interface

The QPG6200L contains two Pulse-Density Modulation (PDM) MEMS Microphone Interfaces. It provides  conversion from a Pulse Density Modulated signal (PDM), which is typically used for low-cost digital  microphones, to a 16-bit Pulse Code Modulated signal (PCM). The PCM signal can then be used for further  processing and compression of the audio using the embedded Audio Signal Processors.

The PDM interfaces supports:

• A Clock and Data pin for interfacing with a PDM MEMS microphone.

• Optionally Capturing Data on the Rising and Falling Edge of the Clock for Stereo operations. • Configurable frequency of Clock signal:

o Default = 2 MHz

o Digital frequency synthesizer for generating frequencies up to 3072 kHz.

• HW CIC Decimation filter with programmable decimation factor (R=1…64) that converts 1-bit input  samples to 16-bit output samples. HW CIC output can be connected to DMA, to allow this processing  chain to be extended with further decimation, equalization, volume control and compression using the  Cortex M4F DSP routines.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 13 of 59 www.qorvo.com

®

7.10 PWM Engine

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L contains a PWM (Pulse-Width Modulation) peripheral.

The PWM peripheral enables the generation of 16-bit pulse width modulated signals on an assigned GPIO. Thanks to its advanced feature set it can support a wide range of use cases, including LED driver use cases.

The main features of the PWM peripheral are:

• Programmable PWM frequency

• Up to 8 PWM channels, each with an independent phase and duty cycle

• Arbitrary waveform generation driven by RAM defined sequences

• Glitch free parameter updates.

7.11 Generic Timers

The QPG6200L contains a number of generic timers; 6x 32-bits or 12x 16-bits (software selectable), allowing  time intervals to be defined by the user. The timers can be used in a prescaled mode, where the counter is  running from the main clock with a configurable divide ratio, or in count mode, where the counter is incremented  on a configurable event (e.g., an external pin event).

Each timer has 4 capture/compare channels allowing generating interrupts when the count value matches the  compare value, or to capture the count value on a configurable event (e.g., an external pin event).

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 14 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

8 System, Interfacing and Programming

8.1 Internal Microcontroller

The internal microcontroller is a high performance 32-bit Arm Cortex-M4 processor with floating point support  and DSP functionality, optimized for low power consumption, performance and code size. This allows the  QPG6200L to operate as a standalone system.

It runs at up to 192 MHz clock speed (*) and can execute code from NVM as well as from RAM, with zero wait  states.

(*) 192 MHz supported over a limited temperature range, 96 MHz supported over the full temperature range.

Table 5: Memory Access Speeds

| MCU RAM access | System RAM access | ROM access |
| --- | --- | --- |
| 32 MHz | 32 MHz | 32 MHz |
| 64 MHz | 32 MHz | 32 MHz |
| 96 MHz | 32 MHz | 32 MHz |
| 192 MHz | 32 MHz | 32 MHz |

Arm clock NVM Memory  access

32 MHz 32 MHz for linear code; 16 MHz worst case

64 MHz 64 MHz for linear code; 16 MHz worst case

96 MHz 96 MHz for linear code; 16 MHz worst case

192 MHz 192 MHz for linear code; 16 MHz worst case

8.2 Memory Architecture

The QPG6200L contains the following memory blocks:

• 336 kB RAM: Low Leakage Random Access Memory (RAM).

This is split in:

o 288 kB MCU RAM, accessible to the internal microcontroller and the DMA Engine o 32 kB RAM available to the radio subsystem

o 16 kB System RAM, accessible to the internal microcontroller, the DMA Engine and other  functional blocks.

• 2 MB (2048 kB) NVM memory (RRAM), for program storage, calibration data and NVM storage of  critical run-time data (e.g., pairing information and frame counters). The contents are retained under  all circumstances (Power-On-Reset, Sleep).

Note: 8 kB of NVM memory is reserved for OTP.

The QPG6200L has a 16-channel DMA Engine that relieves the microcontroller from transferring data internally between RAM and peripherals.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 15 of 59 www.qorvo.com

®

8.3 System States

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L is designed to work in an environment where low power consumption is very important. The  QPG6200L System State is either System Active or System Sleep (stand-by) to achieve low power  consumption. The following sections will explain the differences.

8.3.1 System Active

The System Active state is the default operational state after a Power-On-Reset. In this state all functional  blocks, including the application microcontroller are active or can be activated, based on the configuration set  by the application software. The software running on the application microcontroller will control the operational  state and will initiate a state transition if that is desired. Power consumption is optimized in this state by  disabling inactive clocks and by choosing the appropriate Power Mode (see Power Modes section 8.6).

8.3.2 System Sleep

The System Sleep state (stand-by) is the deepest power saving mode the device can use. In this state the  main functional blocks are not active, including the application microcontroller. Only the LPS (Always On) blocks are available. See also Figure 6.

The System Sleep state can be entered when all functional blocks indicate there are no tasks pending and the  application microcontroller allows the system to go into a deep power down mode, because the next tasks are  far enough in the future, or a next task should be initiated by an external event.

The following events can cause the device to ’wake up’ from the System Sleep state:

• RTC timestamp match - The Real Time Counter block provides a time base, clocked by the 64 kHz or 32 KiHz Real Time Clock or the divided down 32 MHz clock. The block can generate a wake-up  trigger at a certain timestamp configured by application software.

• GPIO pin event - The always on GPIO event block can be configured to wake up the system on a  rising edge and/or falling edge on a GPIO pin.

• Low Power Comparator event - The low power analog comparator can be configured to wake up the  system on a compare valid event.

• Reset - A pin reset, or Power On reset detect will also cause the device to exit the System Sleep state.

The System Sleep state allows for reaching the lowest device power consumption. All HF clock sources  (32 MHz, 64 MHz, 192 MHz) can be disabled and the main digital power blocks can be switched off. When the  device wakes up from the System Sleep state, a wake-up procedure is performed that is similar to a reset  procedure. One of the phases in the wake-up procedure is to restore the state of the different functional blocks.  Depending on the power options used during the System Sleep state, this restoring can take more time.

Essentially there are 2 groups of Sleep modes:

• eLPS and LPS power modes (Digital off modes) – These are the lowest power modes, where the  main digital domain power supply is switched off. All states are lost and the device depends on the  Register Retention settings to be able to restore the state after wake-up.

• HPSret (Digital retained mode) - This is a mode where the digital domain power supply is kept on. In  this case all states are retained, and no restoring is necessary. This a higher current System Sleep  state that allows for a faster wake up but power consumption is higher compared to a Digital off mode.

When an application uses the System Sleep mode to get to the lowest possible power consumption, it can be  necessary to also retain the content of the different data RAM’s, next to the register retention mentioned above.  The amount of RAM that is retained has a significant effect on the current consumption during System Sleep.  it is important that the user is able to only retain the RAM that really holds application state. For this reason,  the user has fine grained software control over what RAM is retained.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 16 of 59 www.qorvo.com

®

8.4 Oscillators & Clocks

PRELIMINARY

QPG6200L Multi-Protocol SoC

The 64 MHz and 192 MHz ring oscillators provide the HF main clocks. The 192 MHz ring oscillator is needed  for all use cases using the radio.

The 192 MHz ring oscillator provides the HF main system clock. For most use cases, and for all use cases  using the radio, this oscillator needs to be used as the main system clock. To use this clock, the 32 MHz  reference clock must be running. Enabling the 192 MHz oscillator is under software control.

The 64 MHz ring oscillator serves as the startup clock and the clock used by the PMU (Power Management  Unit). At wake-up, the free running 64 MHz ring oscillator will provide a clock to allow digital processing while  the 32 MHz crystal is still starting.

The 32 MHz crystal oscillator is based on the required external 32 MHz crystal. This HF oscillator is used as  the source for the high accuracy main system clock and reference frequency for the radio subsystem. And it  is used as a reference for generating the internal 192 MHz clock. Enabling this crystal oscillator is under  software control.

The 64 kHz Low Jitter RC (LjRC) oscillator is an internal LF low-power oscillator designed to support  applications needing an accurate time-base during Sleep (standby), effectively removing the need for a  32 KiHz oscillator with external crystal. High accuracy is obtained through regular calibration of the LJRC  oscillator using the 32 MHz crystal reference clock. This oscillator can be the LF clock source of choice for  cost constrained low-power use cases, as no additional external components are needed.

The optional 32 KiHz crystal oscillator is based on the optional external 32 KiHz crystal. This LF oscillator  can be used for lowest-power use-cases when accurate timing during Sleep is needed. For example, applications that implement long Sleep intervals requiring synchronized wake up events (i.e., Bluetooth Low  Energy) as the more accurate timing results in a smaller guard band and more time spent in Sleep mode. It  provides a very accurate time reference, at the cost of an additional external crystal.

Note that enabling / disabling the oscillators, and the switchover of the main system clock source, is very tightly  coupled to the power mode that is in use.

8.5 Event Scheduler

The Event Scheduler uses an accurate Adaptive Timing Engine to generate a common and calibrated system  time base. This time base is maintained over sleep cycles and can be used throughout the system to schedule  actions, autonomously and periodically, in a just-in-time manner, improving the overall energy consumption of  the system.

Additionally, the Event Scheduler also provides an advanced event handler function for scheduling software  interrupts.

The time base spans up to 30 minutes with a 1 μs resolution.

8.6 Power Modes

In this section the different power modes in the QPG6200L are defined. The power mode is closely related to  the system state of the device as some power modes are only available in one or the other state.

The QPG6200L supports the power modes as stated in Table 6 below.

The names of the power modes that start with “e”, for example; eLPS, are the enhanced power modes that  use the internal DCDC converter to minimize power consumption.

HPSret is a power mode that can be used to bring the system state to Sleep but allowing a fast wake-up. All  states are retained and no restoring is necessary. Typical use cases are short sleep durations (no need for  data backup/restore) and use cases where the 32 MHz crystal oscillator is used as sleep time base.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 17 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Table 6: Main Power Modes (lowest power consumption on top)

| Meaning | System state | Main system clock | Digital supply | Analog supply | DCDC |
| --- | --- | --- | --- | --- | --- |
| enhanced Low Power State | SLEEP | OFF | OFF | OFF | ON |
| Low Power State | SLEEP | OFF | OFF | OFF | OFF |
| High Performance State, retain all states | SLEEP | OFF | ON | ON | OFF |
| enhanced High Performance State | ACTIVE | 192 MHz | ON | ON | ON |
| High Performance State | ACTIVE | 192 MHz | ON | ON | OFF |

Name Benefits

eLPS lower power  compared to LPS

LPS low cost (*) HPSret fast wake-up

eHPS lower power  compared to HPS

HPS low cost (*)

(*) no DCDC inductor needed.

Power modes can be identified by these properties (see table above):

• Main clock: defines what the main clock source is.

• Digital supply: defines if the main digital supply is ON or OFF.

• Analog supply: defines if the analog supply is ON or OFF.

• DCDC: defines if the power supplies are fed by the DCDC converter or the LDO’s (see Figure 8).  • System state: defines if the power mode is applicable to the System Active state or System  Sleep state.

Figure 6 below shows an overview of the different subsystems and digital peripherals. Its color indicates the  ’minimal’ power mode; (e)LPS or (e)HPS mode, that is required for the block to function correctly.  As an example, the block “2.4 GHz radio” has (e)HPS as minimal required power mode, so it will not correctly  work in the (e)LPS power mode.

(e)LPS Mode (e)HPS Mode

Figure 6: (e)LPS and (e)HPS Power Modes

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 18 of 59 www.qorvo.com

®

8.7 Watchdog

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L contains a Watchdog timer that serves to detect and resolve software failures and to trigger  an interrupt, an internal microcontroller reset, or a system reset when the timer reaches a certain timeout value.  Timeout values are software configurable:

• 16-bit time-out values in 16 µs resolution

• Configurable watchdog time-out (16 µs - to 1 s)

• Configurable action (interrupt / Application Microcontroller reset / Chip Soft POR) • Protection to accidental watchdog triggering and disabling.

8.8 Interfacing

8.8.1 UARTs

The QPG6200L contains three Universal Asynchronous Receiver and Transmitters (UARTs) for interfacing  with additional peripheral devices and/or for terminal logging during (software) development. The UARTs  support:

• Full-duplex operation

• Baud rates from 488 Bd to 2 MBd

• Serial frames with 5, 6, 7, 8 or 9 data bits and 1 or 2 stop bits, with framing error detection • Odd or even parity generation and checking

• Buffer overflow detection

• False start bit detection and digital low pass filter for robustness against noise

• Separate interrupts on TX Complete, TX Data Register Empty and RX Complete • Configurable pin mappings, i.e., a RX pin and a TX pin can be made available on the QPG6200L pin-out.  • Configurable hardware flow control.

8.8.2 USB Device

The QPG6200L supports a USB 2.0 Device supporting Full Speed (12 Mbit/s) data rate. • Support for bulk, interrupt, and isochronous transfers

• programmable double buffering for bulk and interrupt endpoints

• 1 kB data buffer

• DMA for copying data to/from the USB data buffer

• supports USB remote wake-up.

8.8.3 QSPI Master

The QSPI Master provides an interface for accessing external memory slaves. Depending on the type of the  memory slave, the interface between master and slave consists of up to four data signals (IO0, IO1, IO2, IO3),  a serial clock (SCLK) and a slave select signal (SSN). It supports the Single SPI, Dual SPI and Quad SPI  transfer modes.

8.8.4 SPI Master

The QPG6200L contains a Serial Peripheral Interface (SPI) for interfacing with additional peripheral devices.  This SPI Master supports:

• Full-duplex synchronous transfers on three lines (MISO, MOSI, SCLK)

• Programmable clock polarity and phase, supports SPI mode 0, 1, 2 and 3

• Programmable data order with MSb-first or LSb-first shifting

• High speed clock generator supporting clock speeds up to 32 MHz

• 4-bit to 16-bit transfer frame format selection

• Three separate interrupts on TX Complete, TX Data Register Empty and RX Complete.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 19 of 59 www.qorvo.com

®

8.8.5 SPI Slave

The QPG6200L contains an SPI Slave interface. This SPI Slave supports: • Generic customer protocols

• Proprietary Qorvo DMA protocol

• SPI mode 0

• SPI clock frequencies up to 16 MHz

• Limited to byte-based operation

• Flexible IO mapping.

8.8.6 I2C Master

PRELIMINARY QPG6200L Multi-Protocol SoC

The QPG6200L contains two I2C (Inter-Integrated Circuit) Master interfaces, also referred to as Two-Wire  Interface (TWI), for interfacing with additional peripheral devices. This I2C Master supports:

• Standard mode and Fast mode

• Short (7-bit) and long (10-bit) addresses

• General call address (0x00)

• Clock stretching.

8.8.7 I2C Slave

The QPG6200L contains an I2C Slave interface. This I2C Slave supports:

• Standard mode and Fast mode

• Short (7-bit) and long (10-bit) addresses

• Configurable Slave Address

• General call address (0x00).

8.8.8 I2S Master/Slave

The QPG6200L contains two I2S (Inter-IC Sound) Master/Slave interfaces for interfacing with a broad range  of I2S devices such as digital audio devices, ADC’s, DAC’s, audio CODEC’s, etc. Each interface can either be  in Master mode or in Slave mode.

These I2S interfaces support:

• Full duplex transfers

• Configurable word length

• Left justified (right justified mode can be emulated)

• Double buffered, DMA capable

• Master clock frequency selectable from 62.5 kHz up to 8 MHz

• External Slave clock input supporting speeds up to 4 MHz.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information © 2023-2024 Qorvo, Inc. Page 20 of 59 www.qorvo.com

®

8.8.9 SPI Programming Interface

PRELIMINARY

QPG6200L Multi-Protocol SoC

The primary programming interface for (production) programming of the NVM is the SPI Slave interface (see  section 8.8.5). For this the signals shown in Table 7 shall be made available for programming.

Table 7: Mapping Signals for Programming via SPI

Signal

GND

VCC_DUT

RESETn

PROG_SSn

PROG_SCLK

PROG_MOSI

PROG_MISO

PROG_ENn

QPG6200L Notes

EDP Ground.

VDD The recommended supply voltage is: 3.3 V.

RESETN The reset signal.

GPIO6 Slave select signal.

GPIO7 Clock provided by the Programmer.

GPIO8 Data from Programmer to device.

GPIO9 Data from device to Programmer.

GPIO5 Low (stable) enables programming mode at startup/reset. Should be kept low at least until the first SPI access in programming

mode. The time between Reset and the first SPI access must be at least

50 ms. If no command has been received within 4 s, the chip will enter

normal application mode.

The following characteristics apply to the SPI programming interface:

• maximum SCLK frequency: 2.25 MHz

• minimum SSn high time: 2 µs

• minimum last SCLK to SSn high time: 10 µs.

8.8.10 Clock Output

The QPG6200L can provide two clocks to peripheral devices (CLK_OUT). The clock frequency is derived from  the 192 MHz crystal oscillator system clock. It can be configured for frequencies up to 96 MHz.

8.9 Application Programming Information

Available from Arm:

Generic User Guide, Cortex™-M4 Devices;

https://developer.arm.com/documentation/dui0553/a/

Available from Qorvo:

IoT Dev Kit for QPG6200L (QPG6200LDK-01) includes:

• Hardware Development Kit

• Software Development Kit

• Tools for evaluation, development and production phases

• Certified stacks and turn-key reference applications

• Documentation for the hardware, software and tools.

For more information: see Qorvo’s website.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 21 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

8.10 NVM Programming and Configuration

The NVM program and configuration memory is not programmed when the chips are shipped by Qorvo. To  enable the functionality, the NVM must be programmed through a dedicated programming protocol. Please  contact Qorvo Support for details of available programming solutions. If programming takes place on the target  PCB, the Program Port signals need to be accessible on the PCB.

8.11 Debug Mode

During startup/reset the QPG6200L can be triggered to come up in debug mode. The SWD/JTAG signals  shown in Table 8 below will then be available. The operation is dependent on the provisioned Secure Debug  configuration.

Table 8: Debug Mode Signals

SWD/JTAG

SWIO/TMS

SWCLK/TCK

TDI

SWV/TDO

PROG_ENn

QPG6200L Notes

GPIO6

GPIO7

GPIO8

GPIO9

GPIO5 Low for about 1 s after Reset, then high, enables debug mode.

~1 s Ready for first SWD/JTAG command PROG_ENn RESETN POR

Figure 7: Entering Debug Mode

While PROG_ENn is asserted low, no transaction should be started on

SPI or UART (otherwise programming mode will be entered).

The QPG6200L will be ready for the first SWD/JTAG command for a

period of 32 s. If then no command has been received, the chip will enter

normal application mode.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 22 of 59 www.qorvo.com

®

9 Power Management

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L has an integrated power management system which includes a SIDO (Single-Inductor Dual Output) DC/DC Converter with several dedicated LDO’s. When the DC/DC Converter is enabled, the power  consumption of the chip can be severely reduced by a factor of 1.5 to 3, depending on the supply voltage.  When the DC/DC Converter is not used, the Global LDO and the Core LDO are used to generate the internal  power supplies.

The chip always powers up using the Global LDO and Core LDO where after the DC/DC Converter can be  enabled or disabled by the application program.

Note that decoupling capacitors CANA and CCORE are needed for VDDANA and VDDCORE respectively. The Bandgap Reference is used for all LDO’s.

9.1 DC/DC Converter

The QPG6200L contains an integrated SIDO DC/DC Converter to reduce the power consumption of the chip.  The DC/DC Converter uses one low-cost external inductor as shown in Figure 8 below (see Table 33 for the required value). It converts the battery supply voltage to lower voltages that are used to supply the analog  LDO’s (feeding the analog/RF blocks) and the digital core blocks.

Figure 8: DC/DC Converter

The QPG6200L offers full flexibility to the application program to enable or disable the DC/DC Converter after  the chip is powered up and out of Sleep mode. During Sleep (eLPS mode), the DC/DC Converter can be enabled to save power.

The DC/DC Converter requires a minimum supply voltage level to be efficient. The chip will detect when the  input voltage gets too low (below 2.2 V) and will then automatically use the Global LDO. When the DC/DC  Converter is enabled while the input voltage is too low, it will not start up and, after 1 ms, the chip will continue  using the Global LDO.

If an application does not make use of the DC/DC Converter, the external inductor (LDCDC) is not required to  be placed. In this case, two pins of the DC/DC Converter can also be configured to be used as GPIOs (GPIO19  and GPIO20).

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 23 of 59 www.qorvo.com

®

9.2 Low Voltage Behavior

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L contains following features that can be combined to implement the desired low voltage behavior:

• VDD Brown-out Interrupt: interrupt that can be software configured to trigger when VDD drops below  the VDD Brown-out threshold while being active. This interrupt can be used to trigger the software to  disable the radio and go into Sleep mode. A higher value gives the application more time for state  cleanup.

The VDD Brown-out threshold is software configurable from 1.75 V to 2.1 V in steps of 50 mV; default  value is 1.75 V.

• Cut-Off: a VDD threshold under which all functions are disabled, and current consumption is strictly  limited.

Brown-out threshold Cut-Off threshold

1.75 V (default) 1.575 (default)

1.80 V 1.60 V

1.85 V 1.625 V

1.90 V 1.65 V

1.95 V 1.675 V

2.0 V 1.70 V

2.05 V 1.725 V

2.1 V 1.75 V

• VMT: Voltage Minimum Threshold under which the chip will not wake up from Sleep mode. Its main  function is to prevent battery leakage.

The VMT range is software configurable from 1.6 V – 3 V in 50 mV steps.

VMT crossing detection time is also software configurable; default is 125 µs. This value can be  extended to 1 ms to save current.

• POR Detector: the Power-On-Reset circuit resets the chip before the VDD voltage reaches the minimal  operating voltage. The chip embeds a Power-On-Reset circuit with internal and external triggers as  shown in the table below.

Trigger condition (typ.)

VDD << 1 V

Vresetn < ½ VDD

Detector Type Response time (typ.)

internal

(on-chip VDD monitor) 1 ms

external

(RESETn input) < 10 ns

note: the internal reset trigger circuit samples VDD, the circuit behaves like a sample-based circuit at 1 kHz. The external reset trigger uses an asynchronous (direct) input.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 24 of 59 www.qorvo.com

®

10 Security

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L is equipped with a low-power Secure Element subsystem providing secure operation  functionalities to the application. This subsystem contains a dedicated secure microcontroller, memories and interfaces. It can work independently from the Application Microcontroller, IEEE 802.15.4 MAC and Bluetooth LL.

The Secure Element Subsystem supports:

• Secure Boot (rooted in ROM with Boot Key in OTP), Physical Unclonable Function (PUF)-based  Secure (Key) Storage, Secure (Key) Provisioning, Secure Debugging, Secure Device  Attestation.

• Hardware accelerated Public Key Crypto engine for these crypto systems: ECC/ECDH/ECDSA,  EdDSA, Primality Test (Rabin-Miller), J Pake and ECMQV.

• Hardware accelerated ECC dramatically improves performance on the following ECC curves: Curve25519/Ed25519, NIST P-192, NIST P-256, NIST P-384, NIST P-521/E-521.

• Hardware accelerated Hash Crypto Engine: SHA-1, SHA-2 (SHA-224, SHA-256, SHA-384, SHA 512), SHA-128.

• Hardware accelerated AES Crypto Engine supporting encryption and decryption of cyphering modes:  ECB, CBC, CTR, CFB, CCM, CCM*, GCM, CMAC.

• Flexible Tampering Detection (Voltage tampering, BBPLL tampering, Boot authentication failure, PK ROM integrity error, SEUC RAM ECC correction or failure, Unauthorized access, Watchdog time-out).

• Cryptographic Random Number Generator featuring TRNG, DRBG, Conditioning and Health test.

• Restriction Locks to certain security-sensitive hardware aspects of QPG6200L, e.g., register maps,  NVM regions, OTP.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 25 of 59 www.qorvo.com

®

11 Electrical Characteristics

PRELIMINARY

QPG6200L Multi-Protocol SoC

The QPG6200L characteristics are determined in the circuits shown in Figure 9 to Figure 11 below.

• Current consumption values for transmit and receive are specified with and without the DC/DC  Converter enabled.

• Transmit as well as receive behavior is measured in accordance with the IEEE 802.15.4 specification  and the Bluetooth Test Specification (RF-PHY.TS.5.0.1).

• All parameters are measured at VDD = 3.0 V and TA = 25 oC, unless otherwise specified. • IEEE 802.15.4 channel rejection is measured with the QPG6200L reference design system as interferer.

Figure 9: Parameter Evaluation Circuit (sheet 2 of 3)

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 26 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Figure 10: Parameter Evaluation Circuit (sheet 1 of 3)

Figure 11: Parameter Evaluation Circuit (sheet 3 of 3)

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 27 of 59 www.qorvo.com

®

11.1 Absolute Maximum Ratings  Table 9: Absolute Maximum Ratings

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameter | Value |
| --- | --- |
| Power supply input voltage | -0.3 to +3.6 |
| Digital IO voltage | -0.3 to VDD+0.3 (Max = +3.6) |
| Analog IO voltage | -0.3 to +3.6 |
| Analog IO voltage | -0.3 to +1.05 |
| Decoupling voltage | -0.3 to +1.05 |
| Decoupling voltage | -0.3 to +1.05 |
| RF IO voltage | -0.3 to +1.8 (ESD clamp diodes may limit applied voltage) |
| Input RF level | +10 |
| Junction Temperature | +145 |
| Storage Temperature | -50 to +150 |
| Reflow Soldering Temperature | +260 |

Symbol Unit VDD

Digital pins

(see Table 36)

ANIO0..ANIO1

Other analog pins

(see Table 36) VDDCORE VDDANA

RF0, RF1

V

PMAX dBm TJ °C Tstg °C Tsol °C

| ESD Test name | Test standard |
| --- | --- |
| ESD Human Body Model (ESD HBM) | ANSI/ESDA/JEDEC JS-001 2023 |
| ESD Charged Device Model (ESD CDM) | ANSI/>ESDA/JEDEC JS-002 2022 |

Test results TBD

TBD

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 28 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

11.2 Recommended Operating Conditions

Table 10: Recommended Operating Conditions

Symbol Units

| Parameter | Condition | Min | Typ | Max |
| --- | --- | --- | --- | --- |
| Power supply input Voltage |  | 1.71 | 3.0 | 3.6 |
| Ambient Temperature |  | -40 | +25 | +125 |
| Reference Crystal Oscillation Frequency |  |  | 32 |  |
| Input HIGH voltage (for all GPIO lines) (*) | VDD=1.71 V | - | - | VDD |
|  | VDD=3.0 V | - | - | VDD |
|  | VDD=3.6 V | 2.0 | - | VDD |
| Input LOW voltage (for all GPIO lines) (*) | VDD=1.8 V | VSS | - | - |
|  | VDD=3.0 V | VSS | - | - |
|  | VDD=3.6 V | VSS | - | 0.8 |
| Output HIGH voltage (for all GPIO lines) (**) | VDD=1.8 V | - | - | VDD |
|  | VDD=3.0 V | 2.4 | - | VDD |
|  | VDD=3.6 V | - | - | VDD |
| Output LOW voltage (**) (for all GPIO lines) (**) | VDD=1.8 V | VSS | - | - |
|  | VDD=3.0 V | VSS | - | 0.4 |
|  | VDD=3.6 V | VSS | - | - |
| Resistance input pull-up resistor (for all GPIO lines) | VDD=1.8 V | - | - | - |
|  | VDD=3.0 V | - | - | 61.4 |
|  | VDD=3.6 V | 16 | - | - |
| Resistance input pull down resistor (for all GPIO lines) | VDD=1.8 V | - | - | - |
|  | VDD=3.0 V | - | - | 61.4 |
|  | VDD=3.6 V | 16 | - | - |
| Input hysteresis voltage | VDD=1.8 V | - | - | - |
|  | VDD=3.0 V | 0.2 | - | - |
|  | VDD=3.6 V | - | - | - |
| Input leakage current | VDD=1.8 V | - | - | - |
|  | VDD=3.0 V | - | - | - |
|  | VDD=3.6 V | -5 | - | +5 |
| Total sourced current for all GPIO output lines combined (excluding the high drive of GPIO0) |  |  |  | 100 |
| Pulse width for GPIO interrupts |  | 250 |  |  |

VDD V TA °C Fref MHz VIH

VIL

V

VOH

VOL

Rpu

Rpd Vh

Ipad

kΩ

V  µA

IOHmA

TINTns

Note (*): for GPIO0 with high drive sink, refer to section 11.14.

Note (**): Drive strength = 2 mA, 4 mA, 8 mA and 12 mA.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 29 of 59 www.qorvo.com

®

11.3 Current Consumption  Table 11: Current Consumption - Common

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameters | Conditions | Min Typ Max |
| --- | --- | --- |
| System state: HPS mode active (DC/DC Off) eHPS mode (DC/DC On) | App. processor: idle radio and sensors: not active App. processor running @ 64 MHz from RAM) App. processor running @ 64 MHz from NVM cache App. processor running @ 192 MHz from RAM App. processor running @ 192 MHz from NVM cache | 1.3 3.3 2.8 6.5 5.1 |
|  | App. processor: idle, radio and sensors: not active App. processor running @ 64 MHz from RAM App. processor running from NVM cache @ 64 MHz App. processor running from RAM @ 192 MHz App. processor running from NVM cache @ 192 MHz | 0.6 1.2 1.2 2.2 1.9 |
| System state: LPS mode (DC/DC Off) sleep eLPS mode (DC/DC On) | 64 kHz LjRC clock |  |
|  | 32 KB RAM retained | 1.8 |
|  | 128 KB RAM retained | 4.4 |
|  | 256 KB RAM retained | TBD |
|  | 64 kHz LjRC clock |  |
|  | 32 KB RAM retained | 0.9 |
|  | 128 KB RAM retained | 1.8 |
|  | all RAM retained | 4.5 |
| eHPSret mode (DC/DC Off) | 32 MHz sleep | 400 |
| eHPSret mode (DC/DC On) (Current consumption depends on crystal specification and load capacitance, |  | 200 |
| Reset Mode |  | 50 |

Symbol Unit Iidle

mA

Istandby

µA

Ireset µA

Current consumption while app. processor is running depends on the user program. Given numbers are max currents for  reference program used. (typical xtal 32 MHz running)

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 30 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Table 12: Current Consumption - Active State - Bluetooth Low Energy - VDD =3 V Symbol Protocol VDD

|  |  |  |  |  | Bluetooth Low Energy |  |  |  |  | 3 V |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Single Antenna |  |  |  |  | Dual Antenna / Diversity |  |  |  |  |  |
| DC/DC On |  | DC/DC Off |  |  |  |  | DC/DC On |  |  | DC/DC Off |  |  |
| Typ | Max | Min | Typ | Max | Condition | Unit | Min | Typ | Max | Min | Typ | Max |
| 2.2 |  |  | 5.5 |  | RX listening* 1 Mbit/s | mA |  | 3.1 |  |  | 8.5 |  |
| 2.2 |  |  | 5.5 |  | RX listening* 2 Mbit/s |  |  | 3.1 |  |  | 8.5 |  |
| 2.2 |  |  | 6 |  | RX listening* 500 kbit/s |  |  | 3.5 |  |  | 8.5 |  |
| 2.2 |  |  | 6 |  | RX listening* 125 kbit/s |  |  | 3.5 |  |  | 8.5 |  |
| TBD |  |  | TBD |  | RX listening* multi-rate** |  |  | TBD |  |  | TBD |  |
| 2.4 |  |  | 5.7 |  | RX receiving packet, all modes except 125 kbit/s |  |  | 3.5 |  |  | 8.5 |  |
| 2.45 |  |  | 5.9 |  | RX receiving packet, 125 kbit/s |  |  | 3.55 |  |  | 8.7 |  |
| 6 |  |  | 14 |  | Tx @ 0 dBm | mA |  | 6 |  |  | 14 |  |
| 7.5 |  |  | 18.5 |  | Tx @ 4 dBm |  |  | 7.5 |  |  | 18.5 |  |
| 19 |  |  | 48 |  | Tx @ 10 dBm |  |  | 19 |  |  | 48 |  |

Iactive

Min

Notes:* “RX listening” is defined as receive mode during preamble hunt.

Digital current included, application controller not running.

** Multi-rate includes 1 Mbit/s, 500 kbit/s and 125 kbit/s (not 2 Mbit/s).

Table 13: Current Consumption - Active State – IEEE 802.15.4 – channel 20, VDD =3 V Symbol Protocol VDD

|  |  |  |  |  | IEEE 802.15.4 |  |  |  |  | 3 V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Single Antenna |  |  |  |  |  |  |  | Dual Antenna / Diversity |  |
| DC/DC On |  | DC/DC Off |  |  |  |  | DC/DC On |  |  |  | DC/DC Off |
| Typ | Max | Min | Typ | Max | Condition | Unit | Min | Typ | Max | Min | Typ |
| 1.25 |  |  | 3 |  | RX listening* single-channel | mA |  | 2.2 |  |  | 5.5 |
| 2.3 |  |  | 6 |  | RX listening* high sensitivity mode |  |  | 3.3 |  |  | 8.5 |
| 2.3 |  |  | 6 |  | RX listening* multi-channel |  |  | 3.3 |  |  | 8.5 |
| 2.4 |  |  | 5.8 |  | Rx receiving packet |  |  | 3.4 |  |  | 8.3 |
| 6 |  |  | 14 |  | Tx @ 0 dBm | mA |  | 6 |  |  | 14 |
| 7.5 |  |  | 18.5 |  | Tx @ 4 dBm |  |  | 7.5 |  |  | 18.5 |
| 19 |  |  | 45 |  | Tx @ 10 dBm |  |  | 19 |  |  | 45 |

Iactive

Min Max

Notes:* “RX listening” is defined as receive mode during preamble hunt.

Digital current included, application controller not running.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 31 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Table 14: Current Consumption - Active State – ConcurrentConnect Multi-Radio – channel 20, VDD= 3 V

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| RX listening* Bluetooth Low Energy 1 Mbit/s only | DC/DC On | 2.3 |
| RX listening* Bluetooth Low Energy 1 Mbit/s + 125/500 kbit/s | DC/DC On | 2.3 |
| RX listening* Bluetooth Low Energy 1 Mbit/s only | DC/DC Off | 6 |
| RX listening* Bluetooth Low Energy 1 Mbit/s + 125/500 kbit/s | DC/DC Off | 6 |

Symbol Unit Iactive

mA

Notes:* “RX listening” is defined as receive mode during preamble hunt.

Digital current included, application controller not running.

Single antenna.

11.4 Receiver Characteristics

Table 15: Receiver Characteristics - Common

| Conditions | Min Typ Max |
| --- | --- |
| 5 dB accuracy AGC activated | -110 0 |
| Resolution | 1 |
| 2.4 GHz | -70 |
| 4.8 GHz | -70 |

Parameter Unit

RSSI range *

(assuming the HAL is used, see  section 6.1.3)

Note: * lowest RSSI value is below  sensitivity level.

dBm dB

LO leakage dBm

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 32 of 59 www.qorvo.com

®

Table 16: Receiver Characteristics - IEEE 802.15.4

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Conditions | Min Typ Max |  |  |
| --- | --- | --- | --- |
| Programmable in 5 MHz steps as defined by IEEE 802.15.4 | 2405 2480 |  |  |
|  | 250 |  |  |
| Single-Channel Mode (Measured in IEEE 802.15.4 channel 20) single antenna antenna diversity | -99 -98.5 |  |  |
| High Sensitivity Mode (Measured in IEEE 802.15.4 channel 20) |  |  |  |
| single antenna | -102 |  |  |
| antenna diversity | -101 |  |  |
| Multi-Channel Mode (Measured in IEEE 802.15.4, 3 channels single antenna antenna diversity | -98.5 -96 |  |  |
| Antenna Diversity Gain (refer to section 6.1.5 for the channel model) | 8 |  |  |
| Sensitivity loss < 1 dB | -220 +220 |  |  |
| 1% PER as defined in IEEE 802.15.4 | +10 |  |  |
| RX mode, AGC enabled | 25 |  |  |
| RX mode, AGC enabled | 0 |  |  |
| as defined in IEEE 802.15.4. IEEE 802.15.4 interferer, +/- 5 MHz | 32 |  |  |
| as defined in IEEE 802.15.4. IEEE 802.15.4 interferer, +/- 10 MHz | 46 |  |  |
| wanted signal at -82 dBm. IEEE 802.15.4 interferer, +/- 15 MHz | 50 |  |  |
| wanted signal at -82 dBm; Wi-Fi centered at +12 MHz / -13 MHz or higher offset frequency | 30 |  |  |
| wanted signal at -82 dBm, Bluetooth carrier at: +/-4 MHz | 30 |  |  |
| +/-6 MHz | 52 |  |  |
| (Measured according to ETSI EN 300 440-1 V1.6.1; 2010-08). |  |  |  |
| -100 MHz from lower band edge |  | -11 |  |
| -40 MHz from lower band edge |  | -13 |  |
| -20 MHz from lower band edge |  | -14 |  |
| +20 MHz from upper band edge |  | -14 |  |
| +40 MHz from upper band edge |  | -12 |  |
| +100 MHz from upper band edge |  | -10 |  |

Parameter Unit RF channels MHz

Data rate kbit/s Receiver sensitivity

dBm

dBm

dBm

dB

RX carrier frequency offset

rangekHz Maximum receive level dBm IIP3 dBm P-1dB RF front-end dBm Adjacent channel rejection dB

Alternate adjacent channel  rejection

dB

Far away channel rejection dB Wi-Fi IEEE 802.11n rejection dB

Bluetooth rejection

(fixed carrier, rejection of FSK

modulated signal with frequency

deviation +/- 160 kHz, BT=0.5)dB

Blocking / desensitization

(e.g., mobile phone signal

rejection)

dBm

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 33 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Table 17: Receiver Characteristics - Bluetooth Low Energy

| Conditions | Min Typ Max |
| --- | --- |
| Channel spacing in 2 MHz steps | 2402 2480 |
|  | -250 250 |
|  | 1 |
|  | 125 and 500 |
|  | 2 |
|  | -500 +500 |
| * (RF-PHY/RCV-LE/CA/BV-01-C); BER = 10-3(Measured in Bluetooth channel 0 = 2402 MHz) |  |
| 2 Mbit/s data rate | -94.5 |
| 1 Mbit/s data rate | -97.5 |
| 500 kbit/s | -100.5 |
| 125 kbit/s | -105 |
| * (RF-PHY/RCV-LE/CA/BV-06-C); BER = 10-3 | >+10 |
| Wanted signal at -67 dBm, modulated interferer in channel, BER = 10-3 (1 Mbps / 2 Mbps) Wanted signal at -72 dBm, modulated interferer in channel, BER = 10-3 (500 kbps) Wanted signal at -79 dBm, modulated interferer in channel, BER = 10-3 (125 kbps) | -9 -9 -9 |
| * (RF-PHY/RCV-LE/CA/BV-03-C); Wanted signal at -67 dBm, BER = 10-3, (1 Mbps / 2 Mbps) modulated interferer at: |  |
| -5 MHz or more | 54 |
| -4 MHz | 52 |
| -3 MHz | 50 |
| -2 MHz | 38 |
| -1 MHz | 5 |
| +1 MHz = image frequency -1 MHz | 5 |
| +2 MHz = image frequency | 33 |
| +3 MHz = image frequency +1 MHz | 44 |
| +4 MHz = image frequency | 52 |
| +5 MHz or more | 54 |

Parameter Unit RF channels MHz Frequency error tolerance kHz

Bit rates (LE 1M) Mbit/s (LE Coded) kbit/s (LE 2M) Mbit/s

Data rate error tolerance ppm Receiver sensitivity

dBm

Receiver saturation dBm Co-Channel rejection

dB

Selectivity

dB

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 34 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

| Conditions | Min Typ Max |
| --- | --- |
| * (RF-PHY/RCV-LE/CA/BV-03-C); For 125 kbps: wanted signal at -79 dBm, BER = 10-3. For 500 kbps: wanted signal at -72 dBm, BER = 10-3. Modulated interferer at: |  |
| -5 MHz or more | 56 |
| -4 MHz | 52 |
| -3 MHz | 48 |
| -2 MHz | 37 |
| -1 MHz | 7 |
| +1 MHz = image frequency -1 MHz | 7 |
| +2 MHz = image frequency | 33 |
| +3 MHz = image frequency +1 MHz | 44 |
| +4 MHz = image frequency | 52 |
| +5 MHz or more | 56 |
| * (RF-PHY/RCV-LE/CA/BV-04-C); 30 ... 2000 MHz | -8 |
| 2003 ... 2399 MHz | -6 |
| 2484 ... 2997 MHz | -5 |
| 3000 MHz ... 12.75 GHz | 3 |
| * (RF-PHY/RCV-LE/CA/BV-05-C); Wanted signal at 2402 MHz, at -64 dBm. Two interferers at 2405 and 2408 MHz respectively, at the given power level. | -28 |

Parameter Unit Selectivity

dB

Out-of-band blocking

dBm

Intermodulation

Note: * As defined in Bluetooth Test Specification RF-PHY.TS.5.0.1, and for 1 Mbit/s bit rate.

Table 18: Receiver Characteristics - ConcurrentConnect Multi-Radio Mode

| Conditions | Min Typ Max |
| --- | --- |
| 1 Mbps | -97.5 |
| 500 kbps | -100.5 |
| 125 kbps | -100.5 |
| IEEE 802.15.4 | -97 |

Parameter Unit Receiver Sensitivity

dBm

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 35 of 59 www.qorvo.com

®

11.5 Transmitter Characteristics  Table 19: Transmitter Characteristics - Common

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Conditions | Min Typ Max |
| --- | --- |
|  | 10 |
| Active TX | -25 |
| Over full temperature range * | 3 |
| Over full power supply voltage range | 0.1 |
| Over Tx frequency | 1 |
| Part to part variation (+/-3σ) | TBD |
| Conducted measurement at max. output power (1 MHz resolution bandwidth, average power and modulated carrier) | -41 |
| Measured at max. output power, modulated signal, on all IEEE 802.15.4 and Bluetooth channels. (1 MHz resolution bandwidth, average power) < 2390 MHz | < -41 |
| > 2483.5 MHz | < -41 |

Parameter Unit Maximum TX output power dBm

Minimum TX output power dBm TX output power variation

dB

TX Harmonics dBm TX out of band emissions

dBm

Note: * TX output power variation over full temperature range is with no

calibration. Device does not require any TX power calibration.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 36 of 59 www.qorvo.com

®

Table 20: Transmitter Characteristics - IEEE 802.15.4

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Conditions | Min Typ Max |
| --- | --- |
| Programmable in 5 MHz steps as defined by IEEE 802.15.4 | 2405 2480 |
|  | 250 |
|  | 2.0 |
| Channel 11-24: Dirac impulse digital filter | 0.5 |
| Channel 25 -26: Gaussian digital filter (for spectral mask to meet FCC band-edge requirements at max. Tx power) | 1.5 |

Parameter Unit RF channels MHz

Data rate kbit/s Chip rate Mchip/s Offset EVM

%

Table 21: Transmitter Characteristics - Bluetooth Low Energy

| Conditions | Min Typ Max |
| --- | --- |
| Channel spacing in 2 MHz steps | 2402 2480 |
|  | 1 |
|  | 125 and 500 |
|  | 2 |
| * (RF-PHY/TRM-LE/CA/BV-03-C) +/-2 MHz | TBD |
| +/-(3+n) MHz (n=0,1,2…) | TBD |
| * (RF-PHY/TRM-LE/CA/BV-06-C) ∆f1avg1 Mbit/s ∆f2avg / ∆f1avg | 225 275 |
|  | >0.92 |
| ∆f1avg2 Mbit/s ∆f2avg / ∆f1avg | 450 550 |
|  | >0.92 |

Parameter Unit RF channels MHz Bit rates (LE 1M) Mbit/s

(LE Coded) kbit/s (LE 2M) Mbit/s In-band emissions

dBm

Frequency deviation

kHz

kHz

Note: * As defined in Bluetooth Test Specification RF-PHY.TS.5.0.1, and for 1 Mbit/s bit rate unless  specified otherwise.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 37 of 59 www.qorvo.com

®

11.6 Digital Timing Characteristics  Table 22: SPI Slave Timing Characteristics

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameter | Reference (Figure 12) | Min Typ Max |
| --- | --- | --- |
| SCLK frequency | t1 | 0 16 |
| SCLK duty cycle clock |  | 50 |
| MOSI setup time | t2 | 10 |
| MOSI hold time | t3 | 10 |
| SCLK low to MISO valid time | t4 | 16 |
| SSn setup time | t5 | 31.25 |
| SSn high to MISO tri-state | t6 | 31.25 |

Symbol Unit

FSCLK MHz %

ns

ns

ns

ns

ns

t1 SCLK t2 t3 MOSI t4 MISO t6 t5 SSn

Figure 12: SPI Slave Signaling Timing Diagram

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 38 of 59 www.qorvo.com

®

Table 23: I2C Timing Characteristics

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameter | Reference (Figure 13) | Standard Mode | Fast Mode |
| --- | --- | --- | --- |
|  |  | Min Max | Min Max |
| SCL frequency | t1 | 100 | 400 |
| Clock High Time | t2 | 4 | 0.6 |
| Clock Low Time | t3 | 4.7 | 1.3 |
| START condition setup time | t4 | 4 | 0.6 |
| START condition hold time | t5 | 4.7 | 0.6 |
| Data hold time | t6 | 0 | 0 |
| Data setup time | t7 | 0.25 | 0.1 |
| STOP condition setup time | t8 | 4 | 0.6 |
| Bus free time between a STOP and a START condition | t9 | 4.7 | 1.3 |

Symbol Unit FSCL kHz tHIGH µs tLOW µs tSU;STA µs tHD;STA µs tHD;DAT µs tSU;DAT µs tSU;STO µs tBUF µs

t1 t3t2 SCL t6 t7 t8 t4 t5 SDA t9

Figure 13: I2C Signaling Timing Diagram

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 39 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

11.7 Reset, Wake up and Sleep Timing Characteristics

Table 24: Reset, Wake up and Sleep Timing

| Remarks | Min Typ Max |
| --- | --- |
| See Figure 14 below. | 10 |
| See Figure 14 below. | 45 |
| See Figure 15 below. | 45 |

Application use cases Unit Power on detect ms

From Power on detect, until  application main function

External Reset, until application  main function

ms ms

RESETN pulse width See Figure 14 below. The RESETN is asynchronous. A practical  minimum is 10 ns.

From sleep trigger in HPS mode until  in LPS standby mode

From sleep trigger in eHPS mode  until in eLPS standby mode

From sleep trigger in HPS mode until  in HPSret standby mode

Wake up from trigger in LPS mode until HPS back in main function

Wake up from trigger in eLPS mode  until eHPS back in main function

Wake up from trigger in HPSret mode until HPS back in main  function

|  | 200 |
| --- | --- |
|  | 253 |
|  | 150 |
|  | 2 |
|  | 2.1 |
|  | 216 |

µs

µs

µs

ms

ms

µs

From Power on detect, until Program starts (optional) Prepare for Rx/Tx running VDD RESETN VDDDIG Program running // Power on detect Tx/Rx possible Start up 32MHz crystal Digital state // init. Internal clock 32MHz clock//

Figure 14: Power On Timing (not to scale)

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 40 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

External Reset, until Program (optional) Prepare for Rx/Tx starts running VDD RESETN VDDDIG Program running // Tx/Rx possible Start up 32MHz crystal Digital // state init. Internal clock 32MHz clock//

Figure 15: External Reset Timing (not to scale)

11.8 NVM Characteristics

Table 25: NVM Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Retention period | 85°C | 10 |
| Number of ERASE cycles |  | 10k |
| VDD for programming | Zero source resistance | 1.8 3.6 |
| Data word size |  | 128 |
| Write time (1 word) | Physical operations, so excl. software overhead and transmission times | 50 |
| Write current | Average delta current | 13 |

Symbol Unit year

V

bits

TWR

µs

IWR mA

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 41 of 59 www.qorvo.com

®

11.9 Crystal Oscillator Specifications 11.9.1 32 MHz Crystal Oscillator

PRELIMINARY

QPG6200L Multi-Protocol SoC

The 32 MHz crystal oscillator is an AGC controlled oscillator that provides a high gain at start-up, to assure  fast start-up times, and low gain when running, to minimize current consumption. It generates the system clock  for the QPG6200L and can also be used as time base generation.

For reliable operation, and to meet the specified Sleep current and startup time, the crystal should comply with  the Qorvo Procurement Specifications for the crystal. These specifications, as shown in Table 26, will become  available at Qorvo’s website soon.

Qorvo’s Crystal Procurement Tool (available soon) can be used to check if the electrical parameters, given in  the crystal specification, can meet Qorvo procurement requirements. If needed, a header file for frequency  offset compensation over temperature can be extracted.

Table 26: 32 MHz Crystal Specifications

| Size | Type |
| --- | --- |
| 3.2 x 2.5 mm | 4 pads SMD |
| 2.0 x 1.6 mm | 4 pads SMD |
| 1.6 x 1.2 mm | 4 pads SMD |

Package Procurement Specification SMD GP_P010_PS_20298 SMD GP_P010_PS_20299 SMD GP_P010_PS_20300

Figure 16 below shows the typical configuration of the oscillator.

The QPG6200L features internal programmable crystal load capacitors that can be set from 13.2 - 25 pF each. Note that the values of the internal and optional external load capacitors (Cx, Cy) are crystal type dependent.

Figure 16: Typical 32 MHz Crystal Configuration

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 42 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

11.9.2 32 KiHz Crystal Oscillator (optional)  The 32.768 kHz (in short: 32 KiHz) crystal oscillator is optional  and can be used for ultra-low power time base generation for  the Event Scheduler with high accuracy. A side effect of this  low power consumption is that the start-up time of the 32 KiHz  oscillator is very dependent on the crystal and the capacitive  load on the X32K oscillator pins.

Within the operational temperature range, the 32 KiHz  oscillator will always start within one second (at 25°C the start up time is less than 0.5 s). The application must make sure  that the QPG6200L does not enter the 32 KiHz Sleep mode before the 32 KiHz oscillator is stable.

QPG6200Figure 17: 32 KiHz Crystal Configuration

For reliable operation and to meet the specified characteristics, the crystal should comply with Qorvo Procurement Specification GP_P010_PS_20408. This specification will become available at Qorvo’s website  soon.

Figure 17 above shows the application circuit of this oscillator.

Note: the crystal load capacitors are integrated in the chip.

11.10 Internal Pull-up / Pull-down Characteristics

Table 27: Internal Pull-up / Pull-down Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Internal pull-up resistance | VDD=3.3V, TA=25oC, Input is grounded. | 41 |
|  | VDD=1.71V, TA=25°C, Input is grounded. | 99 |
| Internal pull-down resistance | VDD=3.3V, TA=25oC, Input is VDD level. | 42 |
|  | VDD=1.71V, TA=25°C, Input is VDD level. | 115 |

Symbol Unit kΩ

11.11 ADC Characteristics

Table 28: General Purpose ADC Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Sample rate |  | 0.04 1 |
| Resolution | Sample rate up to 1 MS/s | 11 |
|  | Oversampling ratio (OSR) = 16 | 13 |
| Measurement range | At ANIO pin single ended differential | 0 VDD -VDD VDD |
| Channel Switching Time |  | 4 |
| Signal-to-Noise Ratio | Single ended, 1.07 MS/s | 50.5 |
|  | Differential, 1.07 MS/s | 56.5 |
| Integral Nonlinearity | Single ended, OSR = 1024 | 2 |
|  | Differential, with chopping, OSR = 512 | 1 |
| Differential Nonlinearity | Differential measurement | > -1 |

Symbol Unit MS/s

bits

bits

VIN V

µs

SNR dB

INL

LSb

DNL LSb

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 43 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Table 29: High-Resolution ADC – General Characteristics

| Parameter | Conditions |  | Min Typ Max |
| --- | --- | --- | --- |
| Sample rate |  |  | 3.9 15.6 |
| Oversampling ratio |  |  | 64 256 |
| Resolution |  |  | 16 |
|  | At the ANIO pin | single-ended | 0 VDD |
|  |  | differential | -VDD VDD |

Symbol Unit kSps

bits

VIN V  Table 30: High-Resolution ADC - SNDR and Nonlinearity Characteristics

Unit

SNDR

dB

INL

LSb

| Symbol Parameter | Conditions | Typ | Typ | Typ | Typ |
| --- | --- | --- | --- | --- | --- |
| Measurement range (*) | Scaler gain | 1/4x | 1x | 4x | 8x |
|  | Single-ended | 0 to +VDD | 0 to +1 V | 0 to +0.25 V | 0 to +0.125 V |
|  | Differential | -VDD to +VDD | -1V to +1 V | -0.25 V to +0.25 V | -0.125 V to +0.125 V |
| Signal-to-(Noise +Distortion) Ratio | Single ended | 77 | 77 | 77 | 77 |
|  | Differential | TBD | TBD | TBD | TBD |
| Integral Nonlinearity | OSR = 256 | 8 | 8 | 8 | 8 |
|  | Differential measurement, OSR = 256 | TBD | TBD | TBD | TBD |
| Differential Nonlinearity | Differential measurement | >-2 | >-2 | >-2 | >-2 |

DNL LSb Note (*): the ADC contains a scaler, which allows several measurement ranges.

11.12 Battery and Temperature Monitor Characteristics

Table 31: Battery Monitor Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Battery level range |  | 1.71 3.6 |
| Resolution of battery level measurement |  | 10 |
| Accuracy of battery level measurement | Max over process, voltage and temperature. | 25 60 |

Symbol Unit V

mV

mV

Table 32: Temperature Monitor Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Temperature measurement range |  | -40 +125 |
| Resolution of temperature measurement |  | 0.15 |
| Accuracy of temperature measurement |  | 3 |
| Temperature measurement conversion time |  | 128 |

Symbol Unit °C

µs

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 44 of 59 www.qorvo.com

®

11.13 DC/DC Converter Information  Table 33: DC/DC Converter Information

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Inductance of DCDC inductor |  | 3.3 |
| Efficiency | L_DCDC= 3.3 µH (10 mA load) | 85 |

Symbol Unit L_DCDC µH

EffDCDC %

Figure 18: DCDC Converter Efficiency at Higher Current Loads

11.14 GPIO0 High Drive Sink Characteristics

Table 34: High Drive Sink Characteristics

| Parameter | Remarks | Min Typ Max |
| --- | --- | --- |
| Current handling | Advised max current, going beyond may reduce lifetime and might cause failure or increased leakage | 600 |
| Off / leakage current | Leakage current when driver is not sinking current | 0.5 |
| On resistance | Equivalent resistance when driver is sinking current | 0.8 |
| Rise time of the current | Time to reach 90% of the target end voltage. Measured with VDD = 3.6 V and external load resistor of 5 Ω (resulting to a 600 mA current when active). | 50 |
| Output Low Voltage | At 600 mA continuous current VDD = 1.71 V | TBD |
|  | VDD = 3.0 V | 0.50 |
|  | VDD = 3.6 V | 0.45 |

Symbol Unit IMAX mA

IOFF µA RON Ω TRISE ns

VOL

V

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 45 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

11.15 64 kHz LjRC Oscillator Characteristics

Table 35: 64 kHz LjRC Oscillator Characteristics

| Parameter | Conditions | Min Typ Max |
| --- | --- | --- |
| Frequency stability | 1σ value, TA=25°C, calibration performed at least every 2.5 s for an averaging period >1 ms | +/-100 |
| Temperature coefficient |  | < +/-100 |

Symbol Unit 64kRC-tolerance ppm

64kRC-Tc ppm/K

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 46 of 59 www.qorvo.com

®

12 Device Information 12.1 Package

12.1.1 Pin Assignments

PRELIMINARY

QPG6200L Multi-Protocol SoC

Figure 19 below shows the (top view) pin connections for the QPG200L. Table 36 lists the pin assignments.

Table 37 and Table 38 provides GPIO assignment options for various functions. For the software  configuration options of the GPIO pins, please refer to section 7.2 section and Table 36.

Unless specified otherwise, at power-up/reset all GPIO pins are default set in floating mode.

Figure 19: Pin Connections – Top View

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 47 of 59 www.qorvo.com

®

Table 36: Pin Assignments

PRELIMINARY

QPG6200L Multi-Protocol SoC

Pin # Notes

1

2

Connection to inductor for

optional DC-DC converter. See

| Name | Type | Description |
| --- | --- | --- |
| VDD | Power | Power supply input |
| LXH_GPIO19 | Power or Digital | Optional DC-DC converter output, or configurable GPIO |
| LXL_GPIO20 | Power or Digital | Optional DC-DC converter output, or configurable GPIO |
| X32KP_GPI25 | Analog or Digital | Optional 32 KiHz reference xtal input, or configurable digital input |
| X32KN_GPI26 | Analog or Digital | Optional 32 KiHz reference xtal output, or configurable digital input |
| GPIO18 | Digital | Configurable GPIO |
| GNDRF0 | RF | RF0 ground return |
| RF0 | RF | RF port 0 for antenna |
| RF1 | RF | RF port 1 for antenna |
| GNDRF1 | RF | RF1 ground return |
| ANIO0_GPI28 | Analog or Digital | ADC’s input, or configurable GPIO (input only) |
| ANIO1_GPI29 | Analog or Digital | ADC’s input, or configurable GPIO (input only) |
| X32MP | Analog | 32 MHz reference crystal input |
| X32MN | Analog | 32 MHz reference crystal ground |
| LPBIAS_GPIO21 | Analog or Digital | Programmable Low Power bias voltage, or configurable GPIO |
| IRLED_GPIO0 | Digital | Configurable GPIO |
| GPIO1 (USB VBUS) | Digital | Configurable GPIO |
| GPIO2 (USBP) | Digital | Configurable GPIO |
| GPIO3 (USBN) | Digital | Configurable GPIO |
| GPIO4 | Digital | Configurable GPIO |
| GPIO5 (PROG_ENN) | Digital | Configurable GPIO |
| GPIO6 (PROG_SSN) | Digital | Configurable GPIO |
| GPIO7 (PROG_SCLK) | Digital | Configurable GPIO |

sections 9.1 and 11.13. 3

4 Both pins should either be used  for the 32 KiHz reference crystal

or defined as GPIO. If a pin is

not used, it must be connected

5

to the Exposed Die Pad (GND)

or software should disable the

input buffer, to prevent leakage

current.

If defined as GPIO, apply

external pull up or pull down to

avoid floating input.

No internal pull-up or pull-down

capability.

6

7 Must be connected to the  Exposed Die Pad (GND).

Preferably with a top-level trace.

8 In all cases the RF pins should  be isolated from ground or VDD. 9 10 Must be connected to the  Exposed Die Pad (GND).

Preferably with a top-level trace.

11

ANIO pins that are not used, are

recommended to be connected

to the Exposed Die Pad (GND). 12

13

The QPG6200L does not

14 support an external clock. 15

16 GPIO has High Drive function,  e.g., for IR LED. Up to 600 mA

output drive strength.

17 USB VBUS 18 Also serves as USB differential  data D+.

19 Also serves as USB differential  data D-.

20

21 Also serves as PROG_ENN. 22 Also serves as PROG_SSN. 23 Also serves as PROG_SCLK.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 48 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

Pin # Name Type Description Notes  24 Also serves as PROG_MOSI.

| GPIO8 (PROG_MOSI) | Digital | Configurable GPIO |
| --- | --- | --- |
| RESETN | Digital | Active-low reset circuit |
| GPIO9 (PROG_MISO) | Digital | Configurable GPIO |
| GPIO10 | Digital | Configurable GPIO |
| GPIO11 | Digital | Configurable GPIO |
| GPIO12 | Digital | Configurable GPIO |
| GND | Digital | Ground |
| VDDCORE | Analog | Digital core voltage decoupling |
| VDDANA | Analog | Analog voltage decoupling |
| EDP | Ground | Exposed Die Pad |
| GND1 | Ground | Additional ground connections, internally connected to EDP via the lead frame. |
| GND2 | Ground |  |
| GND3 | Ground |  |
| GND4 | Ground |  |

25 Internally pulled up, so no  external pull up is required.

This pin shall be available for

NVM programming, see section

8.8.9.

26 Also serves as PROG_MISO. 27

28

29

30 Connected to Exposed Die Pad  for ESD.

31 Decoupling to ground.

32 Decoupling to ground. 33 Analog chip ground. 34

35 36 37

Available for easier ground  routing.

For the software configuration options of the GPIO pins, please refer to Table 37 and Table 38.  Unless specified otherwise, at power-up/reset all GPIO pins are default set to floating mode.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 49 of 59 www.qorvo.com

PRELIMINARY

®

Table 37: GPIO Assignment Table – SPI / I2C / I2S / UART / PDM / SWD/JTAG / USB

QPG6200L Multi-Protocol SoC

| Pin# | Wake Up | QSPI | SPI Master | SPI Slave | I2C Master_ 0/1 | I2C Slave | I2S Master / Slave_0 | I2S Master / Slave_1 | UART 0 /1 / 2 | PDM0 (mic) | PDM1 (mic) | SWD JTAG |  | Trace |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | x | SCLK | MOSI | - | SDA / - | SDA | SDO / WS_IN | - | TX / RX / TX | DATA | CLK | - | - | TRACE_CLK |
| 17 | x | IO_0 | MISO | MISO | SCL / - | SCL | SDI / REF_IN | SDI / REF_IN | RX / TX / RX | CLK |  | DATA SWDIO TMS |  | TRACE_DATA_0 |
| 18 | x | IO_2 | SSn | SSN | SDA / - | SDA | WS / WS_IN | WS / WS_IN | TX / RX / CTS | DATA | CLK | SWCLK TCK |  | TRACE_DATA_1 |
| 19 | x | IO_1 | SCLK | SCLK | SCL / - | SCL | SCK / SCK_IN | SCK / SCK_IN | RX / TX / - | CLK | - | - | TDI | TRACE_DATA_2 |
| 20 | x | IO_3 | MOSI | MOSI | SDA / - | SDA | SDO / - | SDO / - | CTS / CTS / - | DATA | - | SWV | TDO | TRACE_DATA_3 |
| 21 | x | SCLK | MISO | MISO | - | - | SDI / REF_IN | - | - | - | - | - | - | TRACE_CLK |
| 22 | x | IO_0 | SSn | SSn | - | - | WS / WS_IN | - | - | - | - | SWDIO TMS |  | - |
| 23 | x | IO_2 | SCLK | SCLK | - | - | SCK / SCK_IN | - | - | - | - | SWCLK TCK |  | - |
| 24 | x | IO_1 | MOSI | MOSI | - | - | SDO / - | - | TX / RX / - | - | - | - | TDI | - |
| 26 | x | IO_3 | MISO | MISO | - | - | SDI / - | - | RX / TX / - | - | - | SWV | TDO | - |
| 27 | x | SCLK | SSn | SSn | - / SDA | - | WS / WS_IN | - | TX / TX / TX | - | - | - | - | - |
| 28 | x | IO_0 | SCLK | SCLK | SCL / SCL | - | SCK / SCK_IN | - | RX / RX / - | DATA | CLK | - | - | TRACE_CLK |
| 29 | x | IO_2 | MOSI | MOSI | SDA / - | - | SDO / - | SDO / - | CTS / TX / RX | CLK |  | DATA SWDIO TMS |  | TRACE_DATA_0 |
| 6 | x | - | - | - | SCL / SCL | SCL | SDI / - | - | TX / CTS / TX | - | CLK | - | - | - |
| 2 | - | IO_1 | MOSI | - | SCL / - | SCL | SDO / - | - | - / - / RX | - | - | - | - | - |
| 3 | - | IO_3 | MISO | - | SDA / - | SDA | SDI / - | - | - / - / TX | - | - | - | - | - |
| 15 | x | IO_3 | SCLK | - | SCL / - | SCL | SCK / - | - | RX / TX / CTS | CLK | - | - | - | TRACE_DATA_0 |
| 4 | x | - | MISO | - | - | - | - / WS_IN | - | RX / - / CTS | - | - | - | - | - |
| 5 | x | - | - | - | - | - | - / SCK_IN | - | - / - / CTS | - | DATA | - | - | - |
| 11 | x | - | - | - | - | - | - / SCK_IN | - | - | - | DATA | - | - | - |
| 12 | x | - | MISO | - | - | - | SDI / REF_IN | SDI / REF_IN | CTS / CTS / - | DATA | - | - | - | - |

Pin

Name USB

GPIO0 - GPIO1 VBUS GPIO2 DP (+) GPIO3 DM (-) GPIO4 - GPIO5 - GPIO6 - GPIO7 - GPIO8 - GPIO9 - GPIO10 -

GPIO11 - GPIO12 - GPIO18 - GPIO19 - GPIO20 - GPIO21 -

GPI25 - GPI26 - GPI28 - GPI29 -

I2C bus signals (SCL, SDA) require external pull up.

I2S Master uses SDO, SDI, SCK, WS and optionally REF_CLK.

I2S Slave uses SDO, SDI, SCK_IN, WS_IN and optionally REF_CLK. WS_IN and SCK_IN may require external pull down to prevent floating input signals, depending on the behavior  of the master device (e.g. during sleep state).

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 50 of 59 www.qorvo.com

PRELIMINARY

®

QPG6200L Multi-Protocol SoC

Table 38: GPIO Assignment Table – Keybd Scan / IR / PWMXL / LED / Clock Output / Antsw_Ctrl / Coex

| Pin# | Wake Up | Keyboard Scan (column = input, row = output) | IR | PWMXL | LED Pixel Protocol Master | LED | Clock Output | Antsw_ Ctrl | Coex MS | Coex SL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | x | Column 7 |  | OUT (*) PWMXL_0 | - / TX | LED0 | CLKGEN_1 | - | COEX_MS_GRANTB | COEX_SL_REQ |
| 17 | x | Column 0, Row 0 | - | PWMXL_1 | - | LED1 | - | - | COEX_MS_REQB | - |
| 18 | x | Column 1, Row 1 | - | PWMXL_2 | - | LED2 | - | - | COEX_MS_PRIOB | - |
| 19 | x | Column 2, Row 2 | - | PWMXL_3 | - | LED3 | - | - | - | - |
| 20 | x | Column 3, Row 3 | OUT | PWMXL_6 | - | - | CLKGEN_1 | PHY_ANTSW_2 | COEX_MS_REQA | COEX_SL_REQ_NOT |
| 21 | x | Column 4, Row 4 | - | PWMXL_7 | - | - | CLKGEN_0 | - | COEX_MS_PRIOA | COEX_SL_PRIO_0 |
| 22 | x | Column 5, Row 5 | - | PWMXL_0 | - | - | - | - | - | - |
| 23 | x | Column 6, Row 6 | - | PWMXL_1 | - | - | - | - | - | - |
| 24 | x | Column 7, Row 7 | - | PWMXL_2 | TX / - | - | - | - | - | - |
| 26 | x | Column 0, Row 0 | - | PWMXL_3 | - / TX | LED3 | - | PHY_ANTSW_2 | - | - |
| 27 | x | Column 1, Row 1 | - | PWMXL_4 | TX / - | LED2 | CLKGEN_0 | PHY_ANTSW_3 | - | - |
| 28 | x | Column 2, Row 2 | - | PWMXL_5 | - / TX | LED1 | CLKGEN_1 | PHY_ANTSW_0 | COEX_MS_GRANTB | COEX_SL_PRIO_0 |
| 29 | x | Column 3, Row 3 | OUT | PWMXL_0 | TX / - | LED0 | CLKGEN_0 | PHY_ANTSW_1 | COEX_MS_GRANTA | COEX_SL_REQ_NOT |
| 6 | x | Column 1, Row 1 Row 4 | OUT | PWMXL_0 | TX / - | LED3 | CLKGEN_0 | PHY_ANTSW_2 | COEX_MS_GRANTB | COEX_SL_GRANT |
| 2 | - | - | - | PWMXL_5 | - | - | - | PHY_ANTSW_2 |  | COEX_SL_PRIO_1 |
| 3 | - | - | - | PWMXL_0 | - |  | - | PHY_ANTSW_3 |  | COEX_SL_REQ |
| 15 | x | Column 6 | OUT | PWMXL_7 | TX / - | LED3 | CLKGEN_0 | - | COEX_MS_GRANTA | COEX_SL_PRIO_1 |
| 4 | x | Column 2 | - | - | - | - | - | - | COEX_MS_REQB | COEX_SL_GRANT |
| 5 | x | Column 3 | - | - | - | - | - | - | COEX_MS_PRIOB |  |
| 11 | x | Column 1 | - | - | - | - | - | - | COEX_MS_PRIOA |  |
| 12 | x | Column 2 | - | - | - | - | - | - | COEX_MS_REQA | COEX_SL_GRANT |

Pin

Name

Coex

Attenuation

GPIO0 - GPIO1 - GPIO2 - GPIO3 - GPIO4 - GPIO5 - GPIO6 - GPIO7 - GPIO8 - GPIO9 - GPIO10 - GPIO11 - GPIO12 - GPIO18 COEX_ATT_CTRL_2

GPIO19 - GPIO20 - GPIO21 -

GPI25 COEX_ATT_CTRL_1 GPI26 COEX_ATT_CTRL_2 GPI28 COEX_ATT_CTRL_0 GPI29 COEX_ATT_CTRL_1 (*) GPIO0 is generally preferred for IR due to increased drive strength possibility of GPIO0 (see sections 7.2 and 11.14) .

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 51 of 59 www.qorvo.com

PRELIMINARY

QPG6200L

®

Multi-Protocol SoC

12.1.2 Package Drawing and Recommended Land Pattern

PIN 1

TOP SIDE

1:1 BOT

®

Figure 20: Package Drawing and Dimensions Figure 21: Recommended Land Pattern

1:1 RLP

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 52 of 59 www.qorvo.com

®

12.1.3 Package Information  pin 1 indicator

part number

PRELIMINARY

QPG6200L

Multi-Protocol SoC

Figure 22: Package Information

Note: Product code = first 8 characters of the part number

12.1.4 Thermal Characteristics

Table 39: Thermal Characteristics

| Parameter | Conditions | Value |
| --- | --- | --- |
| Thermal resistance from junction to ambient | JEDEC JESD 51-2; JEDEC 2S2P (4L) board as per JESD 51-7 | 40 |
| Junction-to-Top (of package) thermal characterization parameter | JEDEC JESD 51-12 | 15 |

Symbol Unit Theta JA

(RθJA) K/W Psi-JT

(ΨJT) K/W

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 53 of 59 www.qorvo.com

®

12.1.5 Tape and Reel Information

PRELIMINARY

QPG6200L Multi-Protocol SoC

12.1.5.1 Carrier and Cover Tape Dimensions

Figure 23: Carrier and Cover Tape Dimensions

Table 40: Carrier and Cover Tape Dimensions

| Measure | Symbol | Size (in) |
| --- | --- | --- |
| Length | A0 | 0.170 |
| Width | B0 | 0.170 |
| Depth | K0 | 0.049 |
| Pitch | P1 | 0.315 |
| Cavity to Perforation - Length Direction | P2 | 0.079 |
| Cavity to Perforation - Width Direction | F | 0.217 |
| Width | C | 0.362 |
| Width | W | 0.472 |

Feature Size (mm) 4.3

Cavity

Centerline  Distance

4.3

1.25 8.0

2.0

5.50

Cover Tape 9.20 Carrier Tape 12.0

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 54 of 59 www.qorvo.com

®

12.1.5.2 Reel Dimensions

PRELIMINARY

QPG6200L Multi-Protocol SoC

Packaging reels are used to prevent damage to devices during shipping and storage, loaded carrier tape is  typically wound onto a plastic take-up reel. The reel size is 13" diameter. The reels are made from high-impact  injection-molded polystyrene (HIPS), which offers mechanical and ESD protection to packaged devices.

Figure 24: Reel Dimensions

Table 41: Reel Dimensions

| Measure | Symbol | Size (in) |
| --- | --- | --- |
| Diameter | A | 12.992 |
| Thickness | W2 | 0.717 |
| Space Between Flange | W1 | 0.504 |
| Outer Diameter | N | 4.016 |
| Arbor Hole Diameter | C | 0.512 |
| Key Slit Width | B | 0.079 |
| Key Slit Diameter | D | 0.795 |

Feature Size (mm) 330

Flange

Hub

12.1.5.3 Tape Info and Label Placement

18.2 12.8 102.0 13.0 2.0

20.2

Figure 25: Tape and Labels

Notes:

1. Empty part cavities at the trailing and leading ends are sealed with cover tape. See EIA 481. 2. Labels are placed on the flange opposite the sprockets in the carrier tape.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 55 of 59 www.qorvo.com

®

12.2 Moisture/Reflow Sensitivity  Table 42: Moisture/Reflow Sensitivity

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Parameter | Conditions | Value |
| --- | --- | --- |
| Soldering Process |  | Pb-free |
| Peak reflow temperature | 10 s max. | 260 |
| Moisture Sensitivity Level |  | 3 |

Symbol Unit

Tc oC MSL

The Moisture/Reflow Sensitivity is classified according to:

IPC/JEDEC J-STD-020D.1 (March 2008) Joint Industry Standard; Moisture/Reflow Sensitivity  Classification for Non-hermetic Solid-State Surface Mount Devices.

12.3 Environmental Compliance

Table 43: Environmental Compliance

Attribute

Restrictions on the Use of Certain Hazardous Substances in Electrical and Electronic Equipment; Directives 2011/65/EU and 2015/863/EU.

Lead Free

Halogen Free (Chlorine, Bromine)

Antimony Free

TBBP-A (C15H12Br402) Free

PFOS Free

SVHC Free

Symbol Compliant RoHS ✔

Pb

✔

✔

✔

✔

✔

✔

13 Development Kit

Qorvo provides the QPG6200LDK-01, an IoT Development Kit that offers  a complete solution for building multi-standard Matter™, Thread, Zigbee,  and Bluetooth® Low Energy based on the QPG6200L.

This kit enables easy integration of ConcurrentConnect technology in  your application:

• Multi-channel enables Zigbee + Matter concurrently

• Antenna Diversity doubles the effective range

• Multi-radio enables BLE scanning + Matter concurrently. More information available on Qorvo’s website.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 56 of 59 www.qorvo.com

®

Abbreviations

PRELIMINARY

QPG6200L Multi-Protocol SoC

ACL Asynchronous Connection Logical  transport

ADC Analog-to-Digital Converter AEC Automotive Electronics Council AES Advanced Encryption Standard AGC Automatic Gain Control

ANIO Analog Input/Output

AoD Angle of Departure

API Application Program(ming) Interface ARIB (Japan) Association of Radio  Industries and Businesses

ASME American Society of Mechanical  Engineers

CCA Clear Channel Assessment CCM Counter with CBC-MAC (ciphering) CCM* extension of CCM

CDM (ESD) Charged Device Model CSMA/CA Carrier Sense Multiple Access with  Collision Avoidance

DNL Differential Nonlinearity

ECDH Elliptic-Curve Diffie–Hellman ECDSA Elliptic-Curve Digital Signature  Algorithm

ECMQV Elliptic-Curve MQV (Menezes–Qu– Vanstone)

EdDSA Edwards-curve Digital Signature  Algorithm

ESD Electrostatic Discharge

ETSI European Telecommunication  Standardization Institute

EVM Error Vector Magnitude

FCC (US) Federal Communications  Commission

FEM Front-End module

GATT Generic Attribute Protocol GND Ground

GPIO General Purpose Input / Output HAL Hardware Abstraction Layer HBM (ESD) Human Body Model HCI Host Controller Interface

HID Human Interface Device

(e)HPS (enhanced) High Performance State HPSret High Performance State digital retained

IC Integrated Circuit

IEEE Institute of Electrical and Electronics  Engineers

INL Integral Nonlinearity

I2C Inter-Integrated Circuit

I2S Inter-IC Sound

IIP3 Third Order Input Intercept Point IO Input/Output

IR InfraRed

ISM Industrial, Scientific, and Medical  (license-free frequency band)

LDO Low Drop-Out voltage regulator LE (Bluetooth) Low Energy

LED Light Emitting Diode

LFSR linear feedback shift register LjRC Low jitter Resistor–Capacitor (circuit) LL (Bluetooth) Link Layer

(e)LPS (enhanced) Low Power State LQI Link Quality Indication

LSb Least-Significant bit

MAC Medium Access Control layer MCU Micro Controller Unit

MEMS Micro-Electro-Mechanical Systems MOQ Minimum Order Quantity

MOSFET Metal–Oxide–Semiconductor Field Effect Transistor

MSb Most-Significant bit

OTA Over the Air

OTP One-Time Programmable Memory PCB Printed Circuit Board

PDM Pulse-Density Modulation

PER Packet Error Rate

PHY Physical layer

PMU Power Management Unit

POR Power On Reset

PSRR Power Supply Rejection Ratio PUF Physical Unclonable Function PWM Pulse-Width Modulation

QFN Quad Flat No leads (package) RAM Random-Access Memory

RC resistor–capacitor (circuit)

RF Radio Frequency

RSSI Received Signal Strength Indication RoHS Restriction of Hazardous Substances  (Directive)

ROM Read-Only Memory

RX Receive

SHA Secure Hash Algorithm

SIDO Single-Inductor Dual-Output SPI Serial Peripheral Interface

TX Transmit

UART Universal Asynchronous Receiver and  Transmitter

VDD Voltage-Drain-Drain (i.e. positive  voltage supply)

VMT Voltage Minimum Threshold

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 57 of 59 www.qorvo.com

®

Important Notices

PRELIMINARY

QPG6200L Multi-Protocol SoC

The information contained herein is believed to be reliable; however, Qorvo makes no warranties regarding the information  contained herein and assumes no responsibility or liability whatsoever for the use of the information contained herein. All  information contained herein is subject to change without notice. Customers should obtain and verify the latest relevant  information before placing orders for Qorvo products. The information contained herein or any use of such information  does not grant, explicitly or implicitly, to any party any patent rights, licenses, or any other intellectual property rights,  whether with regard to such information itself or anything described by such information. THIS INFORMATION DOES NOT  CONSTITUTE A WARRANTY WITH RESPECT TO THE PRODUCTS DESCRIBED HEREIN, AND QORVO HEREBY  DISCLAIMS ANY AND ALL WARRANTIES WITH RESPECT TO SUCH PRODUCTS WHETHER EXPRESS OR  IMPLIED BY LAW, COURSE OF DEALING, COURSE OF PERFORMANCE, USAGE OF TRADE OR OTHERWISE,  INCLUDING THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

Without limiting the generality of the foregoing, Qorvo products are not warranted or authorized for use as critical  components in medical, life-saving, or life-sustaining applications, or other applications where a failure would reasonably  be expected to cause severe personal injury or death.

Copyright 2023-2024 © Qorvo, Inc. | Qorvo is a registered trademark of Qorvo, Inc.

ConcurrentConnect is a trademark of Qorvo, Inc.

The Bluetooth® word mark and logos are registered trademarks owned by Bluetooth SIG, Inc. Any use of such marks by  Qorvo is under license.

Matter is developed by the Connectivity Standards AllianceTM. This brand, related logos, and marks are trademarks of the  Alliance, all rights reserved. Arm and Cortex are registered trademarks of Arm Limited (or its subsidiaries) in the US and/or  elsewhere. All other trademarks, trade names, product or service names are those of their respective owners.

Product Status

Product Status

Formative / In Design

First Production

Full Production

Not in Production

Marking Definition

ADVANCE INFO Datasheet contains design specifications for product development.  Specifications may change in any manner without notice.

Datasheet contains preliminary data; supplementary data will be

► PRELIMINARY

published at a later date. Qorvo reserves the right to make changes  at any time without notice to improve the design.

(none) Datasheet contains final specifications. Qorvo reserves the right to make  changes at any time without notice to improve the design.

OBSOLETE Datasheet contains specifications on a product that is discontinued. The  datasheet is for reference information only.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 58 of 59 www.qorvo.com

®

Document History

PRELIMINARY

QPG6200L Multi-Protocol SoC

| Date | Section |
| --- | --- |
| 22 Sep 2023 | all |
| 9 May 2024 | Table 38 |
|  | 12 |
| 17 May 2024 | 2 |
|  | 7.3 |
|  | 11 |
|  | 11.3-11.5 |
|  | 12.1.2 |
|  | Figure 6 |
|  | Table 38 |
| 28 May 2024 | 7.2 |
|  | 3 |
| 24 Jul 2024 | 11.3 |
|  | 11 |
|  | 6.2.2 |
|  | 11.13 |
|  | Table 37 and Table 38 |
|  | all |
| 27 Sep 2024 | all |
|  | all |
|  | 2 |
|  | 3, 7.3, 11.11 |
|  | 5 |
|  | 7.2 |
|  | 8.9 |
|  | 10 |
|  | 11.5 |
|  | 11.7 |
|  | 11.9.1 |
|  | 11.9.2 |
|  | 13 |
|  | all |

Version Changes

0.1 Released version.

0.2 High-sink IR for GPIO4 removed. Micbias signal changed to LPbias.

0.3 Block diagram updated.

ANIO / ADC section updated.

Parameter Evaluation Circuit diagrams updated.

Data updated.

Package drawings and land patterns updated.

Power modes diagram updated.

GPIO mapping table updated (IR).

0.4 5V tolerance text added.

GPIO mapping tables updated.

Timers added in Key Features.

0.5 TBD’s removed in max. and max. columns in  current consumption tables.

Parameter evaluation circuit diagrams for QFN32

updated to Rev 0.6.

RF4CE functionality added.

DCDC Efficiency graph added.

GPIO mapping tables updated.

Minor format and textual updates.

0.6 1st draft for QPG6200L. From this version the  general QPG6200 DS converges to the

QPG6200L DS version.

AoA/AoD text removed.

Block diagram updated.

ANIO / ADC sections updated.

Contact information updated.

5V Tolerance text removed.

Application Programming Information updated.

Support for DPA counter measures removed.

EVM filtering details updated.

Table for Reset, Wake up and Sleep Timing

Characteristics renewed.

32 MHz Crystal Oscillator text updated.

32 KiHz Crystal Oscillator text updated.

Development Kit text updated.

Minor text and format updates.

Rev. B Sep 2024 | Subject to change without notice | All rights reserved. | Qorvo Confidential & Proprietary Information  © 2023-2024 Qorvo, Inc. Page 59 of 59 www.qorvo.com