QPG6200LDK-01 Dev Kit research

Termenology:

IoT: Internet Over Things

WPAN: Wireless Personal Area Networks

GPIO: General Purpose Input/Output

ADC: Analog Digital Converter

GP: General Purpose

PWM: Pulse Width Modulation

PDM: Pulse Density Modulation

SPI: Serial Peripheral Interface

UART: Universal Asynchronous Receiver/Transmitter

SDIO:  Secure Digital Input/Output

MEMS: Micro-Electro-Mechanical Systems

PCM: Pulse Code Modulation

Overview:

Assists in the development of IoT products through the use of matter over thread, Zigbee and bluetooth low energy. Features Qorvo’s ConcurrentConnect technology. Allows multiple protocols to operate simultaneously. Includes a 2 x2.4Ghz Antenna PCB such as Bluetooth 5.3 Low Energy and IEEE 802.15.4. Connect via USB-C.

Utilizes IoT to communicate with other compatible devices using IEEE 802.15.4 protocols.

Has arduino headers on the carrier board.

No RF Shielding for the board is required.

Radio:

2.4Ghz RF transceiver compliant with Bluetooth 5.3 and IEEE 802.15.4 for WPAN. Contains integrated RF filter and matching. Compliant with the following worldwide RF regulations: ETSI EN 300 328 (Europe), FCC CFR47 Part 15 (US), and ARIB STD-T-66 (Japan).

Antenna ports output is 50 Ohms single ended.

Supports different radio configurations. Can be configured to have an antenna for receiving and an antenna for transmitting.

Transceiver:

The channel number (k) and center frequency (Fc) relate by the following:

Fc = 2405 + 5(k - 11) [MHz]

Integrated Application microcontroller:

Contains Arm Cortex-M4 processor with DSP functionality.

Peripherals and Interfaces:

- 14 programmable GPIO lines

- 2 Analog Input lines

- 16-bit high-resolution and 11-bit GP ADC

- Temperature Sensor

- Timers: 6x 32-bit or 12x 16-bit

- PWM Engine for 8 outputs

- LED Generator (8-bit PWM) with fading support for 4 signaling LEDs

- LED Pixel Protocol Master (RTZ) for dynamic control of LED light strips

- USB 2.0

- 2x PDM Microphone Interface

- 1x SPI Master and 1x SPI Slave interface

- 2x I2C Master and 1x I2C Slave interface

- 2x I2S Master/Slave interface for digital audio devices

- 3x UART interface

- Keyboard Scanner

- IR Generator; High Drive Sink for IR

Includes 3 UART’s that can interface with peripheral devices and/or terminal logging during software development. The UART’s support Full-duplex operation . The RX and TX pins are configurable on the pin-out.

The QSPI master provides users the ability to interface with external memory slaves.

The SPI master allows users the ability to interface with additional peripheral devices in Full-duplex synchronous transfers on three lines: MISO, MOSI, and SCLK.

The dev board includes 2 I2C master interfaces that allow users to interface with additional peripheral devices.

The dev board includes 2 I2S master interfaces that allow users to interface with additional peripheral devices, such as digital audio devices, ADC’s, DAC’s, and audio CODEC’s.

USB-C:

USB-C ports support virtual communication. These ports cannot be used to display information to an interactive display directly.

Power Management:

Required an operating voltage range of 1.71V - 3.6V. Typical supply voltage is 3V. Includes integrated regulators, SDIO DC/DC Buck Converter.

Two low power sleep modes that retain data and state retention.

PDM Microphone Interface:

The board contains 2 PDM MEMS Microphone interfaces, which provide conversion from a PDM signal to a PCM signal. The PCM signal can be used in further processing using the embedded Audio Signal Processors.

Sources:

Dev Board: https://www.qorvo.com/products/p/QPG6200LDK-01

QPG6200L Data Sheet, Rev B; Qorvo: Greensboro NC, September 2024, Accessed September 3rd 2025.