# Raspberry Pi Gateway Installation Guide

# Project Context:

This section covers the installation and setup of the Raspberry Pi 4 for the QPG6200L IoT Gateway stack, including flashing the SD card, installing Node-RED, and configuring it to run as a service.

# Table of Contents

Project Context:	1

Table of Contents	1

1. Preparing the microSD Card	1

2. First Boot and Connecting to the Pi	2

3. Installing Node-RED	2

4. Optional: Accessing Node-RED Remotely	2

5. Next Steps	3

## 1. Preparing the microSD Card

- Insert the microSD card into a USB card reader and plug it into your PC.

- Download Raspberry Pi Imager from the official site:
https://www.raspberrypi.com/software/

- Launch Raspberry Pi Imager.

- Select OS: Choose Raspberry Pi OS (64-bit) for general compatibility.

- Select Storage: Choose your microSD card.

- Select Customization:

  - Enter Hostname: Choose any name, such as iiot-gateway.

  - Enter your capital, time zone, and keyboard layout.

  - Create a username and password for Raspberry Pi

    - For this gateway:

      - Username: qorvo_radio2

      - Password: qorvo_radio2_UNCC

- Configure Advanced Options:

  - Enable SSH if you want headless access.

  - Skip Raspberry Pi Connect.

  - Set your Wi-Fi SSID and password (if using wireless).

    - If using an open network like NinerWiFi-Guest, select the “Open Network” option.

- Write the image: Click Write and wait for the process to finish.

- Safely eject the microSD card and insert it into the Raspberry Pi.

## 2. First Boot and Connecting to the Pi

- Power on the Raspberry Pi with the microSD card inserted by the power supply plug.

- Connect a monitor and keyboard, and open up Command Prompt in the search bar.

- Ping the gateway using this command: ping <your_hostname> Ex. ping iiot-gateway

  - If it timeouts, do cd C:\Windows\System32

  - You should see it reply back with packets sent and received

- Go into SSH by: shh <username>@<hostname> Ex. ssh qorvo_radio2@iiot-gateway

  - Say yes if you want to connect.

  - Enter the password previously set.

- Update the OS packages by the following commands:

  - sudo apt update

  - sudo apt upgrade -y

## 3. Installing Node-RED

Node-RED is a visual programming environment for connecting IoT devices and dashboards. After this installation, you can customize the dashboard for any purpose.

- Run the Node-RED installer for Linux (official script):

  - bash <(curl -sL https://github.com/node-red/linux-installers/releases/latest/download/update-nodejs-and-nodered-deb)

- This script installs Node.js, Node-RED, and sets up the environment correctly.

- Enable Node-RED to start automatically on boot:

  - sudo systemctl enable nodered.service

- Start Node-RED manually for the first time:

  - node-red-start

- You should see the Node-RED console logs and a message like:

- Server now running at http://127.0.0.1:1880/

- Access the editor from a PC on the same network:

  - Open a browser and navigate to: http://<raspberry-pi-ip>:1880

## 4. Accessing Node-RED Remotely

- For remote access (e.g., from a phone or laptop outside the network), use ngrok or another tunneling service.

- This allows your Node-RED dashboard to function as a web-accessible IoT application.

## 5. Next Steps

Once Node-RED is running:

- Connect your nRF52840 Thread dongle to the Pi.

- Install OTBR to manage the Thread mesh network.

- Install Mosquitto for MQTT message routing.

- Set up dashboard nodes in Node-RED to visualize sensor data.

# Useful Links

EP2: How to Build a Raspberry Pi IIoT Gateway (Industry 4.0 Tutorial)

Raspberry Pi Official Documentation: https://nodered.org/docs/getting-started/raspberrypi