Battery Life Recalculation

Objective

The purpose of this document is to document the process of recalculating the individual remote device battery life. Since SD1 the custom PCB and device has undergone drastic design changes such as the batteries, audio amp circuit, and power supply regulators. All of these changes affect the initial calculations requiring a recalculation to certify accurate results.

Specifications

- UBBP01 (Li-Ion Battery)

  - 1.8 AH

  - 3.7V (Each)

- 1 week of total board run time

Solution

The general equation used to calculate the current draw from all components on the custom PCB is as follows.

Iavg=IactiveTactive+IideleTidle+IsleepTsleepTtotal

Device run time is calculated by dividing the total mAH of the power supply, by the total current draw (mA) of the PCB board.

The following IC’s and devices will be utilized in power calculations

LDO40LPU33RY:

Active current: 45 µA

Idle current: 0 A

Sleep current: 0.2 µA

Total current draw at 100% duty cycle: 0.045 mA

LDO40LPU50RY:

Active current: 45 µA

Idle current: 0 A

Sleep current: 0.2 µA

Total current draw at 100% duty cycle: 0.045 mA

QPG6200L:

Active current: 6.5 mA

Idle current: 400 µA

Sleep current: 50 µA

Total current draw at 20% duty cycle: 1.585 mA

DAC6551AQDGKRQ1

Active current: 160 µA

Idle current: 0 A

Sleep current: 0.8 µA

Total current draw at 10% duty cycle: 0.017 mA

LM48511SQX:

Active current: 13.5 mA

Idle current: 0A

Sleep current: 0.01 µA

Total current draw at 10% duty cycle: 1.35 mA

CSS-66668N:

Active current: 600 mA

Idle current: 0A

Sleep current: 0A

Total current draw at 10% duty cycle: 60 mA

SPG08P4HM4H-1:

Active current: 1.4 mA

Idle Current: 0.29 mA

Sleep current: 35 µA

Total current draw at 10% duty cycle: 0.376mA

MB1000-000:

Active Current: 2mA

Idle Current: 0A

Sleep Current: 0A

Total current draw at 10% duty cycle: 0.2 mA

Total Device Current Draw:  63.618 mA

Device run time: 28.294 Hours